Preliminary Findings from Emerging Technology Scoping Study on Smart Plug Strips

Draft—Do not circulate

Prepared by: Catherine Mercier, Craig Billingsley and Laura Moorefield

Ecos Consulting

[Legal Notice/Acknowledgement Page/Table of Contents]

- Introduction

Consumer electronics, office equipment and other plug loads consume 15 to 20 percent of total residential and commercial electricity in the United States. Much of this energy is consumed when these devices operate in low-power modes but are not actually in use. One way to reduce this unnecessary electricity consumption is to use smart plug strips. Smart plug strips vary in design, but typically employ some combination of load sensors, remote controls, occupancy sensors, and timers to automatically power down plug loads when they are not in use.

A simple but infrequently-used way to save energy is to manually unplug the devices from wall outlets. Consumers can also use conventional plug strips. These products allow multiple devices to be plugged into one unit, which is then plugged into a single wall outlet. (Surge protectors are often incorporated into plug strips). Instead of having to unplug multiple devices, the consumer could simply turn off power to the plug strip via its hard switch. However, this requires consumers to reach below desks, behind home entertainment equipment, or around furniture to access inconvenient places where plug strips usually sit. Additionally a plug strip that is out-of-sight plug strip is often out-of-mind, which means that the consumers may forget to turn them off.

By automatically turning off plug loads when not in use, smart plug strips can provide energy savings in residential and commercial sectors. Applications for smart plug strips include workstations, open office cubicles, home offices, and home entertainment systems. For many applications, smart plug strip manufacturers claim payback periods of less than two years. However, little objective, independent and detailed research is available on these devices.

The objective of this study is to evaluate the smart plug strips that are currently on the market, and to determine the potential energy savings opportunity associated with smart plug strips. We began our research by reviewing previous measurements and market research. We then evaluated the performance and standby power of a sample set of smart plug strips in our laboratory. Next, we conducted two brief field studies with selected smart plug strips, and generalized these results to “typical” users. Finally, we identified areas for future research and product design improvements.

- Description of Technology

Smart plug strip designs are based on typical plug strip designs, but also incorporate additional technologies to automatically disconnect power to certain equipment when not in use. Most smart plugs strip have one “master” control outlet, four to six controlled outlets that will automatically power down designated devices when the control load is turned off by the user, and one or two uncontrolled outlets that are always on. We encountered the following six varieties of smart plug strips during this study:

- Occupancy sensing

- Load sensing

- Timer

- Remote control

- USB interface

- Informational

Note that many of these options could be combined in a single product for increased customization by the consumer. For example, a timer could be used for some outlets on a smart plug strip where other outlets are controlled by a load sensor. We were not able to locate a smart plug strip with combinations of multiple features for testing.

  - Occupancy Sensing

Occupancy sensing plug strips detect the presence or absence of a user, and automatically turn the attached equipment on and off accordingly.

Hard switch

Occupancy sensor lens

LED (blinks when occupancy detected)

Six controlled outlets

Two uncontrolled outlets

Figure 1: Isolé IDP-3050 Occupancy Sensing Smart Plug Strip

Photo Credit: WattStopper,      http://www.wattstopper.com/products/details.html?id=74&category=122&type=Commercial

All use either passive infrared sensors or ultrasonic sensors to detect when a user is present and control the flow of power to specific outlets in a plug strip that are marked as sensor-controlled (BC Hydro 2008a). When the user is not present, these sensor-controlled outlets can be shut down immediately or on a time delay (anywhere from 30 seconds to 30 minutes).These smart plug strips usually have one or two outlets that are always on, meaning that they remain constantly powered and are not affected by the presence of the user.

Ultrasonic sensors emit ultrasonic energy waves at high frequency that reflect off of objects and return to the sensor to detect changes in frequency of the reflected signal. By emitting a signal instead of only receiving it, they are able to cover larger areas than passive infrared sensors and detect smaller motions. However, they are more prone to false triggering.

Figure 2: Sensor Coverage Diagram

Photo Credit: E Source, retrieved from www.mge.com/business/saving/madison/PA_3.html

Ultrasonic sensor can detect motion at any point within the contour lines. Passive infrared 	sensors see only in the wedge-shaped zones, and they don’t generally see as far as ultrasonic 	units.

Occupancy sensors on smart plug strips can be improperly adjusted, or fooled into perceiving the presence of the user when a space is unoccupied; see Figure 2 (BC Hydro 2008a). For continued energy savings, users should follow certain recommendations:

- If in an office setting, involve building personnel in planning for the sensors, train maintenance personnel and office occupants to keep sensors operational rather than disconnecting them when problems occur (BC Hydro 2008a).

- Position sensors carefully so that they see only the area that you want them to see; the biggest cause of false triggering is incorrect sensor positioning (BC Hydro 2008a).

  - Load Sensing

Load sensing smart plug strips detect the drop in current that occurs when the control device enters a low-power mode. A current sensing transformer attached to an outlet on the plug strip monitors the current draw of the designated device. When the current draw of this device drops below a certain threshold, power is disconnected from the controlled outlets on the plug strip. In other words, when the user turns off the device or the device powers down due to enabled power management, the smart plug strip disconnects power to the rest of the devices. For example, if the control device is a computer, the load sensor would detect when the computer enters sleep or standby mode, then disconnect power to the other devices plugged into that strip. As with the occupancy sensing plug strips, load sensing plug strips usually also include always on outlets that are not affected by the control device. Within this category, there are several different models and options. Some have a user-adjustable threshold that can be tuned to correspond to the current the device draws in sleep or standby mode.

Note that load sensors are dependent on the power state of the control device; the controlled devices will be shut off only as often as the control device is powered down. If the threshold of the control device is set incorrectly, the smart plug strip will not detect the reduced current, and peripherals will not turn off.

Six controlled outlets

One control outlet

Three uncontrolled outlets

Hard switch

Figure 3: BITS Limited #LCG3 Load Sensing Smart Strip Plug Strip

Photo Credit: BITS Limited, http://catalog.bitsltd.us/

  - Timers

Smart plug strips can also turn equipment on and off based on a programmable timer instead of or in addition to occupancy and load sensing. An example of a timer only smart plug strip is the  Intermatic DT8003-70. We were not able to obtain this product for testing. Turning devices on and off with a timer ensures energy savings over night, and on weekends and holidays. Timers are a good option in cases where, for example, a laser printer or a copier requires a long warm-up. Timers are also a good option with set-top boxes and modems. Many set-top box hard drives unnecessarily draw power 24 hours a day, seven days a week, even if the user has not asked to record programming during all of these hours. Timers can turn off devices such as set-top boxes and modems during the hours when consumers know they will not need to record shows or access the Internet, thereby reducing annual energy use significantly.

Figure 4: Intermatic Digital Timer Smart Plug Strip

Photo Credit: http://www.bchydro.com/ecatalog/eCatServlet?cmd=generate_pdf&prdId=3619&cId=14&pid=&rid=&eid=

  - Remote Control

Some smart plug strips use wireless remote controls. This technology enables a consumer to disconnect power to all devices plugged into the strip using a small remote control. The advantage of the remote control smart plug strip is that it is a convenient way for consumers to control exactly when devices receive power or don’t without having to reach below desks, behind home entertainment equipment, or around furniture to access inconvenient places where plug strips are typically located. The disadvantage of this technology is that, to save energy, users must be highly motivated and remember to use the remote to turn off the strip.

Wireless remote control

Figure 5: Belkin Conserve Smart Plug Strip

PhotoCredit: Belkin, http://catalog.belkin.com/IWCatProductPage.process?Product_Id=459507

  - USB Interface

Smart plug strips using USB interfaces plug into the computer via a USB connection. When the computer is turned off, a signal from the USB port shuts down the flow of power to peripherals (e.g. printers, scanners, modems, speakers, and standby lights). This technology is applicable to commercial and home offices.

USB ON/OFF function

Figure 6: Ecostrip USB Smart Plug Strip

Photo Credit: Beamingsun, www.theecostrip.com

The only USB smart plug strip model in our dataset is the Ecostrip, manufactured by Beamingsun. The Ecostrip was first introduced in Europe in 2005. According to Ecostrip, sales have grown rapidly in Europe since that introduction (Ecostrip 2008). At the time of this study, this sample was out-of-stock in the U.S.; however, the manufacturer provided us a sample for testing.

  - Informational

Informational smart plug strips incorporate power meters and display screens to show the user how much electricity each of the devices plugged into the strip uses. Informational plug strips may display power, voltage, power factor and/or current.

Figure 7: Kill-A-Watt Informational Smart Plug Strip

Photo Credit: http://www.kaboodle.com/reviews/power-cost-controller-power-strip

- Market Status

Little independent market research information is available on smart plug strips. According to a study published by E Source in July of 1998, smart plug strips were extremely scarce and customers were generally uninformed of their existence at the time (Krepchin 1998). A 2007 PG&E study prepared by Ecos Consulting on this product market corroborated these findings (PG&E 2007). Researchers estimated the stock of smart plug strips to be small—on the order of hundreds of thousands nationally (PG&E 2007).

Despite findings that suggest smart plug strips are not widely used, many manufacturers now offer these products, including: USA Technologies, The Watt Stopper Inc., BITS Limited, Belkin, APC by Scheider Electric, Power Sentry, and Beamingsun. Smart plug strips from these manufacturers have varying degrees of flexibility and sophistication. For this research project, researchers located ten models of smart plug strips that were available for purchase at local retailers and online vendors:

- occupancy-sensing (2)

- load-sensing (5)

- remote control (1)

- USB technology (1)

- informational (1)

- Since the conclusion of this study, a few new models of smart plug strips have become available to consumers.

Table 1: Overview of Smart Plug Strip Features

| Product name | Manufacturer | Technology | # of control outlets | # of controlled outlets | # of uncontrolled outlets | Surge protection | Power or time threshold | Max load (W) | Price | Availability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Isole IDP-3050 | The Watt Stopper Inc. | Passive infrared occupancy sensor |  | 6 | 2 | Yes | Adjustable 30 sec. - 30 min. | 1600 | $85-95 | Available |
| Smart Strip SCG5 | BITS Limited | Load sensor | 1 | 4 | 2 | Yes | Adjustable 28 - 300 W | 1800 | $30-43 | Available |
| Smart Strip LCG2 | BITS Limited | Load sensor fax/modem and coaxial surge protection | 1 | 6 | 3 | Yes | Adjustable 28 - 300 W | 1800 | $31-35 | Available |
| Smart surge protection | Power Sentry | Load sensor | 1 | 4 | 1 | Yes | Not adjustable 5 W |  | $20-25 | Available |
| Plug Miser | U.S.A Technologies | Passive infrared occupancy sensor 1 modem |  | 1 |  | No | Adjustable 3 sec. – 3 hrs | 1230 | $79-84 | Available |
| USB Ecostrip | Beamingsun | USB | 1 | 5 |  | Yes | Not adjustable | 1250 | $35-40 | Out of stock |
| APC Battery Backup ES Series BE750G | APC | Load sensor, cable modem/video and data line protection. 70 minutes of battery backup | 1 | 3 | 5 | Yes | Not adjustable | 450 | $85-98 | Available |
| APC Surge Protector P7GT | APC | Load sensor | 1 | 3 | 3 | Yes | Not adjustable 19 W |  | $21-35 | Available |
| Kill-A-Watt | P3 international | Monitors power, voltage, line frequency, and power factor |  |  | 8 | Yes | N/A | 1980 | $90-100 | Available |
| Conserve | Belkin | Wireless remote-control |  | 2 | 6 | Yes | N/A |  | $34-55 | Available |

- Energy Savings and Costs

In this section, we discuss the incremental costs of smart plug strips and their potential energy savings. We first present energy saving estimates reported by a manufacturer as well as results from previous independent research on smart plug strips. We then present our laboratory evaluations of product performance and standby power of a sample set of smart plug strips. Finally, we discuss our field study results.

  - Previous Research

There is limited research available on quantifying the energy reduction potential of smart plug strips. Research findings reported by Watt Stopper indicated that energy savings of 300 kWh per year per workstation could be realized by installing occupancy sensors on workstations with typical usage patterns (WattStopper 1997). Researchers from the Florida Solar Energy Center monitored the energy consumption of three copiers and one computer workstation before and after the installation of occupancy sensing plug strips. They found that savings for the copiers, which had previously been left on 24 hours a day, ranged from 55 to 62 percent, and that savings for the computer workstation were about 30 percent (Krepchin 1998).

IT-Energy, a Danish research firm, conducted a study estimating standby consumption in Danish homes by 2006, and the possible energy savings from using power saving outlets in Denmark. According to IT-Energy, each energy efficient plug strip for personal computers saves an average of 90 kWh per year in Denmark, ranging from 24 to 181 kWh per year. Each energy efficient plug strip for a primary television saves an average of 61 kWh, ranging from 26 to 137 kWh per year (IT-Energy 2007).  Similarly, BC Hydro estimates saving of 100 kWh per year per unit for models that qualify for their rebate program (BC Hydro 2008c).

PG&E’s 2007 research showed that energy savings between 16 and 26 percent, or between 70 and 184 kWh per year per workstation, could potentially be realized by incorporating smart plug strips into modular office furniture. These energy savings depend on time delay settings, behavioral patterns and effectiveness of installation. PG&E estimates suggest that over a ten-year product life, smart plug strips can yield a net benefit of $60 to $135. These results showed that load-sensing designs yield smaller energy savings than the occupancy-sensing designs; however, we believe that in the field these technologies would yield similar energy savings. Load-sensing smart plug strips are highly sensitive to computer/monitor power management features. In addition, the industry is moving quickly towards power management enablement as the norm. The load-sensing approach has the potential to perform as well as occupancy-sensing designs.

  - Laboratory Testing

For this study, we evaluated nine models of smart plug strips and one uninterruptible power supply (UPS) to confirm that they perform as advertised. We then metered these same smart plug strips in our laboratory to determine if they add any power demand of their own. For each laboratory test, we measured power, power factor, voltage and current.  We tested the smart plug strips in the following conditions:

- plugged in with the switch turned off and no load,

- plugged in with the switch turned on and no load,

- plugged in with the switch turned on  and a 60 W light bulb connected,

- plugged in with the switch turned on and a monitor connected,

- plugged in with the switch turned on and a radio connected,

- plugged in with the switch turned on, and the radio, the monitor and the 60 W light bulb connected.

- We also tested the radio, the monitor, and the 60W light bulb, and the monitor without a plug strip as a base case.

Table 2  presents the test results.

Table 2: Smart Plug Strip Test Results

|  | Plug strip Only |  |  | 60 W Light Bulb |  |  |  | Monitor |  |  |  | Radio |  |  |  | 60 W Light Bulb/ Monitor/ Radio |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Product tested | Hard switch off (W) | Hard switch on (W) | Without strip (W) |  | With strip (W) | Power added by strip (W) | Without strip (W) |  | With strip (W) | Power added by strip (W) | Without strip (W) |  | With strip (W) | Power added by strip (W) | Without strip (W) |  | With strip (W) | Power added by strip (W) |  |
| USB Ecostrip | 0 | 0 | 61.52 |  | 61.87 | 0.4 | 30.05 |  | 28.97 | -1.1 | 6.95 |  | 7.48 | 0.5 | 99.05 |  | 98.15 | 0.9 |  |
| APC Battery Backup ES Series BE750G | 3.05 | 3.03 | 61.52 |  | 66.20 | 4.7 | 30.05 |  | 33 | 3.0 | 6.95 |  | 10.9 | 4.0 | 99.05 |  | 102.3 | 3.25 |  |
| APC Surge Protector P7GT | 0 | 1.52 | 61.52 |  | 63.36 | 1.8 | 30.05 |  | 30.85 | 0.8 | 6.95 |  | 9.01 | 2.0 | 99.05 |  | 100.1 | 1.05 |  |
| Kill-A-Watt | 1.77 | 1.77 | 61.52 |  | 63.32 | 1.8 | 30.05 |  | 31.32 | 1.3 | 6.95 |  | 8.94 | 2.0 | 99.05 |  | 100.45 | 1.4 |  |
| Smart Strip SCG5 | 0 | 0.72 | 61.52 |  | 64.05 | 2.5 | 30.05 |  | 32.3 | 2.3 | 6.95 |  | 8.18 | 1.2 | 99.05 |  | 101.32 | 2.3 |  |
| Smart Strip Power Strip LCG2 w/ Coax, Fax & Modem Protection | 0 | 0.74 | 61.52 |  | 62.60 | 1.1 | 30.05 |  | 32.1 | 2.1 | 6.95 |  | 8.24 | 1.3 | 99.05 |  | 101.22 | 2.2 |  |
| Power Sentry Power Strip | No hard switch | 1.80 | 61.52 |  | 63.52 | 2.0 | 30.05 |  | 31.41 | 1.4 | 6.95 |  | 9.07 | 2.1 | 99.05 |  | 100.56 | 1.5 |  |
| Wattstopper Isole IDP -3050 | 0 | 1.05 | 61.52 |  | 64.23 | 2.7 | 30.05 |  | 32.45 | 2.4 | 6.95 |  | 10.23 | 3.3 | 99.05 |  | 101.1 | 2.0 |  |
| Plug Miser | 1.35 | n/a | 61.52 |  | 63.28 | 1.8 | 30.05 |  | 31.02 | 1.0 | 6.95 |  | 8.83 | 1.9 | 99.05 |  | n/a | n/a |  |
| Belkin Conserve | 0.89 | 0.47 | 61.52 |  | 62.40 | 0.88 | 30.05 |  | 30.55 | 0.50 | 6.95 |  | 7.98 | 1.03 | 99.05 |  | 99.25 | 0.20 |  |

Most of the smart plug strips add a small amount of power to the amount that the devices would draw on their own. The smart plug strips metered drew between 0 and 1.8 watts of power with no devices plugged into them. The UPS drew just over 3 watts of power to maintain the charge of its battery. Still, energy savings from a smart plug strip should, in most cases, exceed the energy it consumes. None of the plug strips had an effect on the power factor. Finally, we found that the Kill-A-Watt, the only informational smart plug strip in our data set, reports voltage with an accuracy of 1%, wattage with an accuracy of 1-2%, line frequency with an accuracy of 2-11%, and power factor accurately. Overall, the smart plug strips in our data set performed as designed and advertised.

  - Field Studies

Because few data are available that document actual energy savings associated with smart plug strips, we conducted two brief field studies with selected plug strips. We monitored the energy consumption of both a home office and a home entertainment before and after installation of a smart plug strip. The power meter recorded the power consumption of the connected equipment at one-minute intervals for a period of two or three business days. We then compared the power consumption profile and duty cycle using a smart plug strip with the data we collected prior to installing the smart plug strip.

    - Home Entertainment Center

The home entertainment center that we metered included the following devices: a receiver, two game consoles, an HD-DVR, a DSL modem, a router, a VCR, a subwoofer, and a DVD player. In this case, the TV was not metered because this home entertainment center uses a projection TV that is mounted on the ceiling and not able to be connected to a smart plug strip. First, we metered the energy consumption of the home entertainment center with a Watts Up PRO ES portable field meter for two days. The system drew 85 watts for 22 hours/day (92% of the time metered) and between 160 and 510 watts for 2 hours/day (8% of the time) when actively in use. See Error: Reference source not found. We then installed a load-sensing smart plug strip and re-metered the power demand of this system. Next, with the load-sensing smart plug strip still in place, we installed a timer on the HD-DVR and metered the power demand of this system a final time. Finally, we replaced the load-sensing smart plug strip with a remote control smart plug strip, kept the timer installed, and re-metered this system to document changes to energy savings caused by a second variety of smart plug strip.

Load-Sensing Smart Plug Strip

For this test, we connected components of the home entertainment center to an APC load-sensing smart plug strip. The APC smart plug strip features one master (control) outlet, three controlled outlets, and three uncontrolled outlets. We plugged the receiver into the master outlet. We connected two game consoles, a VCR, a subwoofer, and a DVD player to the controlled outlets. We plugged the HD-DVR, DSL modem and router into the uncontrolled outlets because these devices sometimes need to operate when the rest of the system is off. When the power button on the remote of the receiver was turned off, the load-sensing smart plug strip disconnected power to the receiver and all other components connected to the controlled outlets; the HD-DVR, DSL modem and router remained powered. When the receiver power button on the receiver was turned on, the smart plug strip re-connected power to the other components making them available to be used, but not actually turning them on. In order to meter the system power demand, we plugged the smart plug strip (with all of its connected loads) into a plug load power meter and data logger.

For the final test, we installed a programmable timer on the HD-DVR. The timer allowed the HD-DVR to remain powered during hours when the owner wanted to record shows, but disconnected the HD-DVR from power during hours when the owner never recorded shows. We metered this system configuration to determine energy savings from the added control of a programmable timer. Figure 8 illustrates the average power demand by hour for the base case home entertainment system and Figure 9 and Figure 10 show how the duty cycles, and therefore overall energy consumption, of these devices are affected by a load-sensing smart plug strip and a timer.

Figure 8: Base Case: Home Entertainment System without a Smart Plug Strip

Figure 9: Home Entertainment System with a Load-Sensing Smart Plug Strip

Figure 10: Home Entertainment System with Load-Sensing Smart Plug Strip and Timer

Our data suggest that using a load-sensing smart plug strip can yield significant energy savings. But if a timer is added to the system, the savings become more substantial because the timer eliminates the HD-DVR’s high power demand (about 50 watts) when the owner does not want to record or play back shows. Using an APC load-sensing smart plug strip with the home theater system that we metered could yield about 244 kWh per year in energy savings. If a load-sensing smart plug strip and a programmable timer were used together to control plug loads, the system owner could expect energy savings of 610 kWh per year. See Table 3 below.

Table 3: Entertainment Center Energy Savings from APC Load-Sensing Smart Plug Strip and Timer

|  |  | Annual savings per unit |
| --- | --- | --- |
| APC smart plug strip | Energy savings | 244 kWh |
|  | Utility bill savings | $24 |
| Timer | Energy savings | 366 kWh |
|  | Utility bill savings | $37 |
| APC smart plug strip and timer | Energy savings | 610 kWh |
|  | Utility bill savings | $61 |

* We assumed an average electric rate of 10 cents per kWh.

The retail price of an APC load-sensing smart plug strip is about $25. The retail price of a separate programmable timer is about $12. We estimate the average cost of a regular power strip with a surge protector to be $20. Therefore, the incremental cost of purchasing an APC load-sensing smart plug strip and a timer is $17. In this case, the annual energy savings will pay for the incremental cost in about four months. A timer integrated into a smart plug strip would have and incremental cost much lower than $17 making the payback period even shorter. It is important to note that actual savings are based on standby power consumption and will vary depending on usage patterns, type of devices connected to the smart plug strip, and user behavior.

One limitation of the APC load-sensing smart plug strip that can affect savings potential is the absence of an embedded programmable timer. As mentioned above, significant energy savings were attributable to the use of a programmable timer. Also, in this case, the power sensing threshold was not adjustable. In our lab, we measured a threshold of 19 watts. If the user’s control device doesn’t fall below 19 watts (above the smart plug strip’s threshold), peripherals won’t be turned off.

Remote Control Smart Plug Strip with Timer Added

We then replaced the APC load-sensing smart plug strip with a Belkin Conserve remote control smart plug strip to see how energy savings would vary with different smart plug strip technologies. We left the timer installed as it was in the previous measurements. We then re-metered the home entertainment center as described above. The remote control easily disconnects and connects the power that flows to devices plugged into controlled outlets, while devices requiring a continuous power supply such as modems or routers can be plugged into the always-on outlets.

Figure 11: Belkin Conserve Remote Control Smart Plug Strip

Photo Credit: Belkin, http://www.notcot.com/archives/2008/01/belkin_conserve.php

Figure 12 shows how the duty cycles of the home entertainment devices change with a Belkin smart plug strip and a timer compared to the baseline case. It is important to understand that the energy savings from a remote control smart plug strip are entirely dependent on the user’s behavior.

Figure 12: Entertainment Center Energy Use with Remote Control Smart Plug Strip and Timer

Table 4: Entertainment Center Energy Savings with Remote Control Smart Plug Strip and Timer

|  |  | Annual savings per unit |
| --- | --- | --- |
| Belkin remote control smart plug strip | Energy savings | 264 kWh |
|  | Utility bill savings | $26 |
| Timer | Energy savings | 363 kWh |
|  | Utility bill savings | $36 |
| Belkin remote control smart plug strip and timer | Energy savings | 628 kWh |
|  | Utility bill savings | $63 |

* We assumed an average electric rate of 10 cents per kWh

Belkin’s remote control smart plug strip lets users easily power off all devices when not in use, saving both energy and money. By using a remote control smart plug strip and a timer, households could save more than $60 per year. The retail price of a Belkin Conserve smart plug strip is about $50. If we assume that the cost of a regular power strip is $20, the incremental cost of purchasing a Belkin Conserve smart plug strip is $30.  The retail price of a separate programmable timer is about $12. In this case, the annual energy savings will pay for the incremental cost in about eight months. A timer integrated into a smart plug strip would have an incremental cost much lower than $42 making the payback period even shorter. One possible limitation of these products is that energy savings entirely depend on user behaviors. The user needs to use the remote switch or the use of this smart plug strip will yield no savings.

    - Home Office

The home office that we metered included a desktop computer, an LCD monitor, a multi-functional device, a modem, a wireless router, and a 2.1 speaker system (two stereo speakers and one subwoofer). We monitored the energy consumption of the home office before and after installation of an Ecostrip USB smart plug strip. Because the Ecostrip requires a USB and a power cord connection between the strip and the control device, a computer is the only device that can serve as the control device.

The Ecostrip has one master control outlet and five controlled outlets. The absence of an uncontrolled outlet was a limitation of this product during our field study. We plugged the computer into the control outlet, and the LCD monitor, multi-functional device, and speakers into the controlled outlets. A USB cord also connected the computer to the Ecostrip. When the user manually powers down the computer, power to the controlled outlets is then disconnected. Because the system’s use needed the modem and wireless router to remain operational even when the desktop computer was powered down, the user had to plug the router into a regular power strip so that it would stay connected to the power source even when the computer was off. For metering, we connected both the Ecostrip, with its associated devices, and the regular plug strip with the modem and router to a Watts Up PRO ES power meter.

User behavior is a significant factor in duty cycle. Based on an interview with the user, we determined that the computer was turned off every night and that a typical work day for this user consisted of 9 hours. Error: Reference source not found and Error: Reference source not found show how the duty cycles of these devices changed with a USB smart plug strip compared to the baseline case.

Figure 13: Base Case: Home Office without Smart Plug Strip

Figure 14: Home Office with USB Smart Plug Strip

Our data suggest that using the Ecostrip with the home office system that we metered would yield about 82 kWh per year in energy savings. Note that the speakers, LCD monitor and multi-function device were the only devices in the home office affected by the smart plug strip. The modem and the router were not connected to the smart plug strip and therefore were not affected.

Table 5: Energy savings with Ecostrip plug strip in home office

|  | Annual savings per unit |
| --- | --- |
| Energy Savings | 82 kWh |
| Utility bill savings | $8.20 |

* We assumed an average electric rate of 10 cents per kWh

The retail price of an Ecostrip smart plug strip is about $35. If we assume that the cost of a regular power strip is estimated at $20, the incremental cost of purchasing an Ecostrip smart plug strip is $15. In this case, the annual energy savings will pay for the incremental cost in two years. It is important to note that actual savings vary depending on the user behavior. In this case, reducing energy use depends on whether the computer is powered down by the user. The controlled devices will shut off only as often as the computer is shut off.

- Typical Energy Savings Scenarios

Results from our field studies indicate that smart plug strips can yield significant energy savings. However, as mentioned previously, energy savings can range widely depending on the user. In order to generalize the finding of our limited field studies, we constructed several scenarios based on average metered device duty cycles from previous field metering research (Foster Porter et al 2006). Below, we present results from modeling energy savings for a typical home entertainment system and a home office.

  - A Typical Home Entertainment System

For this analysis, we assume that a typical home entertainment system today includes an LCD television, an HD-DVR, a DVD player, and an audio receiver. Table 6 shows an example of how the duty cycles of these devices will change with a load-sensing plug strip. Note in this case that only the duty cycles of the DVD player and the receiver will change. The HD-DVR would not be connected to sensor-controlled outlet, and thus would not be affected. We estimate that he load-sensing smart plug strip will yield 21.9 kWh in energy savings per year. Table 7 and Table 8 show examples of how the duty cycles of these devices will change with a timer and with and occupancy-sensing plug strip. We assume that the timer shuts off the devices at night from 11 p.m. to 7 a.m. and for a few hours during day when the equipment is not needed. We estimate that a timer smart plug strip programmed as described would save 114.7 kWh per year. In the case of an occupancy-sensing plug strip, we assume that the plug strip will cut power to any loads attached to sensor-controlled outlets after a period of 30 minutes. The occupancy-sensing option is expected to save 37.8 kWh per year.

Table 6: Annual Energy Use of a Typical Home Entertainment System with and without a Load-Sensing Smart Plug Strip

| Device Type |  | Device Power by Mode |  |  | Base Case Duty Cycles & Energy Use |  |  |  | Duty Cycles & Energy Savings with Smart Plug Strip |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Active Power (W) | Low Power (W) | Standby Power (W) | Active Power Time (%) | Low Power Time (%) | Standby Mode Time (%) | Energy used per year (kWh) | Active Mode Time (%) | Low Power Time (%) | Standby Mode Time (%) | Off Mode Time (%) | Energy used per year (kWh) | Energy Savings (kWh) |
| Master/Control | LCD TV | 69.9 |  | 2.2 | 13% | 0% | 87% | 76.7 | 13% | 0% | 87% | 0% | 76.7 | - |
| Uncontrolled | HD-DVR | 37.6 |  | 34.0 | 29% | 0% | 71% | 307.0 | 29% | 0% | 71% | 0% | 307.0 | - |
| Controlled Outlets | Receiver Audio | 50.1 |  | 3.3 | 25% | 0% | 75% | 142.9 | 13% | 0% | 0% | 87% | 61.9 | 81.0 |
|  | DVD Player | 10.8 | 5.0 | 1.0 | 11% | 1% | 88% | 13.2 | 11% | 1% | 1% | 87% | 6.4 | 6.8 |
| Total Savings (kWh) | 87.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Smart Plug Strip Incremental Energy Use(kWh) | 8.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Net Savings (kWh) | 79 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sources: Assumptions for duty cycles and power use per device are from Foster Porter et al. 2006. The HD-DVR duty cycle is from NRDC 2005. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7: Annual Energy Use of Typical Home Entertainment System with and without a Timer Smart Plug Strip

| Device Type |  | Device Power by Mode |  |  | Base Case Duty Cycles & Energy Use |  |  |  | Duty Cycles and Energy Savings with Timer |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Active Power (W) | Low Power (W) | Standby Power (W) | Active Mode Time (%) | Low Power Mode Time (%) | Standby Mode Time (%) | Energy used per year (kWh) | Active Mode Time (%) | Low Power Time (%) | Standby Mode Time (%) | Off Mode Time (%) | Energy used per year (kWh) | Energy Savings (kWh) |
| Controlled Outlets | LCD TV | 69.9 | - | 2.2 | 13% | 0% | 87% | 76.7 | 13% | 0% | 45% | 42% | 68.5 | 8.2 |
|  | HD-DVR | 37.6 | - | 34.0 | 29% | 0% | 71% | 307.0 | 29% | 0% | 29% | 42% | 217.2 | 89.8 |
|  | Receiver Audio | 50.1 | - | 3.3 | 25% | 0% | 75% | 142.9 | 25% | 0% | 33% | 42% | 129.5 | 13.4 |
|  | DVD Player | 10.8 | 5.0 | 1.0 | 11% | 1% | 88% | 13.2 | 11% | 1% | 46% | 42% | 9.9 | 3.3 |
| Total Savings | 114.7 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| *Sources: Assumptions for duty cycles and power demands per device are from Foster Porter et al. 2006. The HD-DVR duty cycle is from NRDC 2005 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 8: Annual Energy Use of Typical Home Entertainment System with and without an Occupancy-Sensor Smart Plug Strip

| Device Type |  | Power Demand by Mode |  |  | Base Case Duty Cycles and Energy Use |  |  |  | Duty Cycles and Energy Savings with Occupancy Sensor Smart Plug Strip |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Active Power (W) | Low Power (W) | Standby Power (W) | Active Power Time (%) | Low Power Time (%) | Standby Mode Time (%) | Energy used per year (kWh) | Active Mode Time (%) | Low Power Time (%) | Standby Mode Time (%) | Off Mode Time (%) | Energy used per year (kWh) | Energy Savings (kWh) |
| Uncontrolled | HD-DVR | 37.6 |  | 34.0 | 29% | 0% | 71% | 307.0 | 29% | 0% | 71% | 0% | 307.0 | - |
| Controlled Outlets | LCD TV | 69.9 |  | 2.2 | 13% | 0% | 87% | 76.7 | 13% | 0% | 2% | 85% | 60.1 | 16.6 |
|  | Receiver Audio | 50.1 |  | 3.3 | 25% | 0% | 75% | 142.9 | 15% | 0% | 0% | 85% | 71.4 | 71.5 |
|  | DVD Player | 10.8 | 5.0 | 1.0 | 11% | 1% | 88% | 13.2 | 11% | 1% | 3% | 85% | 6.5 | 6.7 |
| Total Savings | 94.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Smart Plug Strip Incremental Energy Use(kWh) | 8.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Net Savings | 86 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| *Sources: Assumptions for duty cycles and power demand per device are from Foster Porter et al. 2006. The HD-DVR duty cycle is from NRDC, 2005 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- Typical Home Office

To estimate the energy savings achievable with a smart plug strip in a typical home office when using a smart plug strip, we assumed that typical home office consists of a desktop computer, an LDC monitor, two computer speakers, one modem, and one multi-function device. We modeled the impact of a load-sensing smart plug strip with the computer connected to the control outlet; the monitor, speakers and multi-function device connected to the controlled outlet; and the modem connected to the uncontrolled outlet. The peripheral devices are disconnected from power when the computer is powered down. Therefore, the duty cycles of the computer and modem remain the same with and without the smart plug strip, but the duty cycles of the peripheral devices do change resulting in energy savings.

Using this scenario, we estimated 42.3 kWh of savings per year. Table 9 shows how the duty cycles of these devices in a typical home office will change with the load-sensing option compared to the baseline without the outlet control.

Table 9: Annual Energy Use of Typical Home Office with and without a Load-Sensing Smart Plug Strip

| Device Type |  | Device Power by Mode |  |  | Base Case Duty Cycles and Energy Use |  |  |  |  | Duty Cycles and Energy Savings with Load-Sensing Smart Plug Strip |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Active Power (W) | Low Power (W) | Standby Power (W) | Active Power Time (%) | Low Power Time (%) | Standby Mode Time (%) | OFF Mode Time (%) | Energy used per year (kWh) | Active Mode Time (%) | Low Power Time (%) | Standby Mode Time (%) | OFF Mode Time (%) | Energy used per year (kWh) | Energy Savings (kWh) |
| Master/Control | Desktop Computer | 69.7 | 17.2 | 4.4 | 35% | 6% | 59% | 0% | 255.2 | 35% | 6% | 59% | 0% | 255.2 | 0 |
| Uncontrolled | Modem | 6.4 | - | - | 100% | 0% | 0% | 0% | 60.4 | 100% | 0% | 0% | 0% | 60.4 | 0 |
| Controlled Outlets | LCD Monitor | 69.9 | 1.7 | 2.2 | 26% | 1% | 73% | 0% | 69.6 | 26% | 1% | 14% | 59% | 63.5 | 6.1 |
|  | Computer Speakers | 8.6 | - | 5.5 | 22% | 0% | 78% | 0% | 21.7 | 22% | - | 19% | 59% | 9.4 | 12.3 |
|  | Multi-function device | 15.2 | 9.1 | 6.2 | 3% | 7% | 83% | 7% | 54.7 | 3% | 7% | 31% | 59% | 22.0 | 28.8 |
| Total Savings | 47.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Smart Plug Strip Incremental Energy Use (kWh) | 8.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Net Savings | 38.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: Assumptions for duty cycles and power demand by mode are from Foster Porter et al. 2006. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- Market Barriers

Despite significant energy savings, several barriers exist to widespread market penetration of smart plug strips. The first is simply a lack of awareness. Consumers are still largely unfamiliar with these devices. Aggressive consumer education campaigns on the energy use of plug loads and the energy savings achievable with smart plug strips is needed. Another market barrier in commercial settings is a lack of clarity on which departments or staff members are responsible for purchasing and installing smart plug strips. As described by Krepchin (Krepchin 1998), one department may be responsible for buying surge protectors and another department may be responsible for buying energy-saving devices. Who is the decision maker regarding smart plug strips?

- Existing Utility Incentive Programs

During this study, we identified three North American utility incentive programs for smart plug strips.

- BC Hydro

BC Hydro provides financial incentives to organizations that replace inefficient products with energy-efficient technologies, or add products onto existing systems to make them more efficient. This program incentive applies to smart plug strips used for commercial computer workstations. BC Hydro claims 100 kWh per year per unit in savings, and offers a $7 incentive per qualifying unit (BC Hydro 2008b).

- Rocky Mountain Power

Rocky Mountain Power offers an incentive for passive infrared and/or ultrasonic sensors for use with electric plug-load applications (e.g. computer monitors, desk lamps, etc.). New construction & retrofit applications are eligible. Rocky Mountain Power offers a $15 incentive per qualifying unit (Rocky Mountain Power 2008).

- Idaho Power

Idaho Power’s Easy Upgrades program is designed to provide incentives for energy-savings retrofits to commercial and industrial facilities. Ten-dollar incentives are available for qualifying models of occupancy-sensor smart plug strips (Idaho Power 2008).

- Conclusion and Recommendations

Consumers can save energy cost-effectively by using most varieties of smart plug strips. Estimates of exactly how much energy can be saved with smart plug strips vary, and are highly dependent on the type of smart plug strip used, which devices are connected to the smart plug strip, how the devices are connected to the smart plug strip, how the user operates the devices that are connected smart plug strip, and, in the case of a remote control model, how the user operates the smart plug strip itself. Each smart plug strip technology has advantages and limitations, and no one model proved to be superior across all applications. Occupancy sensors are prone to false triggering. Load-sensing plug strips are dependent on the power state of the control device; the controlled devices will be shut off only as often as the controls are powered down. Remote control smart plug strips depend on the user remembering to use the remote. And no smart plug strips will save energy if they are improperly installed, or purchased but not used because they do not meet consumers’ needs.

Based on this scoping study, we have the following recommendations for manufacturers, consumers, and utilities or other organizations seeking to achieve energy savings though incentives and promotions of smart plug strips.

Recommendations for manufacturers:

Incorporate always-on outlets - Some devices need to be able to operate even if the rest of the system is off.

Incorporate programmable timers in conjunction with occupancy and load sensor controls –Our findings showed that programmable timers used in conjunction with occupancy and load sensors increased the smart plug strip’s ability to meet consumer needs and thereby increased energy savings. Note that after our research was completed, we learned of two recently available models of smart plug strips –a programmable timer model (the Intermatic Power Strip with Digital Timer) and a model with software controls (the Checktap).

Recommendations for consumers:

Pick a product with an adjustable time delay. Smart power strip products may come with either adjustable or manufacturer-set time delays that determine how long a period of non-occupancy or inactivity must be before the equipment is turned off. A product with an adjustable time delay allows adjustment of the smart power strips to suit individual habits.

Pick a product with low power demand. Smart plug strips draw a small amount of power themselves. The smart plug strips we metered drew between 0.004 and 1.8 watts when no devices were plugged into them. While energy savings will typically outweigh any energy consumed by the smart plug strips, it is important to note that there are a variety of power demands associated with different models of smart plug strips.

Recommendations for smart plug strip program administrators:

Educate consumers about the installation and operation of promoted smart plug strips. Smart plug strip will not achieve estimated energy savings if they are improperly installed and/or improperly operated. For example, if computers are connected to the controlled outlets, they will turn off with each switch cycle, potentially corrupting files, losing data, and frustrating the user. Some devices need to power down through a series of internally programmed steps and should not have the power abruptly disconnected.

  - Suggestions for Future Research

Future research should investigate how more than one control technology could be integrated in one product, and what the additional energy savings would be. In addition, the effect of integrating smart plug strips into offices and home theater furniture warrants further consideration.

To our knowledge, beyond the case studies presented in this report, very few plug load field monitoring studies using smart plug strips have been done. Field monitoring studies provide highly detailed information on product performance and consumer behavior that is essential to the design of effective efficiency programs and policies. Further research is needed to document the impacts of smart plug strips over time. Findings from these studies would allow utilities and governments to develop plug load energy reduction policies and programs based on quantitative field data and qualitative consumer acceptance surveys rather than on hypothesis or laboratory data alone. Further research is also needed to determine potential future linkages of smart plug strips to wired/wireless control interfaces for buildings, including ZigBee, Zwave, XIO and Insteon.

- References

BC Hydro. 2008a. Smart Power Strips. Retrieved from http://www.uppco.com/business/PDF/P_PA_3.pdf)(http://www.bchydro.com/powersmart/technology_tips/buying_guides/office_equipment/smart_power_strips.html. December 15, 2008.

BC Hydro. 2008b. Power Smart Product Incentives. Retrieved from http://www1a.bchydro.com/ecatalog/PromoList.jsp?category=14. December 15, 2008.

BC Hydro. 2008c. Conversation with David Rogers, BC Hydro, 07/15/2008.

Ecostrip. 2008. Retrieved from http://www.theecostrip.com/

Foster Porter, S., L. Moorefield and P. May-Ostendorp. 2006. Final Field Research Report. Prepared for the California Energy Commission by Ecos Consulting.

Home Energy. 2008. The USB Ecostrip. May/June 2008. p.45

Idaho Power. 2008. Easy Upgrades for Simple Retrofits. Retrieved from http://www.idahopower.com/easyupgrades/. December 15, 2008.

IT-Energy APS. 2007. Standby and Energy Saving Sockets. A Status for Denmark.

Kepchin, I. 1998. Smart Plug Strips Get Smart. E Source. ER 98-4

Natural Resources Defense Council. 2005. Tuning in to Energy Efficiency: Prospects for Energy Savings in TV Set-Top Boxes. Prepared by Ecos Consulting.

Pacific Gas and Electric Company. 2007. Analysis of Standards Options for Modular Office Furniture Occupancy Sensors. Prepared by Ecos Consulting.

Rocky Mountain Power. 2008. Other Energy Efficiency Technologies. Retrieved from http://www.usatech.com/energy_management/downloads/rebate_pdfs/RMP_Catalog_2008-02.pdf. December 15, 2008.

WattStopper. 1997. Energy Savings with Desktop Office Power Control. Technical Bulletin.

WattStopper. 2008. Retrieved from http://www.wattstopper.com/products/details.html?id=74. December 15. 2008.