# Affordable Housing New Construction Program Impact Evaluation Report

Energy Efficiency and Demand Response Plan

Program Year 2025 (CY2025)

(01/01/2025-12/31/2025)

Prepared for:

Nicor Gas Company

DRAFT

March 4, 2026

Prepared by:

| Ryan Kroll Driftless Energy |  |  |  |
| --- | --- | --- | --- |

Submitted to:

Nicor Gas Company

1844 Ferry Road

Naperville, IL 60563

Submitted by:

Guidehouse Inc.

167 N. Green Street

12th Floor

Chicago, IL 60607

Contact:

| Charles Maglione, Partner 202.481.7352 cmaglione@guidehouse.com | Jeff Erickson, Director 608.616.4962 jeff.erickson@guidehouse.com | Laura Agapay-Read, Associate Director 312.583.4178 laura.agapay.read@guidehouse.com |
| --- | --- | --- |

Charles Ampong, Associate Director

608.616.4922

charles.ampong@guidehouse.com

This report was prepared by Guidehouse for Nicor Gas Company. The work presented in this report represents Guidehouse’s professional judgment based on the information available at the time this report was prepared. Use of this report by any other party for whatever purpose should not, and does not, absolve such party from using due diligence in verifying the report’s contents. Neither Guidehouse nor any of its subsidiaries or affiliates assumes any liability or duty of care to such parties, and hereby disclaims any such liability.

Table of Contents

1	Introduction	1

2	Program Description	1

3	Program Savings Detail	2

4	Program Savings by Measure	2

5	Impact Analysis Findings and Recommendations	3

Appendix A. Impact Analysis Methodology	4

Appendix B. Program Specific Inputs for the Illinois TRC	7

Table of Tables, Figures, and Equations

Table 1. 2025 Volumetric Findings Detail	1

Table 2. 2025 Installed Measure Quantities	1

Table 3. 2025 Annual Energy Savings Summary	2

Table 4. 2025 Annual Energy Savings by Measure	3

# Introduction

This report presents the results of the impact evaluation of the Nicor Gas 2025 Affordable Housing New Construction program. It presents a summary of the energy impacts for the total program and for relevant measure and program structure details. The appendix presents the impact analysis methodology. Program year 2025 covers January 1 to December 31, 2025 (2025). The program was designed to provide technical assistance and incentives for energy efficient construction and major renovation of affordable multifamily housing. The program targets developers and owners constructing housing for households with incomes at or below 80% of the area median income. The program also aims to educate developers on cost-effective energy efficient building practices. The program has two participation levels: (1) major renovation and (2) new multifamily. The AHNC Program is offered jointly to affordable housing developers and owners served by ComEd and Nicor Gas, where their service territories overlap.

# Program Description

In 2025, the AHNC Program had 24 total projects with 1,030 total dwelling units, 994 or which are income eligible units, as Table 1 shows. Nicor Gas served five of these projects, including 119 total and income-eligible units.

Table 1. 2025 Volumetric Findings Detail

| Participation | Quantity Total (ComEd and Nicor Gas) | Quantity (Nicor Gas) |
| --- | --- | --- |
| Projects* | 24 | 5 |
| Number of Affordable Units† | 994 | 119 |
| Number of Market Rate Units | 36 | 0 |
| Number of Total Units‡ | 1,030 | 119 |
| Building Area (square feet) | 1,289,344 | 173,796 |

* Participants are defined as completed projects

† Affordable units are defined as income-eligible dwelling units.

‡ Total units are defined as total of income-eligible and market rate dwelling units.

Source: Nicor Gas tracking data and evaluation team analysis.

Natural gas savings for these projects were achieved by improvements to HVAC, shell, appliances, and hot water end use types. Table 2 summarizes the installed measure quantities that are the basis for verified energy savings.

Table 2. 2025 Installed Measure Quantities

| Program Category | Measure | Quantity Unit | Installed Quantity |
| --- | --- | --- | --- |
| Shell | High-Performance Windows | Projects | 4 |
| Shell | Reduced Infiltration | Projects | 4 |
| HVAC | High-Performance HVAC Equipment | Projects | 4 |
| HVAC | Advanced HVAC Controls | Projects | 2 |
| Hot Water | High-Performance Water Heating Equipment | Projects | 4 |
| Hot Water | Hot Water Conservation | Projects | 4 |
| Appliances | Efficient Appliances | Projects | 4 |

Source: Nicor Gas tracking data and evaluation team analysis.

# Program Savings Detail

Table 3 summarizes the gas energy savings the Affordable Housing New Construction Program achieved by project 2025. The program completed five projects with natural gas savings in 2025, with project realization rates (RRs) ranging from 87% to 105%. The realization rate is the ratio of the verified savings to the ex ante savings. The overall 2025 program realization rate for the Nicor Gas AHNC program was 91%.

Table 3. 2025 Annual Energy Savings Summary

| Project | Ex Ante Gross Savings (Therms) | Verified Gross RR* | Verified Gross Savings (Therms) | NTG† | NSPO‡ | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- |
| 252446 | 4,252 | 100% | 4,249 | 1.00 | N/A | 4,249 |
| 257123 | 2,904 | 105% | 3,046 | 1.00 | N/A | 3,046 |
| 250181 | 20,088 | 87% | 17,379 | 1.00 | N/A | 17,379 |
| 267616 | 608 | 100% | 609 | 1.00 | N/A | 609 |
| 0086 | 888 | 98% | 872 | 1.00 | N/A | 872 |
| Total or Weighted Average | 28,741 | 91% | 26,154 | 1.00 | N/A | 26,154 |

* Realization Rate (RR) is the ratio of verified gross savings to ex ante gross savings, based on evaluation research findings.

† A deemed value. Available on the SAG web site: https://www.ilsag.info/ntg.

Source: Evaluation team analysis.

† NTG, Net to Gross is the deemed value available on the SAG website: https://www.ilsag.info/evaluator-ntg-recommendations-for-2024/.

‡ The market rate net savings residential non-participant spillover (NPSO) factor of 1.048 is not applicable to this program.

Source: Evaluation team analysis.

# Program Savings by Measure

The 2025 AHNC program includes seven measure categories as shown in the following table. High-performance water heating equipment contributed the most savings, at 39% of the program natural gas savings. High performance HVAC equipment is the next largest category, at 34% of the program savings.

Table 4. 2025 Annual Energy Savings by Measure

| Program Category | Savings Category | Ex Ante Gross Savings (Therms) | Verified Gross RR* |  | Verified Gross Savings (Therms) |  | NTG† | NSPO‡ | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Shell | High-Performance Windows | 402 |  | 108% | 433 | 1.00 |  | N/A | 433 |
| Shell | Reduced Infiltration | 789 |  | 98% | 771 | 1.00 |  | N/A | 771 |
| HVAC | High-Performance HVAC Equipment | 9,655 |  | 100% | 9,655 | 1.00 |  | N/A | 9,655 |
| HVAC | Advanced HVAC Controls | 2,983 |  | 8% | 239 | 1.00 |  | N/A | 239 |
| Hot Water | High-Performance Water Heating Equipment | 11,223 |  | 100% | 11,227 | 1.00 |  | N/A | 11,227 |
| Hot Water | Hot Water Conservation | 3,463 |  | 104% | 3,604 | 1.00 |  | N/A | 3,604 |
| Appliances | Efficient Appliances | 225 |  | 100% | 225 | 1.00 |  | N/A | 225 |
| Total or Weighted Average |  | 28,741 |  | 91% | 26,154 | 1.00 |  | N/A | 26,154 |

Source: Evaluation team analysis.

# Impact Analysis Findings and Recommendations

The evaluation team developed the following findings and recommendations based on our review of the five Nicor Gas projects. Appendix A provides a description of the impact analysis methodology.

Finding 1. The evaluation team adjusted the advanced HVAC control savings for project 250181. Based on the project documentation, the installed thermostats were programmable but not advanced thermostats. The manufacturer does indicate that the thermostats can controlled by third-party software interfaces (e.g. Alexa or IFTTT) but have no native occupancy or other advanced control capability either within the thermostat or within the associated app. Therefore, the advanced HVAC control savings for this project were set to zero, which reduced the advanced HVAC controls savings for the program by 92% and the overall program savings by 9.5%.

Recommendation 1.  Ensure that installed thermostats have native capability for occupancy or other advanced control capabilities.

Finding 2.  The evaluation team made minor adjustments to window areas, wall areas, and faucet aerator quantities and flow rates based on the provided documentation. Combined, these changes resulted in a very slight (0.5%) increase in the program savings.

Recommendation 2.  Consider adding quality control checks to ensure that claimed window areas, wall areas, equipment quantities and specifications are correct.

# Appendix A. Impact Analysis Methodology

The evaluation team conducted site-specific research to verify project savings that were not based on measures specified in the TRM. For the AHNC program, all completed projects with natural gas savings were reviewed.

Engineering Review of Project Files

For each selected project, an in-depth application review was performed to assess the engineering methods, parameters and assumptions used to generate all ex ante impact estimates. For each measure in the sampled project, evaluation team engineers estimated ex post gross savings based on their review of documentation and engineering analysis.

To support this review, the implementation contractor provided project documentation in electronic format for all projects. Documentation included some or all scanned files of hardcopy application forms and supporting documentation from the applicant (invoices, measure specification sheets, and vendor proposals), pre-inspection reports and photos, post inspection reports and photos, and calculation spreadsheets.

The ex ante natural gas savings for each measure are calculated using a template developed for the program. The template calculates the savings for each measure and piece of equipment based on the specifications for the individual equipment installed and the calculation approach specified in the Illinois TRM for the installed measure.  Savings are calculated based on the version of the IL-TRM that was in effect when the project applications were submitted. One project was based on IL-TRM v9.0, two projects were based on IL-TRM v10.0, one project was based on IL-TRM v11.0, and one was based on IL-TRM v12.0.

To calculate AHNC program verified gross savings, the evaluation team verified that the algorithms and deemed input parameters were applied correctly and validated any custom parameters through an examination of project documentation and actual equipment specifications. The evaluation team calculated verified net savings by multiplying the verified gross savings by the net-to-gross (NTG) ratio. Table A-1 presents the parameters used in the verified gross and net savings calculations and indicates the data sources used in the review.  Table A-2 presents measures and equipment reviewed for each project.

Table A-1. Equipment by Savings Category

| Measure | Parameters Reviewed | Data Source(s) |
| --- | --- | --- |
| High-Performance Windows | R_baseline, R_installed, Awindows, Eff_Cool, Eff_Heat | IL-TRM† – Section 5.6.4 IL-TRM† – Section 5.6.8 Project Documentation |
| Air Sealing - Reduced Infiltration | N_heat, N_cool, HDD, CDD,FLH_cooling, LM, ADJ_AirSealingCooling, ADJ_AirSealingHeatFan, IE_NetCorrection | IL-TRM† – Section 5.6.1 Project Documentation |
| High Efficiency Furnaces High Efficiency Boilers | Baseline_AFUE, Installed_AFUE, CAP_Input, FLH_Heating | IL-TRM† – Section 5.3.7 IL-TRM† – Section 5.3.6 Project Documentation |
| Advanced HVAC Controls (Smart Thermostats) | %Elec heat, CF_pjm, Fe, Heating_reduction, Household factor, Eff_ISR_heat, FLH, Cooling_reduction, Eff_IST_cool, Cooling_DemandReduction | IL-TRM† – Section 5.3.16, Project Documentation |
| High-Performance Water Heating Equipment | Baseline UEF, gallons per day, Household, T_in, T_out, Location factor, LM, waste heat portion resulting in cooling savings, waste heat increasing heating load, CF, Hours | IL-TRM† – Section 5.4.3 Project Documentation |
| Faucet Aerators, Showerheads, | GPM_base, L_base, L_low, faucets per household (FPH), drain factor (DF), EPG_electric, CF, NTG†, %DHW, gallons per hour (GPH), throttling factor | IL-TRM† – Section 5.4.4 Project Documentation |
| Clothes Dryers | Load, Ncycles, CF, CEF_base, %Electric | IL-TRM† – Section 5.1.10 Project Documentation |
| Clothes Washer | Ncycles, IMEF_base, %CW, %DHW, %dryer, Hours, IWF_base, CF | IL-TRM† – Section 5.1.2 Project Documentation |
| Dishwasher | Maximum kWh/year, Maximum gallons/cycle, %kWh_op, %kWh_heat, Hours, CF | IL-TRM† – Section 5.1.4 Project Documentation |

* Program Tracking Data (PTD) provided by Nicor Gas, extract dated January 30, 2025.

† State of Illinois Technical Reference Manual versions 7-13.0 from http://www.ilsag.info/technical-reference-manual.html.

‡ Project files and monthly billing data provided by Nicor Gas.

Table A-2. Equipment by Savings Category

| Measure Category | Equipment | Projects |
| --- | --- | --- |
| High-Performance Windows | High-Performance Windows | 252446, 250181, 267616, 0086 |
| Reduced Infiltration | Air-Sealing | 252446, 250181, 267616, 0086 |
| High-Performance HVAC Equipment | High Efficiency Furnaces High Efficiency Boilers | 252446, 250181, 267616, 0086 |
| Advanced HVAC Controls | Advanced Thermostats | 250181, 0086 |
| High-Performance Water Heating Equipment | In-Unit Gas Storage Water Heater In-Unit gas Tankless Water Heater Central Gas Water Heater | 252446, 257123, 250181, 267616 |
| Hot Water Conservation | Low-Flow Showerhead Bathroom Faucet Aerator Kitchen faucet Aerator | 252446, 257123, 250181, 267616 |
| Efficient Appliances | ENERGY STAR Clothes Washer ENERGY STAR Clothes Dryer ENERGY STAR Dishwasher | 252446, 257123, 250181, 267616 |

Source: Evaluation team analysis.

# Appendix B. Program Specific Inputs for the Illinois TRC

Table B-1 shows the Total Resource Cost (TRC) cost-effectiveness analysis inputs available at the time of producing this impact evaluation report. Additional required cost data (e.g., measure costs, program level incentive and non-incentive costs) are not included in this table and will be provided to the evaluation team later. Guidehouse will include annual and lifetime water savings and greenhouse gas reductions in the end of year summary report.

Table B-1. Verified Cost Effectiveness Inputs

| Savings Category | DAC Project* | Units | Quantity | Effective Useful Life | Early Replacement Flag | Verified Gross Annual Water Savings (Gallons) | Ex Ante Gross Savings (Therms) | Verified Gross Savings (Therms) | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| High-Performance Windows | TRUE | Projects | 4 | 40.0 | NO | - | 402 | 433 | 433 |
| Reduced Infiltration | TRUE | Projects | 4 | 20.0 | NO | - | 789 | 771 | 771 |
| High-Performance HVAC Equipment | TRUE | Projects | 4 | 19.3 | NO | - | 9,655 | 9,655 | 9,655 |
| Advanced HVAC Controls | TRUE | Projects | 2 | 11.0 | NO | - | 2,983 | 239 | 239 |
| High-Performance Water Heating Equipment | TRUE | Projects | 4 | 15.0 | NO | - | 11,223 | 11,227 | 11,227 |
| Hot Water Conservation | TRUE | Projects | 4 | 10.0 | NO | 3,357,043 | 3,463 | 3,604 | 3,604 |
| Efficient Appliances | TRUE | Projects | 4 | 13.3 | NO | 292,642 | 225 | 225 | 225 |
|  |  |  |  | 16.4 |  | 3,649,685 | 28,741 | 26,154 | 26,154 |

Source: Evaluation team analysis.