# Home Energy Savings Impact Evaluation Report

Energy Efficiency and Demand Response Plan

Program Year 2025 (CY2025)

(01/01/2025-12/31/2025)

Prepared for:

Nicor Gas Company

DRAFT

March 13, 2026

Prepared by:

| Julia Kaplan Guidehouse | Robert Hodge Guidehouse |  |  |
| --- | --- | --- | --- |

Submitted to:

Nicor Gas Company

1844 Ferry Road

Naperville, IL 60563

Submitted by:

Guidehouse Inc.

167 N. Green Street

12th Floor

Chicago, IL 60607

Contact:

| Charles Maglione, Partner 202.481.7352 cmaglione@guidehouse.com | Jeff Erickson, Director 608.616.4962 jeff.erickson@guidehouse.com | Laura Agapay-Read, Associate Director 312.583.4178 laura.agapay.read@guidehouse.com |
| --- | --- | --- |

Charles Ampong, Associate Director

608.616.4922

charles.ampong@guidehouse.com

This report was prepared by Guidehouse for Nicor Gas Company. The work presented in this report represents Guidehouse’s professional judgment based on the information available at the time this report was prepared. Use of this report by any other party for whatever purpose should not, and does not, absolve such party from using due diligence in verifying the report’s contents. Neither Guidehouse nor any of its subsidiaries or affiliates assumes any liability or duty of care to such parties, and hereby disclaims any such liability.

Table of Contents

1	Introduction	1

2	Program Description	1

3	Program Savings Detail	2

4	Program Savings by Measure	3

5	Impact Analysis Findings and Recommendations	6

5.1	Impact Parameter Estimates	6

5.2	Findings and Recommendations	7

Appendix A. Impact Analysis Methodology	9

Appendix B. Program Specific Inputs for the Illinois TRC	10

Table of Tables, Figures, and Equations

Table 1. 2025 Volumetric Findings Detail	1

Table 2. 2025 Installed Measure Quantities	2

Table 3. 2025 Annual Energy Savings Summary	3

Table 4. 2025 Annual Energy Savings by Measure	3

Table 5. Verified Gross Savings Parameters	6

Table 6. Measure Savings Discrepancies	7

# Introduction

This report presents the results of the impact evaluation of the Nicor Gas 2025 Home Energy Savings (HES) program. It presents a summary of the energy impacts for the total program and broken out by relevant measure and program structure details. The appendix presents the impact analysis methodology. Program year 2025 covers January 1 to December 31, 2025.

# Program Description

The HES program, jointly implemented by Nicor Gas and ComEd, includes an assessment and direct install (DI) component as well as a rebate component for air sealing, insulation, and duct sealing work completed by approved contractors. This report focuses on natural gas savings achieved by Nicor Gas program participants. Savings from electricity measures are included in a separate evaluation report delivered to ComEd.

The HES program provides a free home energy assessment (HEA) performed by an energy advisor. The energy advisor collects information about the home’s energy use by examining the heating system (e.g., furnace or boiler), cooling system (air conditioner), water heater, and attic (if accessible). The energy advisor provides a customized report with recommendations identifying additional ways the customer can save energy and money. As part of the energy assessment, and when appropriate, the energy advisor installs DI measures. The DI measures include showerheads, faucet aerators for bathrooms and kitchen, door sweep air sealing, and a programmable or advanced thermostat. A Self-Assessment Program (SAP) option was added in 2023.

In addition to the free HEA and free DI measures, the HES program offers rebates for prescriptive building shell air sealing and insulation (ASI) measures for eligible homes, installed by an approved contractor, including air sealing, attic insulation, duct sealing, basement sidewall insulation, and wall insulation. Air sealing includes sealing gaps and cracks in a wall where air can enter or exit. The contractor performs a blower door test to measure the air leakage in the home. To receive the instant discount for attic insulation, a participant must have air sealing and attic insulation installed at the same time.

The program had 1,272 participants in 2025 and completed 3,475 projects as shown in the following table.

Table 1. 2025 Volumetric Findings Detail

| Participation | Prescriptive | Self Assessment (SAP) | Total |
| --- | --- | --- | --- |
| Participants * | 742 | 530 | 1,272 |
| Installed Projects † | 1,620 | 1,855 | 3,475 |
| Measure Types Installed ‡ | 6 | 11 | 17 |

* Participants are defined as the distinct count of addresses.

† Installed Projects are defined as the distinct count of project IDs.

‡ Measure Types Installed are defined as the distinct count of Nicor Gas measure names.

Source: Nicor Gas tracking data and evaluation team analysis.

Table 2 summarizes the installed measure quantities that are the basis for verified energy savings.

Table 2. 2025 Installed Measure Quantities

| Program Path | Measure | Quantity Unit | Installed Quantity |
| --- | --- | --- | --- |
| Prescriptive | Air Sealing | Projects | 961 |
|  | Air Sealing Without Attic Insulation | Projects | 575 |
|  | Attic Insulation | Square Feet | 1,246,121 |
|  | Basement/Sidewall Insulation | Square Feet | 19,085 |
|  | Duct Sealing | Projects | 563 |
|  | Wall Insulation | Square Feet | 26,017 |
| Self Assessment (SAP) | Advanced Thermostat (DI) - Manual | Each | 105 |
|  | Advanced Thermostat (DI) - Programmable | Each | 155 |
|  | Air Sealing - Door Sweep - DI | Each | 2,360 |
|  | Bathroom Aerator SF (DI) | Each | 2,682 |
|  | Boiler Pipe Insulation | Linear Feet | 285 |
|  | DHW Pipe Insulation | Linear Feet | 3,873 |
|  | Handheld Showerhead (DI) SF | Each | 724 |
|  | Kitchen Aerator SF (DI) | Each | 941 |
|  | Programmable Thermostat (DI) | Each | 321 |
|  | Shower Flow Reducer | Each | 1,357 |
|  | Showerhead (DI) SF | Each | 383 |

Source: Nicor Gas tracking data and evaluation team analysis.

# Program Savings Detail

Table 3 summarizes the energy savings the HES Program achieved by path and population in 2025.

Table 3. 2025 Annual Energy Savings Summary

| Program Category | Program Path | Ex Ante Gross Savings (Therms) | Verified Gross RR* | Verified Gross Savings (Therms) | NTG† | NSPO‡ | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Non-Disadvantaged Communities | Prescriptive | 310,373 | 1.00 | 311,476 | 1.12 | 1.048 | 364,884 |
|  | SAP | 71,324 | 1.00 | 71,420 | 1.28 | 1.048 | 95,817 |
| Non-DAC Subtotal |  | 381,697 | 1.00 | 382,896 |  |  | 460,701 |
| Disadvantaged Communities | Prescriptive | 97,407 | 1.00 | 97,854 | 1.27 | 1.048 | 131,160 |
|  | SAP | 15,796 | 1.00 | 15,791 | 1.30 | 1.048 | 21,445 |
| DAC Subtotal |  | 113,203 | 1.00 | 113,645 |  |  | 152,617 |
| Total or Weighted Average |  | 494,900 | 1.00 | 496,541 |  |  | 613,318 |

* Realization Rate (RR) is the ratio of verified gross savings to ex ante gross savings, based on evaluation research findings.

† The program-level NTG values are calculated as Verified Net/Verified Gross. The program-level analysis typically used measure-level deemed NTG values, which are listed here: https://www.ilsag.info/evaluator-ntg-recommendations-for-2025.

‡ The Non-Disadvantaged Communities   net savings were multiplied by a residential non-participant spillover (NPSO) factor of 1.048. Based on SAG Policy, for DAC projects, if deemed NTG is multiplied by the NPSO and the resulted NTG value is less than 1.00, the evaluation assigned a DAC NTG of 1.00. If the resulted NTG value is >1.00, evaluation used the >1.00 value for calculation of net savings impact.

Source: Evaluation team analysis.

# Program Savings by Measure

The program includes 34 measures as shown in the following table. The Duct Sealing and Attic Insulation measures contributed the most savings.

Table 4. 2025 Annual Energy Savings by Measure

| Program Category | Program Path | Savings Category | Ex Ante Gross Savings (Therms) | Verified Gross RR* | Verified Gross Savings (Therms) | NTG† | NSPO‡ | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Non-Disadvantaged Communities | Prescriptive | Air Sealing | 56,191 | 1.00 | 56,201 | 0.88 | 1.048 | 51,831 |
|  |  | Air Sealing Without Attic Insulation | 32,504 | 1.00 | 32,510 | 1.21 | 1.048 | 41,225 |
|  |  | Attic Insulation | 89,867 | 1.00 | 89,903 | 0.89 | 1.048 | 83,854 |
|  |  | Basement/Sidewall Insulation | 13,205 | 1.00 | 13,208 | 1.35 | 1.048 | 18,687 |
|  |  | Duct Sealing | 116,074 | 1.01 | 117,123 | 1.35 | 1.048 | 165,706 |
|  |  | Wall Insulation | 2,532 | 1.00 | 2,531 | 1.35 | 1.048 | 3,581 |
|  | SAP | Advanced Thermostat (DI) - Manual | 9,771 | 1.01 | 9,897 | 0.96 | 1.048 | 9,957 |
|  |  | Advanced Thermostat (DI) - Programmable | 9,743 | 1.00 | 9,743 | 0.96 | 1.048 | 9,803 |
|  |  | Air Sealing - Door Sweep - DI | 9,816 | 1.00 | 9,816 | 1.35 | 1.048 | 13,887 |
|  |  | Bathroom Aerator SF (DI) | 1,893 | 1.00 | 1,886 | 1.45 | 1.048 | 2,866 |
|  |  | Boiler Pipe Insulation | 96.19 | 1.00 | 96.04 | 1.35 | 1.048 | 136 |
|  |  | DHW Pipe Insulation | 6,652 | 1.00 | 6,633 | 1.35 | 1.048 | 9,384 |
|  |  | Handheld Showerhead (DI) SF | 5,448 | 1.00 | 5,447 | 1.45 | 1.048 | 8,277 |
|  |  | Kitchen Aerator SF (DI) | 5,938 | 1.00 | 5,937 | 1.45 | 1.048 | 9,022 |
|  |  | Programmable Thermostat (DI) | 8,526 | 1.00 | 8,526 | 1.35 | 1.048 | 12,063 |
|  |  | Shower Flow Reducer | 10,514 | 1.00 | 10,512 | 1.45 | 1.048 | 15,975 |
|  |  | Showerhead (DI) SF | 2,927 | 1.00 | 2,926 | 1.45 | 1.048 | 4,447 |
| Non-DAC Subtotal |  |  | 381,697 | 1.00 | 382,896 |  |  | 460,701 |
| Disadvantaged Communities | Prescriptive | Air Sealing | 3,424 | 1.00 | 3,424 | 1.00 | N/A | 3,424 |
|  |  | Air Sealing Without Attic Insulation | 25,523 | 1.00 | 25,523 | 1.21 | 1.048 | 32,365 |
|  |  | Attic Insulation | 5,111 | 1.00 | 5,109 | 1.00 | N/A | 5,109 |
|  |  | Basement/Sidewall Insulation | 1,676 | 1.00 | 1,676 | 1.35 | 1.048 | 2,371 |
|  |  | Duct Sealing | 61,417 | 1.01 | 61,867 | 1.35 | 1.048 | 87,530 |
|  |  | Wall Insulation | 256 | 1.00 | 255 | 1.35 | 1.048 | 361 |
|  | SAP | Advanced Thermostat (DI) - Manual | 1,533 | 1.00 | 1,533 | 1.01 | N/A | 1,542 |
|  |  | Advanced Thermostat (DI) - Programmable | 2,058 | 1.00 | 2,058 | 1.01 | N/A | 2,070 |
|  |  | Air Sealing - Door Sweep - DI | 1,920 | 1.00 | 1,920 | 1.35 | 1.048 | 2,717 |
|  |  | Bathroom Aerator SF (DI) | 374 | 1.00 | 374 | 1.45 | 1.048 | 568 |
|  |  | Boiler Pipe Insulation | 34.45 | 1.00 | 34.30 | 1.35 | 1.048 | 48.53 |
|  |  | DHW Pipe Insulation | 1,155 | 1.00 | 1,152 | 1.35 | 1.048 | 1,629 |
|  |  | Handheld Showerhead (DI) SF | 1,243 | 1.00 | 1,242 | 1.45 | 1.048 | 1,887 |
|  |  | Kitchen Aerator SF (DI) | 1,328 | 1.00 | 1,328 | 1.45 | 1.048 | 2,018 |
|  |  | Programmable Thermostat (DI) | 3,528 | 1.00 | 3,529 | 1.35 | 1.048 | 4,992 |
|  |  | Shower Flow Reducer | 2,014 | 1.00 | 2,013 | 1.45 | 1.048 | 3,059 |
|  |  | Showerhead (DI) SF | 609 | 1.00 | 609 | 1.45 | 1.048 | 925 |
| DAC Subtotal |  |  | 113,203 | 1.00 | 113,645 |  |  | 152,617 |
| Total or Weighted Average |  |  | 494,900 | 1.00 | 496,541 |  |  | 613,318 |

† The program-level NTG values are calculated as Verified Net/Verified Gross. The program-level analysis typically used measure-level deemed NTG values, which are listed here: https://www.ilsag.info/evaluator-ntg-recommendations-for-2025.

‡ The Non-Disadvantaged Communities net savings were multiplied by a residential non-participant spillover (NPSO) factor of 1.048. Based on SAG Policy, for DAC projects, if deemed NTG is multiplied by the NPSO and the resulted NTG value is less than 1.00, the evaluation assigned a DAC NTG of 1.00. If the resulted NTG value is >1.00, evaluation used the >1.00 value for calculation of net savings impact (the NPSO in this case is left as N/A).

Source: Evaluation team analysis.

# Impact Analysis Findings and Recommendations

## Impact Parameter Estimates

Table 5 shows the unit therm savings and realization rate findings by measure from our review. The realization rate is the ratio of the verified savings to the ex ante savings. Following the table, we provide findings and recommendations, including discussion of all measures with realization rates above or below 100%. Appendix A provides a description of the impact analysis methodology.

Table 5. Verified Gross Savings Parameters

| Measure | Unit Basis | Ex Ante Gross (therms/unit) | Verified Gross (therms/unit) | Realization Rate | IL-TRM v13.0 Section† |
| --- | --- | --- | --- | --- | --- |
| Advanced Thermostat (DI) - Manual | Each | Varies | Varies | 101% | 5.3.16 |
| Advanced Thermostat (DI) - Programmable | Each | Varies | Varies | 100% | 5.3.16 |
| Air Sealing | Projects | Varies | Varies | 100% | 5.6.1 |
| Air Sealing - Door Sweep - DI | Each | Varies | Varies | 100% | 5.6.1 |
| Air Sealing Without Attic Insulation | Projects | Varies | Varies | 100% | 5.6.1 |
| Attic Insulation | Square Feet | Varies | Varies | 100% | 5.6.5 |
| Basement/Sidewall Insulation | Square Feet | Varies | Varies | 100% | 5.6.2 |
| Bathroom Aerator SF (DI) | Each | Varies | Varies | 100% | 5.4.4 |
| Boiler Pipe Insulation | Linear Feet | Varies | Varies | 100% | 5.3.2 |
| DHW Pipe Insulation | Linear Feet | Varies | Varies | 100% | 5.4.1 |
| Duct Sealing | Projects | Varies | Varies | 101% | 5.3.4 |
| Handheld Showerhead (DI) SF | Each | Varies | Varies | 100% | 5.4.5 |
| Kitchen Aerator SF (DI) | Each | Varies | Varies | 100% | 5.4.4 |
| Programmable Thermostat (DI) | Each | Varies | Varies | 100% | 5.3.11† |
| Shower Flow Reducer | Each | Varies | Varies | 100% | 5.4.5 |
| Showerhead (DI) SF | Each | Varies | Varies | 100% | 5.4.5 |
| Wall Insulation | Square Feet | Varies | Varies | 100% | 5.6.4 |

† State of Illinois Technical Reference Manual version 13.0 from http://www.ilsag.info/technical-reference-manual.html.

## Findings and Recommendations

Overall, the findings listed below had a minor impact on the savings, with the program-level realization rate being 100%.

Finding 1. Four instances of Duct Sealing showed a zero “GrossAnnualThermSavings” in the tracking data (MEA-2025.01.27-840760, MEA-2025.02.16-854098, MEA-2025.02.25-859176, MEA-2025.04.16-895722). Guidehouse calculated verified annual savings of 317.5, 450.3, 191.2, and 490.3 therms, respectively.

Recommendation 1. Ensure the “GrossAnnualThermSavings” column in the “Measures” tab of tracking data is populated for all measures, where participation is reported.

Finding 2. Three instances of Advanced Thermostat (DI) – Manual had a verified gross realization rate of 143% (MEA-2025.12.15-1407542, MEA-2025.12.15-1407543, MEA-2025.12.15-1407559). It appears that the ex ante calculation used a heating reduction proportion of 0.071 which is assigned to programmable thermostats in the TRM. In these instances, Guidehouse assigned a heating reduction proportion of 0.102 in verified savings.

Recommendation 2.  Ensure the heating reduction proportion values align with the applicable TRM measure section to accurately calculate “GrossAnnualThermSavings” column in the “Measures” tab of tracking data.

Finding 3. The program tracking data had a NTG value of 1.35 for Bathroom Aerator, Kitchen Aerator, Showerhead, and Shower Flow Reducer. The verified savings used 1.45 NTG, per the IL SAG Evaluator NTG Recommendations for 2025. This resulted in higher verified net savings.

Recommendation 3. Ensure the NTG values align with the SAG recommendation in “NTGRatioMeasure” column in the “Measures” tab of tracking data.

Finding 4. Guidehouse could not reconcile the savings for seven measures (shown in Table 6) where gross realization rates significantly deviate from 100%. Guidehouse used the savings parameters provided in the TRM to verify savings for these measures.

Recommendation 4.  Ensure the actual/custom values in the tracking data align with what is used for the ex ante savings calculation.

Table 6. Measure Savings Discrepancies

| Measure | Measure ID | Instances of Discrepancy |
| --- | --- | --- |
| Air Sealing | MEA-2024.12.31-830234 | 1 |
| Air Sealing Without Attic Insulation | MEA-2024.11.06-801306 | 1 |
| Attic Insulation | MEA-2024.12.31-830230 | 1 |
| Basement/Sidewall Insulation | MEA-2024.11.06-801305 | 1 |
| Bathroom Aerator SF (DI) | MEA-2025.12.31-1409063 | 1 |
| Boiler Pipe Insulation | MEA-2025.12.15-1406954, MEA-2025.12.31-1409167 | 2 |
| Duct Sealing | MEA-2025.05.16-914104, MEA-2025.05.16-914101 | 2 |

Source: Evaluation team analysis.

# Appendix A. Impact Analysis Methodology

The evaluation team used the same impact methodology for each component. Verified gross savings were determined for each program measure by:

- Reviewing the savings algorithm inputs in the final 2025 tracking data for agreement with the IL-TRM v13.0 and IL-TRM Errata, where applicable.

- Validating the savings algorithm was applied correctly.

- Cross-checking per-unit savings values in the program tracking data with the verified values in Guidehouse’s calculations.

- Multiplying the verified per-unit savings value by the quantity reported in the tracking data. The team calculated verified net savings by multiplying the verified gross savings estimates by a NTG ratio. In Program Year 2024, NTG estimates used to calculate the net verified savings were based on past evaluation research and defined by a consensus process through the Illinois SAG.

- For DAC projects, if deemed NTG is multiplied by 1.048 Non-Participant Spillover factor (NPSO) and the resulted NTG value is less than 1.00, the evaluation assigned a DAC NTG of 1.00. If the resulted NTG value is >1.00, evaluation used the >1.00 value for calculation of net savings impact.

# Appendix B. Program Specific Inputs for the Illinois TRC

Table B-1 shows the Total Resource Cost (TRC) cost-effectiveness analysis inputs available at the time of producing this impact evaluation report. Additional required cost data (e.g., measure costs, program level incentive and non-incentive costs) are not included in this table and will be provided to the evaluation team later. Guidehouse will include annual and lifetime water savings and greenhouse gas reductions in the end of year summary report.

Table B-1. Verified Cost Effectiveness Inputs

| Program Path | Savings Category | DAC Project* | Units | Quantity | Effective Useful Life | Early Replacement Flag | Verified Gross Annual Water Savings (Gallons) | Ex Ante Gross Savings (Therms) | Verified Gross Savings (Therms) | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Prescriptive | Air Sealing | FALSE | Projects | 911 | 20.00 | NO |  | 56,191 | 56,201 | 51,831 |
|  | Air Sealing | TRUE | Projects | 50 | 20.00 | NO |  | 3,424 | 3,424 | 3,424 |
|  | Air Sealing Without Attic Insulation | FALSE | Projects | 362 | 20.00 | NO |  | 32,504 | 32,510 | 41,225 |
|  | Air Sealing Without Attic Insulation | TRUE | Projects | 213 | 20.00 | NO |  | 25,523 | 25,523 | 32,365 |
|  | Attic Insulation | FALSE | Square Feet | 1,189,858 | 30.00 | NO |  | 89,867 | 89,903 | 83,854 |
|  | Attic Insulation | TRUE | Square Feet | 56,263 | 30.00 | NO |  | 5,111 | 5,109 | 5,109 |
|  | Basement/Sidewall Insulation | FALSE | Square Feet | 17,164 | 30.00 | NO |  | 13,205 | 13,208 | 18,687 |
|  | Basement/Sidewall Insulation | TRUE | Square Feet | 1,921 | 30.00 | NO |  | 1,676 | 1,676 | 2,371 |
|  | Duct Sealing | FALSE | Projects | 339 | 20.00 | NO |  | 116,074 | 117,123 | 165,706 |
|  | Duct Sealing | TRUE | Projects | 224 | 20.00 | NO |  | 61,417 | 61,867 | 87,530 |
|  | Wall Insulation | FALSE | Square Feet | 24,031 | 30.00 | NO |  | 2,532 | 2,531 | 3,581 |
|  | Wall Insulation | TRUE | Square Feet | 1,986 | 30.00 | NO |  | 256 | 255 | 361 |
| Self Assessment (SAP) | Advanced Thermostat (DI) - Manual | FALSE | Each | 90 | 11.00 | NO |  | 9,771 | 9,897 | 9,957 |
|  | Advanced Thermostat (DI) - Manual | TRUE | Each | 15 | 11.00 | NO |  | 1,533 | 1,533 | 1,542 |
|  | Advanced Thermostat (DI) - Programmable | FALSE | Each | 130 | 11.00 | NO |  | 9,743 | 9,743 | 9,803 |
|  | Advanced Thermostat (DI) - Programmable | TRUE | Each | 25 | 11.00 | NO |  | 2,058 | 2,058 | 2,070 |
|  | Air Sealing - Door Sweep - DI | FALSE | Each | 1,974 | 20.00 | NO |  | 9,816 | 9,816 | 13,887 |
|  | Air Sealing - Door Sweep - DI | TRUE | Each | 386 | 20.00 | NO |  | 1,920 | 1,920 | 2,717 |
|  | Bathroom Aerator SF (DI) | FALSE | Each | 2,238 | 10.00 | NO | 204,818 | 1,893 | 1,886 | 2,866 |
|  | Bathroom Aerator SF (DI) | TRUE | Each | 444 | 10.00 | NO | 44,234 | 374 | 374 | 568 |
|  | Boiler Pipe Insulation | FALSE | Linear Feet | 210 | 11.00 | NO |  | 96.19 | 96.04 | 136 |
|  | Boiler Pipe Insulation | TRUE | Linear Feet | 75 | 11.00 | NO |  | 34.45 | 34.30 | 48.53 |
|  | DHW Pipe Insulation | FALSE | Linear Feet | 3,300 | 12.00 | NO |  | 6,652 | 6,633 | 9,384 |
|  | DHW Pipe Insulation | TRUE | Linear Feet | 573 | 12.00 | NO |  | 1,155 | 1,152 | 1,629 |
|  | Handheld Showerhead (DI) SF | FALSE | Each | 590 | 10.00 | NO | 1,008,668 | 5,448 | 5,447 | 8,277 |
|  | Handheld Showerhead (DI) SF | TRUE | Each | 134 | 10.00 | NO | 229,943 | 1,243 | 1,242 | 1,887 |
|  | Kitchen Aerator SF (DI) | FALSE | Each | 769 | 10.00 | NO | 1,319,371 | 5,938 | 5,937 | 9,022 |
|  | Kitchen Aerator SF (DI) | TRUE | Each | 172 | 10.00 | NO | 295,144 | 1,328 | 1,328 | 2,018 |
|  | Programmable Thermostat (DI) | FALSE | Each | 230 | 16.00 | NO |  | 8,526 | 8,526 | 12,063 |
|  | Programmable Thermostat (DI) | TRUE | Each | 91 | 16.00 | NO |  | 3,528 | 3,529 | 4,992 |
|  | Shower Flow Reducer | FALSE | Each | 1,139 | 10.00 | NO | 1,946,764 | 10,514 | 10,512 | 15,975 |
|  | Shower Flow Reducer | TRUE | Each | 218 | 10.00 | NO | 372,839 | 2,014 | 2,013 | 3,059 |
|  | Showerhead (DI) SF | FALSE | Each | 317 | 10.00 | NO | 541,904 | 2,927 | 2,926 | 4,447 |
|  | Showerhead (DI) SF | TRUE | Each | 66 | 10.00 | NO | 112,710 | 609 | 609 | 925 |
|  |  |  |  |  | 20.97 |  | 6,076,394 | 494,900 | 496,541 | 613,318 |

Source: Evaluation team analysis.