# Income Eligible Multi-Family and Public Housing Impact Evaluation Report

Energy Efficiency Plan: Program Year 2025

(01/01/2025-12/31/2025)

Prepared for:

Nicor Gas Company

DRAFT

March 13, 2026

Prepared by:

| Katie Dolan EcoMetric | Swapnil Lotake EcoMetric | Fahman Khan EcoMetric | Mike Frischmann EcoMetric |
| --- | --- | --- | --- |

Submitted to:

Nicor Gas Company

1844 Ferry Road

Naperville, IL 60563

Submitted by:

Guidehouse Inc.

167 N. Green Street

12th Floor

Chicago, IL 60607

Contact:

| Charles Maglione, Partner 202.481.7352 cmaglione@guidehouse.com | Jeff Erickson, Director 608.616.4962 jeff.erickson@guidehouse.com | Laura Agapay-Read, Associate Director 312.583.4178 laura.agapay.read@guidehouse.com |
| --- | --- | --- |

Charles Ampong, Associate Director

608.616.4922

charles.ampong@guidehouse.com

This report was prepared by Guidehouse for Nicor Gas Company. The work presented in this report represents Guidehouse’s professional judgment based on the information available at the time this report was prepared. Use of this report by any other party for whatever purpose should not, and does not, absolve such party from using due diligence in verifying the report’s contents. Neither Guidehouse nor any of its subsidiaries or affiliates assumes any liability or duty of care to such parties, and hereby disclaims any such liability.

Table of Contents

1	Introduction	2

2	Multi-Family Income Eligible Program	2

2.1	Program Description	2

2.2	Program Savings Detail	4

2.3	Program Savings by Measure	4

2.4	Impact Analysis Findings and Recommendations	7

2.4.1	Impact Parameter Estimates	7

2.4.2	Findings and Recommendations	8

3	Public Housing Energy Savings Program	13

3.1	Program Description	13

3.2	Program Savings Detail	13

3.3	Program Savings by Measure	14

3.4	Impact Analysis Findings and Recommendations	14

3.4.1	Impact Parameter Estimates	14

3.4.2	Findings and Recommendations	15

Appendix A. Impact Analysis Methodology	16

Appendix B. Program Specific Inputs for the Illinois TRC	17

Table of Tables

Table 1. 2025 Volumetric Findings Detail – Income Eligible	2

Table 2. 2025 Installed Measure Quantities – Income Eligible	2

Table 3. 2025 Annual Energy Savings Summary – Income Eligible	4

Table 4. 2025 Annual Energy Savings by Measure – Income Eligible	5

Table 5. Verified Gross Savings Parameters – Income Eligible	7

Table 6. Measure Instances with Unidentified Discrepancies	10

Table 7. 2025 Volumetric Findings Detail – Public Housing	13

Table 8. 2025 Installed Measure Quantities – Public Housing	13

Table 9. 2025 Annual Energy Savings Summary – Public Housing	14

Table 10. 2025 Annual Energy Savings by Measure – Public Housing	14

Table 11. Verified Gross Savings Parameters – Public Housing	15

# Introduction

This report presents the results of the impact evaluation of the Nicor Gas 2025 Multi-Family Income Eligible (MFIE) and Public Housing (PHA) programs. The report is separated into two sections. The IE section includes the Retrofits, Assessment, Illinois Home Weatherization Assistance Program (IHWAP), Self-Assessment Program (SAP) and Healthy Homes (HH) programs. The second section presents results for the PHA program. This report presents a summary of the energy impacts for the total programs and is broken out by relevant measure and program structure details. Appendix A presents the impact analysis methodology. Appendix B presents the Total Resource Cost (TRC) cost-effectiveness analysis inputs. Program year 2025 covers January 1 to December 31, 2025.

# Multi-Family Income Eligible Program

## Program Description

The Nicor Gas MFIE Program offers products and energy saving measures for income-qualified customers in multi-family dwellings within the Nicor Gas service territory. The 2025 Retrofits, Assessments, IHWAP, SAP and Healthy Homes includes direct installation of water efficiency measures (faucet aerators, showerheads, shower timers, controls for domestic hot water), HVAC efficiency measures (advanced thermostats, programmable and reprogrammable thermostats, furnace, boiler controls, steam traps, boiler and furnace tune-ups, pipe insulation, boiler pipe insulation), and building shell measures (air sealing, attic insulation, basement insulation, duct insulation and sealing, wall insulation).

The program had 2,554 participants in 2025 and completed 5,412 projects as shown in the following table.

Table 1. 2025 Volumetric Findings Detail – Income Eligible

| Participation | Retrofits | Healthy Homes | IHWAP | SAP | Total |
| --- | --- | --- | --- | --- | --- |
| Participants * | 275 | 2 | 2 | 109 | 388 |
| Installed Projects † | 3,073 | 2 | 16 | 1 | 3,092 |
| Measure Types Installed ‡ | 43 | 9 | 12 | 16 | 80 |

* Participants are defined as distinct counts of building account numbers.

† Installed Projects are defined as distinct count of project ID.

‡ Measure Types Installed are the distinct count of Nicor measure names

Source: Nicor Gas tracking data and evaluation team analysis.

Table 2 summarizes the installed measure quantities that are the basis for verified energy savings.

Table 2. 2025 Installed Measure Quantities – Income Eligible

| Program Path | Measure | Quantity Unit | Installed Quantity |
| --- | --- | --- | --- |
| Retrofits | Air Filter Replacement | Unit | 82 |
|  | Air Sealing | LN FT | 532,548 |
|  | Assessment/No Savings | Unit | 244,215 |
|  | Attic Insulation | SQ FT | 719,734 |
|  | Basement/Sidewall Insulation | Unit | 805 |
|  | Boiler Reset Controls | Unit | 1 |
|  | Boiler Tune Up | Unit | 105 |
|  | Boilers | Unit | 1 |
|  | Commercial Gas Heat Pump | Unit | 4 |
|  | Controls for Domestic Hot Water | Unit | 628 |
|  | Cover and Gap Sealers for AC | Unit | 234 |
|  | Duct Insulation | Unit | 682 |
|  | Duct Sealing | Unit | 29,927 |
|  | Furnace Tune-Up | Unit | 1 |
|  | Low Flow Aerator- Bathroom (IU) | Unit | 759 |
|  | Low Flow Aerator- Kitchen (IU) | Unit | 1,187 |
|  | Low Flow Showerheads (IU) | Unit | 1,437 |
|  | Pipe Insulation | LN FT | 22,147 |
|  | Programmable Thermostats | Unit | 402 |
|  | Reprogrammable Thermostat | Unit | 29 |
|  | Shower Timer | Unit | 2,601 |
|  | Steam Boiler Averaging Controls | Unit | 72 |
|  | Steam Trap | Unit | 318 |
|  | Storage Tank Insulation | SQ FT | 191 |
| IHWAP | Air Sealing | LN FT | 4,262 |
|  | Assessment/No Savings | Unit | 151 |
|  | Attic Insulation | SQ FT | 19,084 |
|  | High Efficiency Furnace - IU | Unit | 48 |
|  | Low Flow Aerator - Bathroom (IU) | Unit | 21 |
|  | Low Flow Aerator - Kitchen (IU) | Unit | 26 |
|  | Low Flow Showerheads (IU) | Unit | 17 |
|  | Programmable Thermostats | Unit | 48 |
|  | Wall Insulation | SQ FT | 22,848 |
| SAP | Advanced Thermostats | Unit | 67 |
|  | Air Sealing | Unit | 122 |
|  | Assessment/No Savings | Unit | 109 |
|  | Boiler Pipe Insulation | LN FT | 18 |
|  | DHW Pipe Insulation | LN FT | 150 |
|  | Low Flow Aerator - Bathroom (IU) | Unit | 129 |
|  | Low Flow Aerator - Kitchen (IU) | Unit | 53 |
|  | Low Flow Showerheads (IU) | Unit | 130 |
|  | Programmable Thermostats | Unit | 9 |
|  | Water Heater Temperature Setback | Unit | 24 |
| Healthy Homes | Advanced Thermostats | Unit | 1 |
|  | Air Filter Replacement | Unit | 1 |
|  | Air Purifier/Cleaner | Unit | 2 |
|  | Air Sealing | LN FT | 230 |
|  | Assessment/No Savings | Unit | 15 |
|  | Duct Sealing | Unit | 1 |
|  | Furnace Tune-Up | Unit | 1 |

Source: Nicor Gas tracking data and evaluation team analysis.

## Program Savings Detail

Table 3 summarizes the energy savings the Multifamily Income Eligible Program achieved by path in 2025.

Table 3. 2025 Annual Energy Savings Summary – Income Eligible

| Program Category | Program Path | Ex Ante Gross Savings (Therms) | Verified Gross RR* | Verified Gross Savings (Therms | NTG† | NSPO‡ | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Private | Retrofits | 817,877 | 98% | 804,656 | 1.00 | N/A | 804,656 |
|  | IHWAP | 23,630 | 53% | 12,457 | 1.00 | N/A | 12,457 |
|  | SAP | 5,794 | 86% | 4,960 | 1.00 | N/A | 4,960 |
|  | Healthy Homes | 569 | 100% | 569 | 1.00 | N/A | 569 |
| Total |  | 847,869 | 97% | 822,642 | 1.00 |  | 822,642 |

* Realization Rate (RR) is the ratio of verified gross savings to ex ante gross savings, based on evaluation research findings.

† NTG, Net to Gross is the deemed value available on the SAG website: https://www.ilsag.info/evaluator-ntg-recommendations-for-2025/.

‡The Non-Participant Spillover (NPSO) does not apply to this program.
Source: Evaluation team analysis.

## Program Savings by Measure

The program includes 61 broad measure categories as shown in the following table. The Air Sealing and Pipe Insulation measures contributed the most savings.

Table 4. 2025 Annual Energy Savings by Measure – Income Eligible

| Program Path | Savings Category | Ex Ante Gross Savings (Therms) | Verified Gross RR* | Verified Gross Savings (Therms) | NTG† | NSPO‡ | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Retrofits | Air Filter Replacement | 829 | 100% | 829 | 1.00 | N/A | 829 |
|  | Air Sealing | 221,131 | 101% | 222,271 | 1.00 | N/A | 222,271 |
|  | Attic Insulation | 163,082 | 100% | 163,083 | 1.00 | N/A | 163,083 |
|  | Basement/Sidewall Insulation | 664 | 128% | 853 | 1.00 | N/A | 853 |
|  | Boiler Reset Controls | 798 | 100% | 798 | 1.00 | N/A | 798 |
|  | Boiler Tune Up | 22,630 | 100% | 22,630 | 1.00 | N/A | 22,630 |
|  | Boilers | 170 | 95% | 162 | 1.00 | N/A | 162 |
|  | Commercial Gas Heat Pump | 7,033 | 45% | 3,139 | 1.00 | N/A | 3,139 |
|  | Controls for Domestic Hot Water | 39,376 | 100% | 39,376 | 1.00 | N/A | 39,376 |
|  | Cover and Gap Sealers for AC | 1,689 | 100% | 1,689 | 1.00 | N/A | 1,689 |
|  | Duct Insulation | 402 | 100% | 402 | 1.00 | N/A | 402 |
|  | Duct Sealing | 40,872 | 74% | 30,128 | 1.00 | N/A | 30,128 |
|  | Furnace Tune-Up | 16 | 100% | 16 | 1.00 | N/A | 16 |
|  | Low Flow Aerator- Bathroom (IU) | 1,517 | 100% | 1,518 | 1.00 | N/A | 1,518 |
|  | Low Flow Aerator- Kitchen (IU) | 11,754 | 100% | 11,754 | 1.00 | N/A | 11,754 |
|  | Low Flow Showerheads (IU) | 19,565 | 100% | 19,559 | 1.00 | N/A | 19,559 |
|  | Pipe Insulation | 188,136 | 100% | 188,079 | 1.00 | N/A | 188,079 |
|  | Programmable Thermostats | 16,297 | 100% | 16,297 | 1.00 | N/A | 16,297 |
|  | Reprogrammable Thermostat | 1,176 | 100% | 1,176 | 1.00 | N/A | 1,176 |
|  | Shower Timer | 10,807 | 100% | 10,807 | 1.00 | N/A | 10,807 |
|  | Steam Boiler Averaging Controls | 4,398 | 100% | 4,398 | 1.00 | N/A | 4,398 |
|  | Steam Trap | 64,387 | 100% | 64,387 | 1.00 | N/A | 64,387 |
|  | Storage Tank Insulation | 1,148 | 114% | 1,306 | 1.00 | N/A | 1,306 |
|  | Retrofits Subtotal | 817,877 | 98% | 804,656 | 1.00 |  | 804,656 |
| IHWAP | Air Sealing | 2,373 | 100% | 2,373 | 1.00 | N/A | 2,373 |
|  | Attic Insulation | 1,777 | 53% | 933 | 1.00 | N/A | 933 |
|  | High Efficiency Furnace - IU | 15,362 | 41% | 6,235 | 1.00 | N/A | 6,235 |
|  | Low Flow Aerator- Bathroom (IU) | 42 | 100% | 42 | 1.00 | N/A | 42 |
|  | Low Flow Aerator- Kitchen (IU) | 257 | 100% | 257 | 1.00 | N/A | 257 |
|  | Low Flow Showerheads (IU) | 75 | 100% | 75 | 1.00 | N/A | 75 |
|  | Programmable Thermostats | 1,944 | 100% | 1,944 | 1.00 | N/A | 1,944 |
|  | Wall Insulation | 1,800 | 33% | 598 | 1.00 | N/A | 598 |
|  | IHWAP Subtotal | 23,630 | 53% | 12,457 | 1.00 |  | 12,457 |
| SAP | Advanced Thermostats | 3,334 | 78% | 2,615 | 1.00 | N/A | 2,615 |
|  | Air Sealing | 608 | 91% | 554 | 1.00 | N/A | 554 |
|  | Boiler Pipe Insulation | 8 | 100% | 8 | 1.00 | N/A | 8 |
|  | DHW Pipe Insulation | 302 | 80% | 241 | 1.00 | N/A | 241 |
|  | Low Flow Aerator- Bathroom (IU) | 123 | 100% | 123 | 1.00 | N/A | 123 |
|  | Low Flow Aerator- Kitchen (IU) | 431 | 100% | 431 | 1.00 | N/A | 431 |
|  | Low Flow Showerheads (IU) | 537 | 100% | 537 | 1.00 | N/A | 537 |
|  | Programmable Thermostats | 445 | 100% | 445 | 1.00 | N/A | 445 |
|  | Water Heater Temperature Setback | 6.08 | 100% | 6.08 | 1.00 | N/A | 6.08 |
|  | SAP Subtotal | 5,794 | 86% | 4,960 | 1.00 |  | 4,960 |
| Healthy Homes | Advanced Thermostats | 67 | 100% | 67 | 1.00 | N/A | 67 |
|  | Air Filter Replacement | 8 | 100% | 8 | 1.00 | N/A | 8 |
|  | Air Purifier/Cleaner | - | - | - | 1.00 | N/A | - |
|  | Air Sealing | 96 | 100% | 96 | 1.00 | N/A | 96 |
|  | Duct Sealing | 310 | 100% | 310 | 1.00 | N/A | 310 |
|  | Furnace Tune-Up | 89 | 100% | 89 | 1.00 | N/A | 89 |
|  | Healthy Homes Subtotal | 569 | 100% | 569 | 1.00 |  | 569 |
| Total |  | 847,869 | 97% | 822,642 | 1.00 |  | 822,642 |

* Realization Rate (RR) is the ratio of verified gross savings to ex ante gross savings, based on evaluation research findings.

† NTG, Net to Gross is the deemed value available on the SAG website: https://www.ilsag.info/evaluator-ntg-recommendations-for-2025/.

‡The Non-Participant Spillover (NPSO) does not apply to this program.
Source: Evaluation team analysis.

## Impact Analysis Findings and Recommendations

### Impact Parameter Estimates

Table 5 shows the unit Therm savings and realization rate findings by measure from our review. The realization rate is the ratio of the verified savings to the ex ante savings. Following the table, we provide findings and recommendations, including a discussion of all measures with realization rates above or below 100%. Appendix A provides a description of the impact analysis methodology.

Table 5. Verified Gross Savings Parameters – Income Eligible

| Measure | Unit Basis | Ex Ante Gross (therms/unit) | Verified Gross (therms/unit) | Realization Rate | IL-TRM v13.0 Section† |
| --- | --- | --- | --- | --- | --- |
| Advanced Thermostats | Unit | Varies | Varies | 101% | 5.3.16 |
| Air Filter Replacement | Unit | Varies | Varies | 100% | 5.3.21 |
| Air Sealing | LN FT | Varies | Varies | 100% | 5.6.1 |
| Attic Insulation | SQ FT | Varies | Varies | 99% | 5.6.5 |
| Basement/Sidewall Insulation | Unit | Varies | Varies | 128% | 5.6.2 |
| Boiler Pipe Insulation | LN FT | Varies | Varies | 383% | 5.3.2 |
| Boiler Reset Controls | Unit | 798.34 | 798.34 | 100% | 5.4.4 |
| Boiler Tune Up | Unit | Varies | Varies | 100% | 4.4.2 |
| Boilers | Unit | 169.71 | 162.00 | 95% | 4.4.10 |
| Commercial Gas Heat Pump | Unit | 1,758.25 | 784.77 | 45% | 4.4.55 |
| Controls for Domestic Hot Water | Unit | 62.70 | 62.70 | 100% | 4.3.8 |
| Cover and Gap Sealers for AC | Unit | Varies | Varies | 100% | 4.4.38 |
| DHW Pipe Insulation | LN FT | Varies | Varies | 47% | 5.3.16 |
| Duct Insulation | Unit | 0.59 | 0.59 | 100% | 5.3.4 |
| Duct Sealing | Unit | Varies | Varies | 74% | 5.3.4 |
| Furnace Tune-Up | Unit | Varies | Varies | 100% | 5.3.13 |
| High Efficiency Furnace – IU | Unit | 320.04 | 129.91 | 41% | 4.4.11 |
| Low Flow Aerator – Bathroom (IU) | Unit | Varies | Varies | 100% | 5.3.7 |
| Low Flow Aerator – Kitchen (IU) | Unit | Varies | Varies | 100% | 5.4.4 |
| Low Flow Showerheads (IU) | Unit | Varies | Varies | 100% | 5.4.4 |
| Pipe Insulation | LN FT | Varies | Varies | 100% | 5.4.5 |
| Programmable Thermostats | Unit | Varies | Varies | 104% | 4.4.14 |
| Reprogrammable Thermostat | Unit | Varies | Varies | 100% | 5.3.11 |
| Shower Timer | Unit | 4.16 | 4.16 | 100% | 5.3.11 |
| Steam Boiler Averaging Controls | Unit | 61.09 | 61.09 | 100% | 5.4.9 |
| Steam Trap | Unit | Varies | Varies | 100% | 4.4.36 |
| Storage Tank Insulation | SQ FT | Varies | 6.84 | 114% | 4.4.16 |
| Wall Insulation | SQ FT | 0.08 | 0.03 | 33% | 4.3.12 |
| Water Heater Temperature Setback | Unit | Varies | Varies | 100% | 5.6. |

† State of Illinois Technical Reference Manual version 13.0 from http://www.ilsag.info/technical-reference-manual.html.

Source: Nicor Gas tracking data and evaluation team analysis

### Findings and Recommendations

The evaluation team developed findings and recommendations based on the 2025 evaluation. The findings and recommendations are organized by path type in the following sections.

#### Retrofits

Finding 1. In eleven instances of the Air Filter Replacement measure, ex ante calculations used Gas Heating Consumption as 1,052 Therms, corresponding to the Rockford climate zone. However, tracking data reported 1,005 Therms, corresponding to the Chicago climate zone. The evaluation team used installation address to verify the climate zone and applied Gas Heating Consumption as 1,052 Therms for Rockford climate zone, consistent with IL-TRM v13.0. This measure accounts for 0.1% of Retrofit component’s reported gross Therms savings and the realization rate is 100% for this measure.

Finding 2. For one instance of Low Flow Aerator - Kitchen (DI) MF-IU (MEA-2025.04.28-904848), ex ante calculations applied a Household Factor of 2.3 to calculate savings. However, the tracking data reported a Household Factor of 2.1. The evaluation team used a Household factor of 2.3, consistent with IL-TRM v13.0 to calculate verified savings.

Finding 3. For 456 instances of Handheld Showerhead (DI) MF-IU and 775 instances of Showerhead (DI) MF-IU, ex ante calculations applied a Household Factor of 2.3 to calculate savings. However, the tracking data reported a Household Factor of 2.1. The evaluation team used Household Factor of 2.3, consistent with IL-TRM v13.0, to calculate verified savings.

Finding 4. For one instance of Low Flow Showerheads (IU) (MEA-2025.10.08-970749), ex ante calculations applied a Household Factor of 2.3 and in-service-rate (ISR) of 96.5%, consistent with the IL-TRM v13.0 to calculate savings. However, the tracking data reported a Household factor of 2.1 and a 98% ISR. The evaluation team applied a Household factor of 2.3 and a 96.5% ISR, consistent with IL-TRM v13.0, to calculate verified savings.

Recommendation 1.  For measures in findings 1-4, ensure inputs reported in the tracking data align with the inputs used in ex ante savings calculations and the IL-TRM v13.0.

Finding 5. In one instance of Air Sealing measure (MEA-2025.09.18-961878), ex ante calculations used an In-Service Rate (ISR) of 60%, applicable to Rope Caulk measures in Opt-in Weatherization Kits. The evaluation team applied a 100% ISR, applicable to Direct Install measures, consistent with the program delivery method. This measure accounts for 27% of Retrofit component’s reported gross Therms savings and the impact of this adjustment resulted in a realization rate of 101% for this measure.

Recommendation 2.  Ensure the ISR applied in savings calculations for all air sealing measures reflects the program delivery method.

Finding 6. For one Non-Condensing Boilers, >85% measure instance (MEA-2025.03.25-879009), ex ante calculations applied an installed boiler efficiency of 88.2% to calculate savings. The evaluation team applied an installed boiler efficiency of 88%, consistent with the value reported in the tracking data. This measure accounts for 0.02% of Retrofit component’s reported gross Therms savings and the impact of this adjustment resulted in a realization rate of 95% for this measure.

Finding 7. For twenty-five instances of Duct Sealing measure, ex ante calculations applied a Heating Input Capacity of 800 kBtu/hr to calculate savings. The evaluation team applied a Heating Input Capacity of 478 kBtu/hr, consistent with the value reported in the tracking data. This measure accounts for 5% of Retrofit component’s reported gross Therms savings and the impact of this adjustment resulted in a realization rate of 74% for this measure.

Recommendation 3.  For measures in findings 6 and 7, ensure inputs used in ex ante savings calculations are consistent with the corresponding values reported in the tracking data.

Finding 8. For several measure instances, shown in Table 6, the evaluation team was unable to replicate the ex ante savings. The evaluation applied inputs reported in the tracking data and algorithms from IL-TRM v13.0 to calculate verified savings.

Table 6. Measure Instances with Unidentified Discrepancies

| Measure | Instances with Discrepancy | Total Instances | Ex Ante Gross (therms) | Verified Gross (therms) | Realization Rate | Percentage of Impacted Instances |
| --- | --- | --- | --- | --- | --- | --- |
| Air Sealing | 14 | 189 | 221,131 | 222,271 | 101% | 7% |
| Basement/Sidewall Insulation | 2 | 2 | 664 | 853 | 128% | 100% |
| Commercial Gas Heat Pump | 4 | 4 | 7,033 | 3,139 | 45% | 100% |
| Pipe Insulation | 3 | 226 | 188,136 | 188,079 | 100% | 1% |
| Storage Tank Insulation | 4 | 4 | 1,148 | 1,306 | 114% | 100% |

Source: Nicor Gas tracking data and evaluation team analysis.

Recommendation 4.  For measures in Table 6, ensure savings are calculated using the applicable algorithms from IL-TRM v13.0 and the corresponding inputs reported in the tracking data.

#### Illinois Home Weatherization Assistance Program (IHWAP)

Finding 13. For all four instances of Low Flow Aerator - Bath (DI) MF-IU measure, ex ante calculations used baseline flow rate of 1.53 gpm, applicable for Bathroom Aerators to calculate savings. However, tracking data reported 1.63 gpm, applicable for Kitchen Aerators. The evaluation team used baseline flow rate of 1.53 gpm, consistent with IL-TRM v13.0 to calculate verified savings. This measure accounts for 0.2% of IHWAP component’s reported gross Therms savings and the realization rate is 100% for this measure.

Finding 14. For all three instances of Handheld Showerhead (DI) MF-IU measure, ex ante calculations used Household Factor of 2.3 and ISR of 96.5% to calculate savings. However, tracking data reported 2.1 and 97% for Household Factor and ISR, respectively. The evaluation team used Household Factor of 2.3 and ISR of 96.5%, consistent with IL-TRM v13.0 to calculate verified savings. This measure accounts for 0.3% of IHWAP component’s reported gross Therms savings and the realization rate is 100% for this measure.

Recommendation 9.  Ensure inputs reported in the tracking data align with inputs used in ex ante calculations and the IL-TRM v13.0.

Finding 15. The evaluation team was unable to replicate the ex ante savings for all four instances of Wall Insulation. The evaluation used algorithms from Section 5.6.4 of the IL-TRM v13.0 and inputs reported in the tracking data to calculate verified savings. This measure accounts for 8% of IHWAP component’s reported gross Therms savings and the impact of this adjustment resulted in a realization rate of 33% for this measure.

Finding 16. The evaluation team was unable to replicate the ex ante savings for all four instances of Attic Insulation. The evaluation used algorithms from Section 5.6.5 of IL-TRM v13.0 and inputs reported in the tracking data to calculate verified savings. This measure accounts for 7% of IHWAP component’s reported gross Therms savings and the impact of this adjustment resulted in a realization rate of 52% for this measure.

Finding 17.  The evaluation team was unable to replicate the ex ante savings for all four instances of Furnace, >95% AFUE - MF IU measure, The evaluation used algorithms from Section 5.3.7 of IL-TRM v13.0 and inputs reported in the tracking data to calculate verified savings. This measure accounts for 65% of IHWAP component’s reported gross Therms savings and the impact of this adjustment resulted in a realization rate of 41% for this measure.

Recommendation 10.  Ensure that all Furnace savings are calculated using algorithms from the IL-TRM v13.0 and inputs reported in the tracking data.

#### Self-Assessment Program

Finding 18. For three instances of Advanced Thermostat (DI) – Manual measure (MEA-2025.01.30-843949, MEA-2025.01.30-843955 and MEA-2025.02.26-860012), ex ante calculations applied an 8.5% Heating Reduction factor, applicable when the existing thermostat type is unknown. However, the tracking data reported a 10.2% Heating Reduction factor.

For two instances of Advanced Thermostat (DI) – Manual measure (MEA-2025.05.21-915221 and MEA-2025.07.01-927942), ex ante calculations applied a 7.1% Heating Reduction factor, applicable when the existing thermostat is programmable.

For all these measure instances, the evaluation team applied a Heating Reduction factor of 10.2%, applicable when the existing thermostat type is Manual, consistent with the measure name and IL-TRM v13.0. Advanced Thermostats account for 57% of SAP component’s reported gross Therms savings and the impact of the adjustments from Findings 18 and 19 resulted in a realization rate of 78% for this measure.

Recommendation 11.  Ensure correct Heating Reduction factor applied reflects the baseline measure characterization associated with the Advanced Thermostats measure, as indicated by the measure name.

Finding 19. For 18 out of 40 total instances of Advanced Thermostat (DI) – Programmable measure, ex ante savings calculations used ISR for heating as 100%, associated with Direct Install measures. The evaluation team applied a 54% ISR, associated with self-install thermostats, consistent with the program delivery method and IL-TRM v13.0. Advanced Thermostats account for 57% of SAP component’s reported gross Therms savings and the impact of the adjustments from Findings 18 and 19 resulted in a realization rate of 78% for this measure.

Finding 20. For 67 out of 69 total instances of Air Sealing – Door Sweep – DI measure, ex ante calculations used an ISR of 68%, applicable to Door Sweeps distributed in Opt-in Weatherization Kits. The evaluation team applied a 62% ISR, applicable to Distributed Self-Install Income-Qualified measures, consistent with the program delivery method and IL-TRM v13.0. This measure accounts for 10% of SAP component’s reported gross Therms savings and the impact of this adjustment resulted in a realization rate of 91%.

Recommendation 12.  Use ISR consistent with the program delivery method for Air Sealing – Door Sweep – DI measure.

Finding 21. For all six instances of Boiler Pipe Insulation, the tracking data did not report the following input parameters.

Pipe heat loss coefficient of uninsulated pipe

Inside circumference of the pipe

Given the relatively small magnitude of savings for this measure, the evaluation team reviewed the available inputs reported in the tracking data and confirmed alignment with applicable IL-TRM guidance. The missing inputs are required to fully replicate and evaluate this measure and should be reported in future program years.

Recommendation 13. Ensure all inputs relevant to the savings calculation for Boiler Pipe Insulation are reported in the tracking data.

Finding 22. For all instances of DHW Pipe Insulation, the evaluation team was unable to replicate the ex ante savings. The evaluation used algorithms from IL-TRM v13.0 and inputs reported in the tracking data to calculate verified savings. This measure accounts for 5% of SAP component’s reported gross Therms savings and the impact of this adjustment resulted in a realization rate of 80% for this measure.

Recommendation 14. Ensure savings for DHW Pipe Insulation are calculated using inputs reported in the tracking data and consistent with the measure description.

# Public Housing Energy Savings Program

## Program Description

The Public Housing Energy Savings Program (PHES) Program works with public housing authorities (PHAs) in ComEd, Nicor Gas, Peoples Gas (PGL), and North Shore Gas (NSG) territories to achieve electric and gas savings. The PHAs themselves are the program participants, though the residents of the properties are directly affected by the program through in-unit and common area upgrades. Gas savings opportunities include both heating and water heating equipment upgrades and envelope measures (attic insulation, air sealing, duct sealing, duct insulation, cover and gap sealers for AC). The program also included gas saving measures, such as high efficiency furnace, furnace tune-up and boiler tune up.

The program had 14 participants in 2025 and completed 23 projects as shown in the following table.

Table 7. 2025 Volumetric Findings Detail – Public Housing

| Participation | Public Housing |
| --- | --- |
| Participants * | 14 |
| Installed Projects † | 23 |
| Measure Types Installed ‡ | 9 |

* Participants are defined as distinct counts of building account numbers.

† Installed Projects are defined as distinct count of project ID.

‡ Measure Types Installed are the distinct count of Nicor measure names

Source: Nicor Gas tracking data and evaluation team analysis.

Table 8 summarizes the installed measure quantities that are the basis for verified energy savings.

Table 8. 2025 Installed Measure Quantities – Public Housing

| Program Path | Measure | Quantity Unit | Installed Quantity |
| --- | --- | --- | --- |
| Public Housing | Air Sealing | LN FT | 55,308 |
|  | Assessment/No Savings | Unit | 3,242 |
|  | Attic Insulation | SQ FT | 92,181 |
|  | Boiler Tune Up | Unit | 4 |
|  | Cover and Gap Sealers for AC | Unit | 24 |
|  | Duct Insulation | Unit | 1,385 |
|  | Duct Sealing | Unit | 540 |
|  | Furnace Tune-Up | Unit | 471 |
|  | High Efficiency Furnace - CA | Unit | 400 |

Source: Nicor Gas tracking data and evaluation team analysis.

## Program Savings Detail

Table 9 summarizes the energy savings the Public Housing Program achieved by path in 2025.

Table 9. 2025 Annual Energy Savings Summary – Public Housing

| Program Path | Ex Ante Gross Savings (Therms) | Verified Gross RR* | Verified Gross Savings (Therms) | NTG† | NSPO‡ | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- |
| Public Housing | 41,027 | 100% | 41,027 | 1.00 | N/A | 41,027 |

* Realization Rate (RR) is the ratio of verified gross savings to ex ante gross savings, based on evaluation research findings.

† Net to Gross is the deemed value available on the SAG website: https://www.ilsag.info/evaluator-ntg-recommendations-for-2025/.

‡The Non-Participant Spillover (NPSO) does not apply to this program.
Source: Evaluation team analysis.

## Program Savings by Measure

The program includes nine measures as shown in the following table. The Air Sealing and Furnace-Tune Up measures contributed the most savings.

Table 10. 2025 Annual Energy Savings by Measure – Public Housing

| Program Path | Savings Category |  | Ex Ante Gross Savings (Therms) | Verified Gross RR* | Verified Gross Savings (Therms) | NTG† | NSPO‡ | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Public Housing | Air Sealing |  | 23,008 | 100% | 23,008 | 1.00 | N/A | 23,008 |
|  | Attic Insulation |  | 6,083 | 100% | 6,083 | 1.00 | N/A | 6,083 |
|  | Boiler Tune Up |  | 1,304 | 100% | 1,304 | 1.00 | N/A | 1,304 |
|  | Cover and Gap Sealers for AC |  | 164 | 100% | 164 | 1.00 | N/A | 164 |
|  | Duct Insulation |  | 817 | 100% | 817 | 1.00 | N/A | 817 |
|  | Duct Sealing |  | 544 | 100% | 544 | 1.00 | N/A | 544 |
|  | Furnace Tune-Up |  | 7,787 | 100% | 7,787 | 1.00 | N/A | 7,787 |
|  | High Efficiency Furnace – CA |  | 1,320 | 100% | 1,320 | 1.00 | N/A | 1,320 |
| Total |  |  | 41,027 | 100% | 41,027 | 1.00 |  | 41,027 |

* Realization Rate (RR) is the ratio of verified gross savings to ex ante gross savings, based on evaluation research findings.

† Net to Gross is the deemed value available on the SAG website: https://www.ilsag.info/evaluator-ntg-recommendations-for-2025/.
‡The Non-Participant Spillover (NPSO) does not apply to this program.

Source: Evaluation team analysis.

## Impact Analysis Findings and Recommendations

### Impact Parameter Estimates

Table 11 shows the unit Therm savings and realization rate findings by measure from our review. The realization rate is the ratio of the verified savings to the ex-ante savings. Following the table, we provide findings and recommendations, including discussion of all measures with realization rates above or below 100%. Appendix A provides a description of the impact analysis methodology.

Table 11. Verified Gross Savings Parameters – Public Housing

| Measure | Unit Basis | Ex Ante Gross (therms/unit) | Verified Gross (therms/unit) | Realization Rate | IL-TRM v13.0 Section† |
| --- | --- | --- | --- | --- | --- |
| Air Sealing | LN FT | Varies | Varies | 100% | 5.6.1 |
| Attic Insulation | SQ FT | Varies | Varies | 100% | 5.6.5 |
| Boiler Tune Up | Unit | Varies | Varies | 100% | 4.4.2 |
| Cover and Gap Sealers for AC | Unit | Varies | Varies | 100% | 4.4.38 |
| Duct Insulation | Unit | 0.59 | 0.59 | 100% | 5.3.4 |
| Duct Sealing | Unit | Varies | Varies | 100% | 5.3.4 |
| Furnace Tune-Up | Unit | Varies | Varies | 100% | 5.3.13 |
| High Efficiency Furnace - CA | Unit | 3.3 | 3.3 | 100% | 4.4.11 |

† State of Illinois Technical Reference Manual version 13.0 from http://www.ilsag.info/technical-reference-manual.html.

Source: Nicor Gas program tracking data and evaluation team analysis

### Findings and Recommendations

Finding 23. For all 471 instances of Furnace Tune-ups, ex ante calculations used EFLH of 1,022 and 976 for Rockford and Chicago climate zones, respectively. However, tracking data reported 1,782 EFLH for both the climate zones. The evaluation team used EFLH of 1,022 and 976 hours for Rockford and Chicago climate zones, respectively, consistent with the ex ante calculations and IL-TRM v13.0.

Recommendation 15.  Ensure that deemed inputs recorded in the tracking data for Furnace Tune-ups are consistent with the values used in ex ante savings calculations and the IL-TRM v13.0.

# Appendix A. Impact Analysis Methodology

The evaluation team used the same impact methodology for each component. Verified gross savings were determined for each program measure by:

- Reviewing the savings algorithm inputs in the measure workbook for agreement with the IL-TRM v13.0 and IL-TRM Errata, where applicable.

- Validating the savings algorithm was applied correctly.

- Cross-checking per-unit savings values in the program tracking data with the verified values in the measure workbook or in Guidehouse’s calculations if the workbook did not agree with the IL-TRM v13.0.

- Multiplying the verified per-unit savings value by the quantity reported in the tracking data. The team calculated verified net savings by multiplying the verified gross savings estimates by an NTG ratio. In Program Year 2025, NTG estimates used to calculate the net verified savings were based on past evaluation research and defined by a consensus process through the Illinois SAG.

- Guidehouse sourced methodologies and assumptions from the Illinois IL-TRM v13.0 and the final 2025 tracking data.

# Appendix B. Program Specific Inputs for the Illinois TRC

Table B-1 and Table B-2 shows the Total Resource Cost (TRC) cost-effectiveness analysis inputs available at the time of producing this impact evaluation report for Income Eligible and Public Housing programs, respectively. Additional required cost data (e.g., measure costs, program level incentive and non-incentive costs) are not included in this table and will be provided to the evaluation team later. Guidehouse will include annual and lifetime water savings and greenhouse gas reductions in the end of year summary report.

Table B-1. Verified Cost Effectiveness Inputs – Income Eligible

| Program Path | Savings Category | DAC Project* | Units | Quantity | Effective Useful Life |  | Early Replacement Flag |  | Verified Gross Annual Water Savings (Gallons) | Ex Ante Gross Savings (Therms) | Verified Gross Savings (Therms) | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Retrofits | Air Filter Replacement | FALSE | Unit | 82 |  | 3.0 | NO |  | - | 829 | 829 | 829 |
|  | Air Sealing | FALSE | LN FT | 532,548 |  | 20.0 | NO |  | - | 221,131 | 222,271 | 222,271 |
|  | Assessment/No Savings | FALSE | Unit | 244,215 |  | - | NO |  | - | - | - | - |
|  | Attic Insulation | FALSE | SQ FT | 719,734 |  | 30.0 | NO |  | - | 163,082 | 163,083 | 163,083 |
|  | Basement/Sidewall Insulation | FALSE | Unit | 805 |  | 30.0 | NO |  | - | 664 | 853 | 853 |
|  | Boiler Reset Controls | FALSE | Unit | 1 |  | 16.0 | NO |  | - | 798 | 798 | 798 |
|  | Boiler Tune Up | FALSE | Unit | 105 |  | 3.0 | NO |  | - | 22,630 | 22,630 | 22,630 |
|  | Boilers | FALSE | Unit | 1 |  | 16.5 | NO |  | - | 170 | 162 | 162 |
|  | Commercial Gas Heat Pump | FALSE | Unit | 4 |  | 20.0 | NO |  | - | 7,033 | 3,139 | 3,139 |
|  | Controls for Domestic Hot Water | FALSE | Unit | 628 |  | 15.0 | NO |  | - | 39,376 | 39,376 | 39,376 |
|  | Cover and Gap Sealers for AC | FALSE | Unit | 234 |  | 5.0 | NO |  | - | 1,689 | 1,689 | 1,689 |
|  | Duct Insulation | FALSE | Unit | 682 |  | 19.7 | NO |  | - | 402 | 402 | 402 |
|  | Duct Sealing | FALSE | Unit | 29,927 |  | 19.7 | NO |  | - | 40,872 | 30,128 | 30,128 |
|  | Furnace Tune-Up | FALSE | Unit | 1 |  | 3.0 | NO |  | - | 16 | 16 | 16 |
|  | Low Flow Aerator- Bathroom (IU) | FALSE | Unit | 759 |  | 10.0 | NO |  | 344,895 | 1,517 | 1,518 | 1,518 |
|  | Low Flow Aerator- Kitchen (IU) | FALSE | Unit | 1,187 |  | 10.0 | NO |  | 2,217,660 | 11,754 | 11,754 | 11,754 |
|  | Low Flow Showerheads (IU) | FALSE | Unit | 1,437 |  | 10.0 | NO |  | 3,104,559 | 19,565 | 19,559 | 19,559 |
|  | Pipe Insulation | FALSE | LN FT | 22,147 |  | 12.0 | NO |  | - | 188,136 | 188,079 | 188,079 |
|  | Programmable Thermostats | FALSE | Unit | 402 |  | 8.0 | NO |  | - | 16,297 | 16,297 | 16,297 |
|  | Reprogrammable Thermostat | FALSE | Unit | 29 |  | 2.0 | NO |  | - | 1,176 | 1,176 | 1,176 |
|  | Shower Timer | FALSE | Unit | 2,601 |  | 2.0 | NO |  | 1,729,187 | 10,807 | 10,807 | 10,807 |
|  | Steam Boiler Averaging Controls | FALSE | Unit | 72 |  | 20.0 | NO |  | - | 4,398 | 4,398 | 4,398 |
|  | Steam Trap | FALSE | Unit | 318 |  | 6.0 | NO |  | 454,875 | 64,387 | 64,387 | 64,387 |
|  | Storage Tank Insulation | FALSE | SQ FT | 191 |  | 15.0 | NO |  | - | 1,148 | 1,306 | 1,306 |
| IHWAP | Air Sealing | FALSE | LN FT | 4,262 |  | 20.0 | NO |  | - | 2,373 | 2,373 | 2,373 |
|  | Assessment/No Savings | FALSE | Unit | 151 |  | - | NO |  | - | - | - | - |
|  | Attic Insulation | FALSE | SQ FT | 19,084 |  | 30.0 | NO |  | - | 1,777 | 933 | 933 |
|  | High Efficiency Furnace - IU | FALSE | Unit | 48 |  | 20.0 | NO |  | - | 15,362 | 6,235 | 6,235 |
|  | Low Flow Aerator- Bathroom (IU) | FALSE | Unit | 21 |  | 10.0 | NO |  | 9,543 | 42 | 42 | 42 |
|  | Low Flow Aerator- Kitchen (IU) | FALSE | Unit | 26 |  | 10.0 | NO |  | 48,576 | 257 | 257 | 257 |
|  | Low Flow Showerheads (IU) | FALSE | Unit | 17 |  | 10.0 | NO |  | 11,907 | 75 | 75 | 75 |
|  | Programmable Thermostats | FALSE | Unit | 48 |  | 8.0 | NO |  | - | 1,944 | 1,944 | 1,944 |
|  | Wall Insulation | FALSE | SQ FT | 22,848 |  | 30.0 | NO |  | - | 1,800 | 598 | 598 |
| SAP | Advanced Thermostats | FALSE | Unit | 67 |  | 2.0 | NO |  | - | 3,334 | 2,615 | 2,615 |
|  | Air Sealing | FALSE | LN FT | 122 |  | 20.0 | NO |  | - | 608 | 554 | 554 |
|  | Assessment/No Savings | FALSE | Unit | 109 |  | - | NO |  | - | - | - | - |
|  | Boiler Pipe Insulation | FALSE | LN FT | 18 |  | 15.0 | NO |  | - | 8 | 8 | 8 |
|  | DHW Pipe Insulation | FALSE | LN FT | 150 |  | 12.0 | NO |  | - | 302 | 241 | 241 |
|  | Low Flow Aerator- Bathroom (IU) | FALSE | Unit | 129 |  | 10.0 | NO |  | 32,334 | 123 | 123 | 123 |
|  | Low Flow Aerator- Kitchen (IU) | FALSE | Unit | 53 |  | 10.0 | NO |  | 95,805 | 431 | 431 | 431 |
|  | Low Flow Showerheads (IU) | FALSE | Unit | 130 |  | 10.0 | NO |  | 99,370 | 537 | 537 | 537 |
|  | Programmable Thermostats | FALSE | Unit | 9 |  | 8.0 | NO |  | - | 445 | 445 | 445 |
|  | Water Heater Temperature Setback | FALSE | Unit | 24 |  | 2.0 | NO |  | - | 6 | 6 | 6 |
| Healthy Homes | Advanced Thermostats | FALSE | Unit | 1 |  | 2.0 | NO |  | - | 67 | 67 | 67 |
|  | Air Filter Replacement | FALSE | Unit | 1 |  | 3.0 | NO |  | - | 8 | 8 | 8 |
|  | Air Purifier/Cleaner | FALSE | Unit | 2 |  | - | NO |  | - | - | - | - |
|  | Air Sealing | FALSE | LN FT | 230 |  | 20.0 | NO |  | - | 96 | 96 | 96 |
|  | Assessment/No Savings | FALSE | Unit | 15 |  | - | NO |  | - | - | - | - |
|  | Duct Sealing | FALSE | Unit | 1 |  | 19.7 | NO |  | - | 310 | 310 | 310 |
|  | Furnace Tune-Up | FALSE | Unit | 1 |  | 3.0 | NO |  | - | 89 | 89 | 89 |
| Total or Weighted Average |  |  |  |  |  | 17.3 |  |  | 8,148,710 | 847,869 | 822,642 | 822,642 |

Source: Evaluation team analysis.

Table B-2. Verified Cost Effectiveness Inputs – Public Housing

| Program Path | Savings Category | DAC Project* | Units | Quantity | Effective Useful Life | Early Replacement Flag | Verified Gross Annual Water Savings (Gallons) | Ex Ante Gross Savings (Therms) | Verified Gross Savings (Therms) | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Public Housing | Air Sealing | FALSE | LN FT | 55308 | 20.00 | NO | - | 23,008 | 23,008 | 23,008 |
|  | Assessment/No Savings | FALSE | Unit | 3242 | - | NO | - | - | - | - |
|  | Attic Insulation | FALSE | SQ FT | 92181 | 30.0 | NO | - | 6,083 | 6,083 | 6,083 |
|  | Boiler Tune Up | FALSE | Unit | 4 | 3.0 | NO | - | 1,304 | 1,304 | 1,304 |
|  | Cover and Gap Sealers for AC | FALSE | Unit | 24 | 5.0 | NO | - | 164 | 164 | 164 |
|  | Duct Insulation | FALSE | Unit | 1385 | 19.7 | NO | - | 817 | 817 | 817 |
|  | Duct Sealing | FALSE | Unit | 540 | 19.7 | NO | - | 544 | 544 | 544 |
|  | Furnace Tune-Up | FALSE | Unit | 471 | 3.0 | NO | - | 7,787 | 7,787 | 7,787 |
|  | High Efficiency Furnace - CA | FALSE | Unit | 400 | 20.0 | NO | - | 1,320 | 1,320 | 1,320 |
| Total or Weighted Average |  |  |  |  | 17.6 |  |  | 41,027 | 41,027 | 41,027 |

Source: Evaluation team analysis.