# Single Family Income Eligible Program Impact Evaluation Report

Energy Efficiency Plan: Program Year 2025

(01/01/2025-12/31/2025)

Prepared for:

Nicor Gas Company

DRAFT

March 23, 2026

Prepared by:

| Aidan Lewis Guidehouse | Yeab Lakew Guidehouse |  |  |
| --- | --- | --- | --- |

Submitted to:

Nicor Gas Company

1844 Ferry Road

Naperville, IL 60563

Submitted by:

Guidehouse Inc.

167 N. Green Street

12th Floor

Chicago, IL 60607

Contact:

| Charles Maglione, Partner 202.481.7352 cmaglione@guidehouse.com | Jeff Erickson, Director 608.616.4962 jeff.erickson@guidehouse.com | Laura Agapay-Read, Associate Director 312.583.4178 laura.agapay.read@guidehouse.com |
| --- | --- | --- |

Charles Ampong, Associate Director

608.616.4922

charles.ampong@guidehouse.com

This report was prepared by Guidehouse for Nicor Gas Company. The work presented in this report represents Guidehouse’s professional judgment based on the information available at the time this report was prepared. Use of this report by any other party for whatever purpose should not, and does not, absolve such party from using due diligence in verifying the report’s contents. Neither Guidehouse nor any of its subsidiaries or affiliates assumes any liability or duty of care to such parties and hereby disclaims any such liability.

Table of Contents

1	Introduction	3

2	Program Description	4

3	Program Savings Detail	7

4	Program Savings by Measure	8

5	Impact Analysis Findings and Recommendations	10

5.1	Impact Parameter Estimates	11

5.2	Findings and Recommendations	12

Appendix A. Impact Analysis Methodology	14

Appendix B. Program Specific Inputs for the Illinois TRC	15

Table of Tables, Figures, and Equations

Table 1. 2025 Volumetric Findings Detail	2

Table 2. 2025 Installed Measure Quantities	2

Table 3. 2025 Annual Energy Savings Summary	5

Table 4. 2025 Annual Energy Savings by Measure	5

Table 5. Verified Gross Savings Parameters	9

# Introduction

This report presents the results of the impact evaluation of the Nicor Gas 2025 Single Family Income Eligible (SFIE) Program. It presents a summary of the energy impacts for the total program and broken out by relevant measure and program structure details. The appendices present the impact analysis methodology and inputs for benefit-cost analysis. Program year 2025 covers January 1 to December 31, 2025.

# Program Description

The SFIE program offers installation of energy efficiency measures for qualifying homeowners with a household income at or below 80% of the area median income (AMI). Eligible measures include programmable and advanced thermostats, water efficiency devices, weatherization measures, pipe insulation, heating and cooling equipment, and custom energy-saving measures for eligible properties. The program

The SFIE program has three main implementation paths that are offered jointly by Nicor Gas, ComEd, Peoples Gas (PGL), and North Shore Gas (NSG): (1) Illinois Home Weatherization Assistance Program (IHWAP), (2) Home Energy Assessment (HEA), and (3) Retrofits (contractor channel).

- The IHWAP provides weatherization services to low-income residents through local community action agencies or not for profit agencies.

- The Retrofit path offers assessments, direct installation of energy efficiency measures, replacement of inefficient equipment, weatherization services for customers, technical assistance, and educational information through approved agencies or partners (e.g., Chicago Bungalow Association) that perform outreach and intake.

- The HEA offers energy-saving products installed in homes at no cost to the customer. Homeowners can reduce their energy and water use with the installation of products available to owners of single-family homes, two-flats, and individually metered condos and townhomes.

Four additional implementation paths are administered by Nicor Gas with 100% therm allocation to Nicor Gas. These paths include installation of water heating efficiency measures (faucet aerators, showerheads, gas water heaters); advanced and programmable thermostats; air sealing, attic, duct, and rim insulation; and high efficiency boilers and furnaces.

- The SFIE HEA offer includes the Self-Assessment Program (SAP) where a customer completes an online survey and has a kit of products shipped to their home for direct installation.

- Joint Ameren Illinois and Nicor Gas Energy Efficiency Offering (EEO), consisting of Ameren-Nicor Gas service area measures.

- Healthy Homes (HH), which was a new program Nicor Gas had launched since 2023.  It offers installation of health and safety measures, including vents, asbestos and mold remediation.

- Community Pilot program path which was introduced in 2025.

The program had 3,174 participants in 2025 and completed 3,284 projects as shown in the following table.

Table 1. 2025 Volumetric Findings Detail

| Participation | Ameren Nicor SF | HH SF | IHWAP SF | Retrofits SF | HEA | SAP | Community Pilot | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Participants * | 47 | 23 | 165 | 522 | 2280 | 109 | 28 | 3174 |
| Installed Projects † | 48 | 23 | 169 | 561 | 2321 | 109 | 53 | 3284 |
| Measure Types Installed ‡ | 15 | 16 | 17 | 15 | 9 | 8 | 12 | 20 |

* Participants are defined as the distinct count of addresses

† Installed Projects are defined as the distinct count of project ID

‡ Measure Types installed are the distinct count of Nicor Gas Measure Names

Source: Nicor Gas tracking data and evaluation team analysis.

Table 2 summarizes the installed measure quantities that are the basis for verified energy savings.

Table 2. 2025 Installed Measure Quantities

| Program Path | Savings Category | Ex Ante Gross Savings (Therms) | Verified Gross RR* | Verified Gross Savings (Therms) | NTG† | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- |
| Ameren Nicor | Advanced Thermostats | 2,083 | 100% | 2,083 | 1.0 | 2,083 |
|  | Air Handler Filter Cleaning/Replacement | 252 | 100% | 252 | 1.0 | 252 |
|  | Air Sealing - Method 1 | 6,903 | 100% | 6,903 | 1.0 | 6,903 |
|  | Air Sealing - Method 2 | 120 | 100% | 120 | 1.0 | 120 |
|  | Ceiling/Attic Insulation | 3,673 | 100% | 3,673 | 1.0 | 3,673 |
|  | Domestic Hot Water Pipe Insulation | 563 | 100% | 563 | 1.0 | 563 |
|  | Duct Insulation and Sealing | 16,163 | 100% | 16,163 | 1.0 | 16,163 |
|  | Floor Insulation Above Crawlspace | 238 | 100% | 238 | 1.0 | 238 |
|  | Gas High Efficiency Furnace | 664 | 100% | 664 | 1.0 | 664 |
|  | Gas Water Heater | 471 | 100% | 471 | 1.0 | 471 |
|  | Low Flow Faucet Aerators | 188 | 100% | 188 | 1.0 | 188 |
|  | Low Flow Showerheads | 47 | 100% | 47 | 1.0 | 47 |
|  | Residential Furnace Tune Up | 2,583 | 100% | 2,583 | 1.0 | 2,583 |
|  | Rim/Band Joist Insulation | 583 | 93% | 543 | 1.0 | 543 |
|  | Wall Insulation | 146 | 100% | 146 | 1.0 | 146 |
|  | Space Heating Boiler Tune-Up | 140 | 100% | 140 | 1.0 | 140 |
| HEA | Advanced Thermostats | 59,332 | 99% | 58,614 | 1.0 | 58,614 |
|  | Air Handler Filter Cleaning/Replacement | 11,748 | 100% | 11,747 | 1.0 | 11,747 |
|  | Air Sealing - Method 2 | 9,512 | 100% | 9,512 | 1.0 | 9,512 |
|  | Boiler Pipe Insulation | 39 | 100% | 39 | 1.0 | 39 |
|  | Domestic Hot Water Pipe Insulation | 4,586 | 99% | 4,552 | 1.0 | 4,552 |
|  | Low Flow Faucet Aerators | 3,914 | 99% | 3,864 | 1.0 | 3,864 |
|  | Low Flow Showerheads | 10,810 | 98% | 10,618 | 1.0 | 10,618 |
|  | Programmable Thermostats | 29,572 | 100% | 29,470 | 1.0 | 29,470 |
|  | Water Heater Temperature Setback | 343 | 100% | 342 | 1.0 | 342 |
| HH | Advanced Thermostats | 1,012 | 100% | 1,012 | 1.0 | 1,012 |
|  | Air Handler Filter Cleaning/Replacement | 78 | 100% | 78 | 1.0 | 78 |
|  | Air Sealing - Method 1 | 2,164 | 100% | 2,164 | 1.0 | 2,164 |
|  | Air Sealing - Method 2 | 105 | 100% | 105 | 1.0 | 105 |
|  | Basement Sidewall Insulation | 248 | 16% | 41 | 1.0 | 41 |
|  | Ceiling/Attic Insulation | 1,162 | 100% | 1,162 | 1.0 | 1,162 |
|  | Domestic Hot Water Pipe Insulation | 78 | 100% | 78 | 1.0 | 78 |
|  | Duct Insulation and Sealing | 5,263 | 100% | 5,263 | 1.0 | 5,263 |
|  | Floor Insulation Above Crawlspace | 138 | 100% | 138 | 1.0 | 138 |
|  | Gas High Efficiency Furnace | 158 | 100% | 158 | 1.0 | 158 |
|  | Gas Water Heater | 58 | 100% | 58 | 1.0 | 58 |
|  | Low Flow Faucet Aerators | 75 | 100% | 75 | 1.0 | 75 |
|  | Low Flow Showerheads | 59 | 100% | 59 | 1.0 | 59 |
|  | Residential Furnace Tune Up | 555 | 100% | 555 | 1.0 | 555 |
|  | Rim/Band Joist Insulation | 90 | 93% | 83 | 1.0 | 83 |
|  | Wall Insulation | 57 | 100% | 57 | 1.0 | 57 |
| IHWAP | Advanced Thermostats | 5,244 | 100% | 5,244 | 1.0 | 5,244 |
|  | Air Sealing - Method 1 | 14,478 | 100% | 14,478 | 1.0 | 14,478 |
|  | Air Sealing - Method 2 | 48 | 100% | 48 | 1.0 | 48 |
|  | Basement Sidewall Insulation | 3,471 | 29% | 1,000 | 1.0 | 1,000 |
|  | Ceiling/Attic Insulation | 17,008 | 100% | 17,008 | 1.0 | 17,008 |
|  | Domestic Hot Water Pipe Insulation | 2,933 | 100% | 2,933 | 1.0 | 2,933 |
|  | Duct Insulation and Sealing | 2,565 | 100% | 2,565 | 1.0 | 2,565 |
|  | Floor Insulation Above Crawlspace | 257 | 100% | 257 | 1.0 | 257 |
|  | Gas High Efficiency Boiler | 1,414 | 100% | 1,414 | 1.0 | 1,414 |
|  | Gas High Efficiency Furnace | 43,251 | 100% | 43,251 | 1.0 | 43,251 |
|  | Gas Water Heater | 3,820 | 100% | 3,820 | 1.0 | 3,820 |
|  | Low Flow Faucet Aerators | 261 | 99% | 259 | 1.0 | 259 |
|  | Low Flow Showerheads | 133 | 100% | 133 | 1.0 | 133 |
|  | Programmable Thermostats | 1,674 | 100% | 1,674 | 1.0 | 1,674 |
|  | Residential Furnace Tune Up | 449 | 100% | 449 | 1.0 | 449 |
|  | Rim/Band Joist Insulation | 1,297 | 100% | 1,292 | 1.0 | 1,292 |
|  | Wall Insulation | 2,809 | 100% | 2,809 | 1.0 | 2,809 |
| Retrofits | Advanced Thermostats | 581 | 100% | 581 | 1.0 | 581 |
|  | Air Sealing - Method 1 | 26,651 | 100% | 26,651 | 1.0 | 26,651 |
|  | Air Sealing - Method 2 | 6,841 | 100% | 6,841 | 1.0 | 6,841 |
|  | Basement Sidewall Insulation | 4,037 | 90% | 3,617 | 1.0 | 3,617 |
|  | Ceiling/Attic Insulation | 35,136 | 100% | 35,136 | 1.0 | 35,136 |
|  | Domestic Hot Water Pipe Insulation | 419 | 100% | 419 | 1.0 | 419 |
|  | Duct Insulation and Sealing | 12,241 | 100% | 12,241 | 1.0 | 12,241 |
|  | Floor Insulation Above Crawlspace | 120 | 100% | 120 | 1.0 | 120 |
|  | Gas High Efficiency Boiler | 652 | 100% | 652 | 1.0 | 652 |
|  | Gas High Efficiency Furnace | 7,491 | 100% | 7,491 | 1.0 | 7,491 |
|  | Gas Water Heater | 324 | 100% | 324 | 1.0 | 324 |
|  | Programmable Thermostats | 695 | 98% | 680 | 1.0 | 680 |
|  | Residential Furnace Tune Up | 3,535 | 100% | 3,535 | 1.0 | 3,535 |
|  | Rim/Band Joist Insulation | 2,169 | 99% | 2,155 | 1.0 | 2,155 |
|  | Wall Insulation | 16,833 | 100% | 16,833 | 1.0 | 16,833 |
|  | Space Heating Boiler Tune-Up | 171 | 100% | 171 | 1.0 | 171 |
| SAP | Advanced Thermostats | 3,334 | 99% | 3,284 | 1.0 | 3,284 |
|  | Air Sealing - Method 2 | 608 | 100% | 608 | 1.0 | 608 |
|  | Boiler Pipe Insulation | 8 | 100% | 8 | 1.0 | 8 |
|  | Domestic Hot Water Pipe Insulation | 302 | 94% | 284 | 1.0 | 284 |
|  | Low Flow Faucet Aerators | 554 | 93% | 514 | 1.0 | 514 |
|  | Low Flow Showerheads | 537 | 112% | 602 | 1.0 | 602 |
|  | Programmable Thermostats | 445 | 100% | 445 | 1.0 | 445 |
|  | Water Heater Temperature Setback | 6 | 98% | 6 | 1.0 | 6 |
| Community Pilot | Advanced Thermostats | 1,391 | 100% | 1,391 | 1.0 | 1,391 |
|  | Air Sealing - Method 2 | 2,302 | 100% | 2,302 | 1.0 | 2,302 |
|  | Ceiling/Attic Insulation | 494 | 100% | 494 | 1.0 | 494 |
|  | Domestic Hot Water Pipe Insulation | 400 | 100% | 400 | 1.0 | 400 |
|  | Duct Insulation and Sealing | 3,496 | 100% | 3,496 | 1.0 | 3,496 |
|  | Gas High Efficiency Furnace | 1,175 | 100% | 1,175 | 1.0 | 1,175 |
|  | Gas Water Heater | 54 | 100% | 54 | 1.0 | 54 |
|  | Programmable Thermostats | 157 | 100% | 157 | 1.0 | 157 |
|  | Residential Furnace Tune Up | 535 | 100% | 535 | 1.0 | 535 |
|  | Rim/Band Joist Insulation | 185 | 59% | 110 | 1.0 | 110 |
|  | Space Heating Boiler Tune-Up | 46 | 100% | 46 | 1.0 | 46 |
| Total |  | 411,008 | 99% | 406,610 | 1.00 | 406,610 |

Source: Nicor Gas tracking data and evaluation team analysis.

# Program Savings Detail

Table 3 summarizes the energy savings the SFIE program achieved by path in 2025.

Table 3. 2025 Annual Energy Savings Summary

| Program Path | Ex Ante Gross Savings (Therms) | Verified Gross RR* | Verified Gross Savings (Therms) | NTG† | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- |
| Ameren Nicor | 34,815 | 100% | 34,775 | 1.0 | 34,775 |
| HEA | 129,856 | 99% | 128,757 | 1.0 | 128,757 |
| HH | 11,299 | 98% | 11,086 | 1.0 | 11,086 |
| IHWAP | 101,111 | 98% | 98,633 | 1.0 | 98,633 |
| Retrofits | 117,897 | 100% | 117,447 | 1.0 | 117,447 |
| SAP | 5,794 | 99% | 5,751 | 1.0 | 5,751 |
| Community Pilot | 10,236 | 99% | 10,160 | 1.0 | 10,160 |
| Total | 411,008 | 99% | 406,610 | 1.0 | 406,610 |

* Realization Rate (RR) is the ratio of verified gross savings to ex ante gross savings, based on evaluation research findings.

† NTG, Net to Gross is the deemed value available on the SAG website: https://www.ilsag.info/evaluator-ntg-recommendations-for-2025/.

Source: Evaluation team analysis.

# Program Savings by Measure

The program includes 20 measures as shown in the following table. The Advanced Thermostats and Ceiling/Attic Insulation measures contributed the most savings.

Table 4. 2025 Annual Energy Savings by Measure

| Program Path | Savings Category | Ex Ante Gross Savings (Therms) | Verified Gross RR* | Verified Gross Savings (Therms) | NTG | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- |
| Ameren Nicor | Advanced Thermostats | 2,083 | 100% | 2,083 | 1.0 | 2,083 |
| Ameren Nicor | Air Handler Filter Cleaning/Replacement | 252 | 100% | 252 | 1.0 | 252 |
| Ameren Nicor | Air Sealing - Method 1 | 6,903 | 100% | 6,903 | 1.0 | 6,903 |
| Ameren Nicor | Air Sealing - Method 2 | 120 | 100% | 120 | 1.0 | 120 |
| Ameren Nicor | Ceiling/Attic Insulation | 3,673 | 100% | 3,673 | 1.0 | 3,673 |
| Ameren Nicor | Domestic Hot Water Pipe Insulation | 563 | 100% | 563 | 1.0 | 563 |
| Ameren Nicor | Duct Insulation and Sealing | 16,163 | 100% | 16,163 | 1.0 | 16,163 |
| Ameren Nicor | Floor Insulation Above Crawlspace | 238 | 100% | 238 | 1.0 | 238 |
| Ameren Nicor | Gas High Efficiency Furnace | 664 | 100% | 664 | 1.0 | 664 |
| Ameren Nicor | Gas Water Heater | 471 | 100% | 471 | 1.0 | 471 |
| Ameren Nicor | Low Flow Faucet Aerators | 188 | 100% | 188 | 1.0 | 188 |
| Ameren Nicor | Low Flow Showerheads | 47 | 100% | 47 | 1.0 | 47 |
| Ameren Nicor | Residential Furnace Tune Up | 2,583 | 100% | 2,583 | 1.0 | 2,583 |
| Ameren Nicor | Rim/Band Joist Insulation | 583 | 93% | 543 | 1.0 | 543 |
| Ameren Nicor | Wall Insulation | 146 | 100% | 146 | 1.0 | 146 |
| Ameren Nicor | Space Heating Boiler Tune-Up | 140 | 100% | 140 | 1.0 | 140 |
| HEA | Advanced Thermostats | 59,332 | 99% | 58,614 | 1.0 | 58,614 |
| HEA | Air Handler Filter Cleaning/Replacement | 11,748 | 100% | 11,747 | 1.0 | 11,747 |
| HEA | Air Sealing - Method 2 | 9,512 | 100% | 9,512 | 1.0 | 9,512 |
| HEA | Boiler Pipe Insulation | 39 | 100% | 39 | 1.0 | 39 |
| HEA | Domestic Hot Water Pipe Insulation | 4,586 | 99% | 4,552 | 1.0 | 4,552 |
| HEA | Low Flow Faucet Aerators | 3,914 | 99% | 3,864 | 1.0 | 3,864 |
| HEA | Low Flow Showerheads | 10,810 | 98% | 10,618 | 1.0 | 10,618 |
| HEA | Programmable Thermostats | 29,572 | 100% | 29,470 | 1.0 | 29,470 |
| HEA | Water Heater Temperature Setback | 343 | 100% | 342 | 1.0 | 342 |
| HH | Advanced Thermostats | 1,012 | 100% | 1,012 | 1.0 | 1,012 |
| HH | Air Handler Filter Cleaning/Replacement | 78 | 100% | 78 | 1.0 | 78 |
| HH | Air Sealing - Method 1 | 2,164 | 100% | 2,164 | 1.0 | 2,164 |
| HH | Air Sealing - Method 2 | 105 | 100% | 105 | 1.0 | 105 |
| HH | Basement Sidewall Insulation | 248 | 16% | 41 | 1.0 | 41 |
| HH | Ceiling/Attic Insulation | 1,162 | 100% | 1,162 | 1.0 | 1,162 |
| HH | Domestic Hot Water Pipe Insulation | 78 | 100% | 78 | 1.0 | 78 |
| HH | Duct Insulation and Sealing | 5,263 | 100% | 5,263 | 1.0 | 5,263 |
| HH | Floor Insulation Above Crawlspace | 138 | 100% | 138 | 1.0 | 138 |
| HH | Gas High Efficiency Furnace | 158 | 100% | 158 | 1.0 | 158 |
| HH | Gas Water Heater | 58 | 100% | 58 | 1.0 | 58 |
| HH | Low Flow Faucet Aerators | 75 | 100% | 75 | 1.0 | 75 |
| HH | Low Flow Showerheads | 59 | 100% | 59 | 1.0 | 59 |
| HH | Residential Furnace Tune Up | 555 | 100% | 555 | 1.0 | 555 |
| HH | Rim/Band Joist Insulation | 90 | 93% | 83 | 1.0 | 83 |
| HH | Wall Insulation | 57 | 100% | 57 | 1.0 | 57 |
| IHWAP | Advanced Thermostats | 5,244 | 100% | 5,244 | 1.0 | 5,244 |
| IHWAP | Air Sealing - Method 1 | 14,478 | 100% | 14,478 | 1.0 | 14,478 |
| IHWAP | Air Sealing - Method 2 | 48 | 100% | 48 | 1.0 | 48 |
| IHWAP | Basement Sidewall Insulation | 3,471 | 29% | 1,000 | 1.0 | 1,000 |
| IHWAP | Ceiling/Attic Insulation | 17,008 | 100% | 17,008 | 1.0 | 17,008 |
| IHWAP | Domestic Hot Water Pipe Insulation | 2,933 | 100% | 2,933 | 1.0 | 2,933 |
| IHWAP | Duct Insulation and Sealing | 2,565 | 100% | 2,565 | 1.0 | 2,565 |
| IHWAP | Floor Insulation Above Crawlspace | 257 | 100% | 257 | 1.0 | 257 |
| IHWAP | Gas High Efficiency Boiler | 1,414 | 100% | 1,414 | 1.0 | 1,414 |
| IHWAP | Gas High Efficiency Furnace | 43,251 | 100% | 43,251 | 1.0 | 43,251 |
| IHWAP | Gas Water Heater | 3,820 | 100% | 3,820 | 1.0 | 3,820 |
| IHWAP | Low Flow Faucet Aerators | 261 | 99% | 259 | 1.0 | 259 |
| IHWAP | Low Flow Showerheads | 133 | 100% | 133 | 1.0 | 133 |
| IHWAP | Programmable Thermostats | 1,674 | 100% | 1,674 | 1.0 | 1,674 |
| IHWAP | Residential Furnace Tune Up | 449 | 100% | 449 | 1.0 | 449 |
| IHWAP | Rim/Band Joist Insulation | 1,297 | 100% | 1,292 | 1.0 | 1,292 |
| IHWAP | Wall Insulation | 2,809 | 100% | 2,809 | 1.0 | 2,809 |
| Retrofits | Advanced Thermostats | 581 | 100% | 581 | 1.0 | 581 |
| Retrofits | Air Sealing - Method 1 | 26,651 | 100% | 26,651 | 1.0 | 26,651 |
| Retrofits | Air Sealing - Method 2 | 6,841 | 100% | 6,841 | 1.0 | 6,841 |
| Retrofits | Basement Sidewall Insulation | 4,037 | 90% | 3,617 | 1.0 | 3,617 |
| Retrofits | Ceiling/Attic Insulation | 35,136 | 100% | 35,136 | 1.0 | 35,136 |
| Retrofits | Domestic Hot Water Pipe Insulation | 419 | 100% | 419 | 1.0 | 419 |
| Retrofits | Duct Insulation and Sealing | 12,241 | 100% | 12,241 | 1.0 | 12,241 |
| Retrofits | Floor Insulation Above Crawlspace | 120 | 100% | 120 | 1.0 | 120 |
| Retrofits | Gas High Efficiency Boiler | 652 | 100% | 652 | 1.0 | 652 |
| Retrofits | Gas High Efficiency Furnace | 7,491 | 100% | 7,491 | 1.0 | 7,491 |
| Retrofits | Gas Water Heater | 324 | 100% | 324 | 1.0 | 324 |
| Retrofits | Programmable Thermostats | 695 | 98% | 680 | 1.0 | 680 |
| Retrofits | Residential Furnace Tune Up | 3,535 | 100% | 3,535 | 1.0 | 3,535 |
| Retrofits | Rim/Band Joist Insulation | 2,169 | 99% | 2,155 | 1.0 | 2,155 |
| Retrofits | Wall Insulation | 16,833 | 100% | 16,833 | 1.0 | 16,833 |
| Retrofits | Space Heating Boiler Tune-Up | 171 | 100% | 171 | 1.0 | 171 |
| SAP | Advanced Thermostats | 3,334 | 99% | 3,284 | 1.0 | 3,284 |
| SAP | Air Sealing - Method 2 | 608 | 100% | 608 | 1.0 | 608 |
| SAP | Boiler Pipe Insulation | 8 | 100% | 8 | 1.0 | 8 |
| SAP | Domestic Hot Water Pipe Insulation | 302 | 94% | 284 | 1.0 | 284 |
| SAP | Low Flow Faucet Aerators | 554 | 93% | 514 | 1.0 | 514 |
| SAP | Low Flow Showerheads | 537 | 112% | 602 | 1.0 | 602 |
| SAP | Programmable Thermostats | 445 | 100% | 445 | 1.0 | 445 |
| SAP | Water Heater Temperature Setback | 6 | 98% | 6 | 1.0 | 6 |
| Community Pilot | Advanced Thermostats | 1,391 | 100% | 1,391 | 1.0 | 1,391 |
| Community Pilot | Air Sealing - Method 2 | 2,302 | 100% | 2,302 | 1.0 | 2,302 |
| Community Pilot | Ceiling/Attic Insulation | 494 | 100% | 494 | 1.0 | 494 |
| Community Pilot | Domestic Hot Water Pipe Insulation | 400 | 100% | 400 | 1.0 | 400 |
| Community Pilot | Duct Insulation and Sealing | 3,496 | 100% | 3,496 | 1.0 | 3,496 |
| Community Pilot | Gas High Efficiency Furnace | 1,175 | 100% | 1,175 | 1.0 | 1,175 |
| Community Pilot | Gas Water Heater | 54 | 100% | 54 | 1.0 | 54 |
| Community Pilot | Programmable Thermostats | 157 | 100% | 157 | 1.0 | 157 |
| Community Pilot | Residential Furnace Tune Up | 535 | 100% | 535 | 1.0 | 535 |
| Community Pilot | Rim/Band Joist Insulation | 185 | 59% | 110 | 1.0 | 110 |
| Community Pilot | Space Heating Boiler Tune-Up | 46 | 100% | 46 | 1.0 | 46 |
| Total |  | 411,008 | 99% | 406,610 | 1.00 | 406,610 |

Source: Evaluation team analysis.

# Impact Analysis Findings and Recommendations

## Impact Parameter Estimates

Table 5 shows the unit per therm savings and realization rate findings by measure from our review. The realization rate is the ratio of the verified savings to the ex ante savings. Following the table, we provide findings and recommendations, including discussion of all measures with realization rates above or below 100%. Appendix A provides a description of the impact analysis methodology.

Table 5. Verified Gross Savings Parameters

| Measure | Unit Basis | Ex Ante Gross (therms/unit) | Verified Gross (therms/unit) | Realization Rate | IL-TRM v13.0 Section* |
| --- | --- | --- | --- | --- | --- |
| Advanced Thermostats | Unit | 81.18 | 80.32 | 99% | 5.3.16 |
| Air Handler Filter Cleaning/Replacement | Unit | 7.90 | 7.90 | 100% | 5.3.21 |
| Air Sealing - Method 1 | Unit | 103.28 | 103.28 | 100% | 5.6.1 |
| Air Sealing - Method 2 | Unit | 0.96 | 0.96 | 100% | 5.6.1 |
| Basement Sidewall Insulation | SQ FT | 0.16 | 0.10 | 60% | 5.6.2 |
| Boiler Pipe Insulation | LN FT | 0.44 | 0.44 | 100% | 5.3.2 |
| Ceiling/Attic Insulation | SQ FT | 0.11 | 0.11 | 100% | 5.6.5 |
| Domestic Hot Water Pipe Insulation | LN FT | 1.97 | 1.96 | 99% | 5.4.1 |
| Duct Insulation and Sealing | Unit | 151.06 | 151.06 | 100% | 5.3.4 |
| Floor Insulation Above Crawlspace | SQ FT | 0.09 | 0.09 | 100% | 5.6.3 |
| Gas High Efficiency Boiler | Unit | 413.11 | 413.11 | 100% | 5.3.6 |
| Gas High Efficiency Furnace | Unit | 280.53 | 280.53 | 100% | 5.3.7 |
| Gas Water Heater | Unit | 54.33 | 54.33 | 100% | 5.4.2 |
| Low Flow Faucet Aerators | Unit | 3.86 | 3.79 | 98% | 5.4.4 |
| Low Flow Showerheads | Unit | 9.30 | 9.20 | 99% | 5.4.5 |
| Programmable Thermostats | Unit | 63.07 | 62.84 | 100% | 5.3.11 |
| Residential Furnace Tune Up | Unit | 34.65 | 34.65 | 100% | 5.3.13 |
| Rim/Band Joist Insulation | Unit | 0.14 | 0.13 | 97% | 5.6.6 |
| Wall Insulation | SQ FT | 0.09 | 0.09 | 100% | 5.6.4 |
| Water Heater Temperature Setback | Unit | 0.25 | 0.25 | 100% | 5.4.6 |

* State of Illinois Technical Reference Manual version 13.0 from http://www.ilsag.info/technical-reference-manual.html

## Findings and Recommendations

Finding 1. The tracking data reported electric fuel type for several Hot Water measures, thus verified savings are zero for these instances (except in the case of water heater temperature setback measures). This includes nine Bathroom Aerator SF (DI), six DHW Pipe Insulation, nine Handheld Showerhead (DI) SF, nine Kitchen Aerator SF (DI), four Shower Flow Reducer, and seven Showerhead (DI) SF measures.

Recommendation 1.  Ensure the fuel type is considered when calculating therms savings. If a 	          measure is entirely electric, no gas savings can be claimed.

Finding 2. The evaluation team found 99 out of 978 low flow showerheads measures (including Handheld Showerhead (DI) SF, Shower Flow Reducer, and Showerhead (DI) SF) used the multifamily value (2.3) for household factor rather than the single-family value (2.76). In the case of an unknown household factor, the single-family value should always be used, even for mobile homes. The realization rate for these measures is 120%.

Recommendation 2.  In the case of a missing household factor value for low flow showerheads, ensure that the single-family value for is used, even for mobile homes, according to the TRM’s guidelines.

Finding 3. The evaluation team found four DWH Pipe Insulation measures (MEA-2025.04.03-886571, MEA-2025.04.03-886569, MEA-2025.04.03-886575, MEA-2025.04.03-886572) had a pipe length of 0.6. The other DHW Pipe Insulation measures had pipe length values of whole numbers, ranging from 1 to 28. The evaluation team concluded that these four measures had a typo in data entry, and a pipe length of 6 was used, resulting in a realization rate of 100%.

Recommendation 3.  Ensure that tracking data is consistent with units and scale.

Finding 4. One low flow faucet aerator, Bathroom Aerator SF (DI) (MEA-2025.06.25-925943), used kitchen values for the drain factor, retrofit daily length of faucet use (L_low), and baseline daily length of faucet use (L_base) inputs rather than bathroom values. This resulted in a realization rate of 120%.

Recommendation 4.  Ensure that all bathroom aerators use the corresponding bathroom values for drain factor, L_low, and L_base, and vice versa for kitchen aerators, according to the TRM’s guidelines.

Finding 5. The tracking data did not include the R-value added Above Ground input for Basement/Sidewall Insulation measures, so the evaluation team assumed a zero value for R-value added Above Ground for all Basement/Sidewall measures. This caused a realization rate range between 3%-30% for 28 out of 137 measures. The rest of Basement/Sidewall measures were not impacted because they did not have above ground insulation installed.

Recommendation 5.   Ensure R-value added Above Ground input is included in the tracking data.

Finding 6. The evaluation team found that 103 out of 896 of the low flow faucet aerators (both Bathroom Aerator SF (DI) and Kitchen Aerator SF (DI)) utilized an in-service rate (ISR) value of 0.77, corresponding to a virtual assessment. The evaluation team was able to recreate these savings, using the ISR of 0.77 provided in the tracking data, but the measure names should include “(VA)” to signify that these were “Virtual Assessment followed by Unverified Self-Install”, assuming this is the case.

Recommendation 6.  Ensure that “(VA)” is in the measure names of measures that included a virtual assessment for low flow faucet aerators.

Finding 7. The evaluation team found 18 out of 1384 water heater temperature setback measures had an electric fuel type, but ex ante therms savings were calculated using Tpre and Tpost values corresponding to a natural gas fuel type. This resulted in a realization rate of 72% for these measures.

Recommendation 7.  Ensure that the correct Tpre and Tpost (temperature) values corresponding to the fuel type are used for water heater temperature setback measures.

Finding 8. The evaluation team found 10 out of 1384 water heater temperature setback measures were indicated as having a shared water heater in the tracking data but also had a recovery efficiency (RE_gas) value of 0.78 in the tracking data. An RE_gas input value of 0.78 corresponds to a Single-Family home. Ex ante savings for these measures used the 0.78 RE_gas value and thus had a realization rate of 86%.

The evaluation team found that these 10 measures could have a realization rate of 100% if a RE_gas value of 0.67, corresponding to a “Shared DHW”, was provided in the tracking data.

Recommendation 8.  Ensure that the correct value for RE_gas is provided in the tracking data for all water heater temperature setback measures.

Finding 9. The evaluation team found that Nicor data was missing values defining whether a space was conditioned or unconditioned for insulation measures. The team was able to pull a column “conditioned_space” from ComEd’s data, but this remained blank for 27 Rim/Band Joist insulation measures. The evaluation team assumed these blank rows indicated an unconditioned space. Utilizing this assumption, we were unable to recreate the ex–ante savings for six out of 27 of these rows. As a result, six out of 261 Rim/Band Joist Insulation measures have realization rates below 100% (two at 58%, two at 59%, and two at 62%).

Recommendation 9.  Ensure that the tracking data contains information about whether a space is conditioned or unconditioned for the insulation measures.

Finding 10. The evaluation team found that for two Air Sealing - Door Sweep measures (MEA-2025.02.12-851345, MEA-2025.02.19-855605), an ISR of 0.68 was used to calculate the ex-ante therms savings.  There were 249 “Air Sealing - Door Sweep” measures in the data, and the ISR provided for these was either 1 or blank. The evaluation team believes that the two measures with an ISR of 0.68 were either misnamed or calculated incorrectly.

Recommendation 10.  If the ISR provided is left blank, ensure that the calculations remain consistent for a given measure name, or that the measure is named correctly.

Finding 11. The evaluation team found 67 of the 540 Air Sealing - Door Sweep - DI measures used an ISR of 0.68 to calculate the ex-ante therms savings. Direct Install air sealing measures should have an ISR of 1.0 according to the TRM. All 540 of these measures had the “in_service_rate” column in the data blank.

Recommendation 11.  If the ISR provided is left blank, ensure that the calculations remain consistent for a given measure name and the measure is named correctly.

Finding 12. For three instances of Advanced Thermostat (DI) – Manual measure (MEA-2025.01.30-843949, MEA-2025.01.30-843955 and MEA-2025.02.26-860012), ex ante calculations applied an 8.5% Heating Reduction factor, applicable when the existing thermostat type is unknown. However, the tracking data reported a 10.2% Heating Reduction factor.

For two instances of Advanced Thermostat (DI) – Manual measure (MEA-2025.05.21-915221 and MEA-2025.07.01-927942), ex ante calculations applied a 7.1% Heating Reduction factor, applicable when the existing thermostat is programmable.

For all these measure instances, the evaluation team applied a Heating Reduction factor of 10.2%, applicable when the existing thermostat type is Manual, consistent with the measure name and IL-TRM v13.0.

Recommendation 12.  Ensure correct Heating Reduction factor applied reflects the baseline measure characterization associated with the measure, as indicated by the measure name.

Finding 13. For 18 out of 40 total instances of Advanced Thermostat (DI) – Programmable measure in the Self-Assessment Program (SAP), ex ante savings calculations used an ISR for heating as 100%, associated with Direct Install measures. The evaluation team applied a 54% ISR, associated with self-install thermostats, consistent with the program delivery method and IL-TRM v13.0. Advanced Thermostats account for 57% of SAP component’s reported gross therms savings and the impact of the adjustments from Findings 18 and 19 resulted in a realization rate of 78% for this measure.

Recommendation 13.  Ensure the correct ISR value is used and reflects the baseline measure characterization associated with the measure, as indicated by the measure name.

Finding 14. For four instances of Advanced Thermostat (DI) – Blended measure, ex ante calculations used Heating Reduction as 10.2%. However, the tracking data reported Heating Reduction as 8.5%. The evaluation team used Heating Reduction as 8.5%, applicable for Blended existing thermostat type, consistent with the measure name. The impact of this adjustment resulted in a realization rate of 83% for these measure instances.

Recommendation 14.  Ensure savings are calculated, consistent with the measure name.

# Appendix A. Impact Analysis Methodology

Impact Analysis Supplemental Information

The evaluation team determined the verified gross for each program measure by the following methodology:

Reviewing the savings algorithm inputs in the measure workbook with the IL TRM v13 and IL-TRM Errata, when applicable.

Referencing SFHU Savings Calculator to validate the savings algorithm was applied correctly.

- Cross-checking ComEd’s 2025 year-end data to validate missing data.

# Appendix B. Program Specific Inputs for the Illinois TRC

Table B-1 shows the Total Resource Cost (TRC) cost-effectiveness analysis inputs available at the time of producing this impact evaluation report. Additional required cost data (e.g., measure costs, program level incentive and non-incentive costs) are not included in this table and will be provided to the evaluation team later. Guidehouse will include annual and lifetime water savings and greenhouse gas reductions in the end of year summary report.

Table B-1. Verified Cost Effectiveness Inputs

| Program Path | Savings Category | DAC Project* | Units | Quantity | Effective Useful Life | Early Replacement Flag | Verified Gross Annual Water Savings (Gallons) | Ex Ante Gross Savings (Therms) | Verified Gross Savings (Therms) | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ameren Nicor | Advanced Thermostats | FALSE | Unit | 25 | 11.00 | NO | - | 2,083 | 2,083 | 2,083 |
| Ameren Nicor | Air Handler Filter Cleaning/Replacement | FALSE | Unit | 37 | 3.00 | NO | - | 252 | 252 | 252 |
| Ameren Nicor | Air Sealing - Method 1 | FALSE | Unit | 39 | 20.00 | YES | - | 6,903 | 6,903 | 6,903 |
| Ameren Nicor | Air Sealing - Method 2 | FALSE | Unit | 19 | 20.00 | YES | - | 120 | 120 | 120 |
| Ameren Nicor | Ceiling/Attic Insulation | FALSE | SQ FT | 20,115 | 30.00 | YES | - | 3,673 | 3,673 | 3,673 |
| Ameren Nicor | Domestic Hot Water Pipe Insulation | FALSE | LN FT | 266 | 15.00 | NO | - | 563 | 563 | 563 |
| Ameren Nicor | Duct Insulation and Sealing | FALSE | Unit | 37 | 20.00 | NO | - | 16,163 | 16,163 | 16,163 |
| Ameren Nicor | Floor Insulation Above Crawlspace | FALSE | SQ FT | 1,700 | 30.00 | YES | - | 238 | 238 | 238 |
| Ameren Nicor | Gas High Efficiency Furnace | FALSE | Unit | 4 | 20.00 | YES | - | 664 | 664 | 664 |
| Ameren Nicor | Gas Water Heater | FALSE | Unit | 4 | 14.50 | YES | - | 471 | 471 | 471 |
| Ameren Nicor | Low Flow Faucet Aerators | FALSE | Unit | 38 | 10.00 | NO | 213.56 | 188 | 188 | 188 |
| Ameren Nicor | Low Flow Showerheads | FALSE | Unit | 14 | 10.00 | NO | 43.21 | 47 | 47 | 47 |
| Ameren Nicor | Residential Furnace Tune Up | FALSE | Unit | 28 | 3.00 | NO | - | 2,583 | 2,583 | 2,583 |
| Ameren Nicor | Rim/Band Joist Insulation | FALSE | Unit | 5,420 | 30.00 | YES | - | 583 | 543 | 543 |
| Ameren Nicor | Wall Insulation | FALSE | SQ FT | 1,000 | 30.00 | YES | - | 146 | 146 | 146 |
| Ameren Nicor | Space Heating Boiler Tune-Up | FALSE | Unit | 1 | 3.00 | NO | - | 140 | 140 | 140 |
| HEA | Advanced Thermostats | FALSE | Unit | 726 | 11.00 | NO | - | 59,332 | 58,614 | 58,614 |
| HEA | Air Handler Filter Cleaning/Replacement | FALSE | Unit | 1,482 | 3.00 | NO | - | 11,748 | 11,747 | 11,747 |
| HEA | Air Sealing - Method 2 | FALSE | Unit | 1,300 | 20.00 | YES | - | 9,512 | 9,512 | 9,512 |
| HEA | Boiler Pipe Insulation | FALSE | LN FT | 90 | 11.00 | NO | - | 39 | 39 | 39 |
| HEA | Domestic Hot Water Pipe Insulation | FALSE | LN FT | 24,12 | 15.00 | NO | 4,486.45 | 3,914 | 3,864 | 3,864 |
| HEA | Low Flow Faucet Aerators | FALSE | Unit | 948 | 10.00 | NO | 10,029.10 | 10,810 | 10,618 | 10,618 |
| HEA | Low Flow Showerheads | FALSE | Unit | 1,053 | 10.00 | NO | - | 29,572 | 29,470 | 29,470 |
| HEA | Programmable Thermostats | FALSE | Unit | 469 | 16.00 | YES | - | 343 | 342 | 342 |
| HEA | Water Heater Temperature Setback | FALSE | Unit | 1,362 | 2.00 | NO | - | 1,012 | 1,012 | 1,012 |
| HH | Advanced Thermostats | FALSE | Unit | 10 | 11.00 | NO | - | 78 | 78 | 78 |
| HH | Air Handler Filter Cleaning/Replacement | FALSE | Unit | 10 | 3.00 | NO | - | 2,164 | 2,164 | 2,164 |
| HH | Air Sealing - Method 1 | FALSE | Unit | 17 | 20.00 | YES | - | 105 | 105 | 105 |
| HH | Air Sealing - Method 2 | FALSE | Unit | 135 | 20.00 | YES | - | 248 | 41 | 41 |
| HH | Basement Sidewall Insulation | FALSE | SQ FT | 868 | 30.00 | YES | - | 1,162 | 1,162 | 1,162 |
| HH | Ceiling/Attic Insulation | FALSE | SQ FT | 13,257 | 30.00 | YES | - | 78 | 78 | 78 |
| HH | Domestic Hot Water Pipe Insulation | FALSE | LN FT | 37 | 15.00 | NO | - | 5,263 | 5,263 | 5,263 |
| HH | Floor Insulation Above Crawlspace | FALSE | SQ FT | 800 | 30.00 | YES | - | 138 | 138 | 138 |
| HH | Gas High Efficiency Furnace | FALSE | Unit | 1 | 20.00 | YES | - | 158 | 158 | 158 |
| HH | Gas Water Heater | FALSE | Unit | 1 | 14.50 | YES | - | 58 | 58 | 58 |
| HH | Low Flow Faucet Aerators | FALSE | Unit | 8 | 10.00 | NO | 84.14 | 75 | 75 | 75 |
| HH | Low Flow Showerheads | FALSE | Unit | 11 | 10.00 | NO | 54.29 | 59 | 59 | 59 |
| HH | Residential Furnace Tune Up | FALSE | Unit | 7 | 3.00 | NO | - | 555 | 555 | 555 |
| HH | Rim/Band Joist Insulation | FALSE | Unit | 819 | 30.00 | YES | - | 90 | 83 | 83 |
| HH | Wall Insulation | FALSE | SQ FT | 537 | 30.00 | YES | - | 57 | 57 | 57 |
| IHWAP | Advanced Thermostats | FALSE | Unit | 59 | 11.00 | NO | - | 5,244 | 5,244 | 5,244 |
| IHWAP | Air Sealing - Method 1 | FALSE | Unit | 164 | 20.00 | YES | - | 14,478 | 14,478 | 14,478 |
| IHWAP | Air Sealing - Method 2 | FALSE | Unit | 73 | 20.00 | YES | - | 48 | 48 | 48 |
| IHWAP | Basement Sidewall Insulation | FALSE | SQ FT | 15,691 | 30.00 | YES | - | 3,471 | 1,000 | 1,000 |
| IHWAP | Ceiling/Attic Insulation | FALSE | SQ FT | 169,359 | 30.00 | YES | - | 17,008 | 17,008 | 17,008 |
| IHWAP | Domestic Hot Water Pipe Insulation | FALSE | LN FT | 1,434 | 15.00 | NO | - | 2,933 | 2,933 | 2,933 |
| IHWAP | Duct Insulation and Sealing | FALSE | Unit | 10 | 20.00 | YES | - | 2,565 | 2,565 | 2,565 |
| IHWAP | Floor Insulation Above Crawlspace | FALSE | SQ FT | 4,589 | 30.00 | YES | - | 257 | 257 | 257 |
| IHWAP | Gas High Efficiency Boiler | FALSE | Unit | 2 | 25.00 | YES | - | 1,414 | 1,414 | 1,414 |
| IHWAP | Gas High Efficiency Furnace | FALSE | Unit | 142 | 20.00 | YES | - | 43,251 | 43,251 | 43,251 |
| IHWAP | Gas Water Heater | FALSE | Unit | 70 | 14.50 | YES | - | 3,820 | 3,820 | 3,820 |
| IHWAP | Low Flow Faucet Aerators | FALSE | Unit | 116 | 10.00 | NO | 315.97 | 261 | 259 | 259 |
| IHWAP | Low Flow Showerheads | FALSE | Unit | 38 | 10.00 | NO | 123.04 | 133 | 133 | 133 |
| IHWAP | Programmable Thermostats | FALSE | Unit | 26 | 16.00 | YES | - | 1,674 | 1,674 | 1,674 |
| IHWAP | Residential Furnace Tune Up | FALSE | Unit | 11 | 3.00 | NO | - | 449 | 449 | 449 |
| IHWAP | Rim/Band Joist Insulation | FALSE | Unit | 9,600 | 30.00 | YES | - | 1,297 | 1,292 | 1,292 |
| IHWAP | Wall Insulation | FALSE | SQ FT | 25,317 | 30.00 | YES | - | 2,809 | 2,809 | 2,809 |
| Retrofits | Advanced Thermostats | FALSE | Unit | 7 | 11.00 | NO | - | 581 | 581 | 581 |
| Retrofits | Air Sealing - Method 1 | FALSE | Unit | 266 | 20.00 | YES | - | 26,651 | 26,651 | 26,651 |
| Retrofits | Air Sealing - Method 2 | FALSE | Unit | 13,729 | 20.00 | YES | - | 6,841 | 6,841 | 6,841 |
| Retrofits | Basement Sidewall Insulation | FALSE | SQ FT | 32,143 | 30.00 | YES | - | 4,037 | 3,617 | 3,617 |
| Retrofits | Ceiling/Attic Insulation | FALSE | SQ FT | 329,200 | 30.00 | YES | - | 35,136 | 35,136 | 35,136 |
| Retrofits | Domestic Hot Water Pipe Insulation | FALSE | LN FT | 208 | 15.00 | NO | - | 419 | 419 | 419 |
| Retrofits | Duct Insulation and Sealing | FALSE | Unit | 177 | 20.00 | YES | - | 12,241 | 12,241 | 12,241 |
| Retrofits | Floor Insulation Above Crawlspace | FALSE | SQ FT | 900 | 30.00 | YES | - | 120 | 120 | 120 |
| Retrofits | Gas High Efficiency Boiler | FALSE | Unit | 3 | 25.00 | YES | - | 652 | 652 | 652 |
| Retrofits | Gas High Efficiency Furnace | FALSE | Unit | 36 | 20.00 | YES | - | 7,491 | 7,491 | 7,491 |
| Retrofits | Gas Water Heater | FALSE | Unit | 11 | 14.50 | YES | - | 324 | 324 | 324 |
| Retrofits | Programmable Thermostats | FALSE | Unit | 10 | 16.00 | YES | - | 695 | 680 | 680 |
| Retrofits | Residential Furnace Tune Up | FALSE | Unit | 156 | 3.00 | NO | - | 3,535 | 3,535 | 3,535 |
| Retrofits | Rim/Band Joist Insulation | FALSE | Unit | 14,681 | 30.00 | YES | - | 2,169 | 2,155 | 2,155 |
| Retrofits | Wall Insulation | FALSE | SQ FT | 184,292 | 30.00 | YES | - | 16,833 | 16,833 | 16,833 |
| Retrofits | Space Heating Boiler Tune-Up | FALSE | Unit | 4 | 3.00 | NO | - | 171 | 171 | 171 |
| SAP | Advanced Thermostats | FALSE | Unit | 56 | 11.00 | NO | - | 3,334 | 3,284 | 3,284 |
| SAP | Air Sealing - Method 2 | FALSE | Unit | 122 | 20.00 | YES | - | 608 | 608 | 608 |
| SAP | Boiler Pipe Insulation | FALSE | LN FT | 18 | 11.00 | YES | - | 8 | 8 | 8 |
| SAP | Domestic Hot Water Pipe Insulation | FALSE | LN FT | 150 | 15.00 | NO | - | 302 | 284 | 284 |
| SAP | Low Flow Faucet Aerators | FALSE | Unit | 182 | 10.00 | NO | 782.10 | 554 | 514 | 514 |
| SAP | Low Flow Showerheads | FALSE | Unit | 130 | 10.00 | NO | 598.28 | 537 | 602 | 602 |
| SAP | Programmable Thermostats | FALSE | Unit | 9 | 16.00 | YES | - | 445 | 445 | 445 |
| SAP | Water Heater Temperature Setback | FALSE | Unit | 24 | 2.00 | NO | - | 6 | 6 | 6 |
| Community Pilot | Advanced Thermostats | FALSE | Unit | 16 | 11.00 | NO | - | 1,391 | 1,391 | 1,391 |
| Community Pilot | Air Sealing - Method 2 | FALSE | Unit | 4,929 | 20.00 | YES | - | 2,302 | 2,302 | 2,302 |
| Community Pilot | Ceiling/Attic Insulation | FALSE | SQ FT | 5,313 | 30.00 | YES | - | 494 | 494 | 494 |
| Community Pilot | Domestic Hot Water Pipe Insulation | FALSE | LN FT | 198 | 15.00 | NO | - | 400 | 400 | 400 |
| Community Pilot | Duct Insulation and Sealing | FALSE | Unit | 22 | 20.00 | NO | - | 3,496 | 3,496 | 3,496 |
| Community Pilot | Gas High Efficiency Furnace | FALSE | Unit | 5 | 20.00 | YES | - | 1,175 | 1,175 | 1,175 |
| Community Pilot | Gas Water Heater | FALSE | Unit | 1 | 14.50 | YES | - | 54 | 54 | 54 |
| Community Pilot | Programmable Thermostats | FALSE | Unit | 2 | 16.00 | YES | - | 157 | 157 | 157 |
| Community Pilot | Residential Furnace Tune Up | FALSE | Unit | 19 | 3.00 | NO | - | 535 | 535 | 535 |
| Community Pilot | Rim/Band Joist Insulation | FALSE | Unit | 1,165 | 30.00 | YES | - | 185 | 110 | 110 |
| Community Pilot | Space Heating Boiler Tune-Up | FALSE | Unit | 1 | 3.00 | NO | - | 46 | 46 | 46 |
| Total or Weighted Average |  |  |  |  | 18.80 |  | 16,730 | 411,008 | 406,610 | 406,610 |

* DAC – Disadvantaged Communities

Source: Evaluation team analysis.