# Home Energy Efficiency Rebates Program Impact Evaluation Report

Energy Efficiency Plan: Program Year 2025

(01/01/2025-12/31/2025)

Prepared for:

Nicor Gas Company

DRAFT

March 27, 2026

Prepared by:

| Julia Kaplan Guidehouse | Robert Hodge Guidehouse |  |  |
| --- | --- | --- | --- |

Submitted to:

Nicor Gas Company

1844 Ferry Road

Naperville, IL 60563

Submitted by:

Guidehouse Inc.

167 N. Green Street

12th Floor

Chicago, IL 60607

Contact:

| Charles Maglione, Partner 202.481.7352 cmaglione@guidehouse.com | Jeff Erickson, Director 608.616.4962 jeff.erickson@guidehouse.com | Laura Agapay-Read, Associate Director 312.583.4178 laura.agapay.read@guidehouse.com |
| --- | --- | --- |

Charles Ampong, Associate Director

608.616.4922

charles.ampong@guidehouse.com

This report was prepared by Guidehouse for Nicor Gas Company. The work presented in this report represents Guidehouse’s professional judgment based on the information available at the time this report was prepared. Use of this report by any other party for whatever purpose should not, and does not, absolve such party from using due diligence in verifying the report’s contents. Neither Guidehouse nor any of its subsidiaries or affiliates assumes any liability or duty of care to such parties and hereby disclaims any such liability.

Table of Contents

1	Introduction	1

2	Program Description	1

3	Program Savings Detail	2

4	Program Savings by Measure	2

5	Impact Analysis Findings and Recommendations	4

5.1	Impact Parameter Estimates	4

5.2	Findings and Recommendations	4

Appendix A. Impact Analysis Methodology	6

Appendix B. Program Specific Inputs for the Illinois TRC	7

Table of Tables, Figures, and Equations

Table 1. 2025 Volumetric Findings Detail	1

Table 2. 2025 Installed Measure Quantities	1

Table 3. 2025 Annual Energy Savings Summary	2

Table 4. 2025 Annual Energy Savings by Measure	2

Table 5. Verified Gross Savings Parameters	4

# Introduction

This report presents the results of the impact evaluation of the Nicor Gas 2025 Home Energy Efficiency Rebates (HEER) program. It presents a summary of the energy impacts for the total program and broken out by relevant measure and program structure details. The appendices present the impact analysis methodology and Illinois total resource cost (TRC) inputs. Program year 2025 covered January 1 to December 31, 2025.

# Program Description

The HEER program provides Nicor Gas customers with rebates for purchasing high annual fuel utilization efficiency (AFUE) furnaces and boilers, tankless water heaters, advanced thermostats, and pool covers. Participants may apply for the rebates themselves, or contractors may assist them in the rebate application process. Rebates are processed and sent to residential customers after installation of qualified measures. Members of the Nicor Gas Contractor Circle may offer rebates as instant discounts.

The program had 9,763 participants in 2025 and completed 10,247 projects as shown in Table 1.

Table 1. 2025 Volumetric Findings Detail

| Participation | Total |
| --- | --- |
| Participants * | 9,763 |
| Installed Projects † | 10,247 |
| Measure Types Installed ‡ | 8 |

* Participants are defined as the distinct count of addresses.

† Installed Projects are defined as the distinct count of project IDs.

‡ Measure Types Installed are defined as the distinct count of Nicor Gas measure names.

Source: Nicor Gas tracking data and evaluation team analysis.

Table 2 summarizes the installed measure quantities that are the basis for verified energy savings.

Table 2. 2025 Installed Measure Quantities

| Measure | Quantity Unit | Installed Quantity |
| --- | --- | --- |
| Advanced Thermostat - Manual Baseline | Each | 1,470 |
| Advanced Thermostat - Programmable Baseline | Each | 1,477 |
| Boilers, >95% AFUE <300 MBH | Each | 96 |
| Combination Boilers, >95% AFUE <300 MBH | Each | 71 |
| Furnace, >95% AFUE | Each | 4,482 |
| Furnace, >97% AFUE | Each | 3,969 |
| Outdoor Pool Covers | Each | 10 |
| Tankless Water Heater | Each | 379 |

Source: Nicor Gas tracking data and evaluation team analysis.

# Program Savings Detail

Table 3 summarizes the energy savings the HEER Program achieved in 2025.

Table 3. 2025 Annual Energy Savings Summary

| Program Category | Ex Ante Gross Savings (Therms) | Verified Gross RR* | Verified Gross Savings (Therms) | NTG† | NPSO‡ | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- |
| Non-Disadvantaged Communities | 1,857,211 | 100% | 1,863,415 | 0.86 | 1.048 | 1,684,654 |
| Disadvantaged Communities (DAC) | 112,362 | 100% | 112,770 | 1.00 | N/A | 112,770 |
| Total | 1,969,573 | 100% | 1,976,184 |  |  | 1,797,424 |

* Realization Rate (RR) is the ratio of verified gross savings to ex ante gross savings, based on evaluation research findings.

† The program-level NTG values are calculated as Verified Net/Verified Gross. The program-level analysis typically used measure-level deemed NTG values (see Table 4), which are listed here: https://www.ilsag.info/evaluator-ntg-recommendations-for-2025.

‡ The Non-Disadvantaged Communities net savings were multiplied by a residential non-participant spillover (NPSO) factor of 1.048.

Source: Evaluation team analysis.

# Program Savings by Measure

The program includes 8 measures as shown in the following table. The advanced thermostat and furnace measures contributed the most savings. Projects in the disadvantaged communities (DAC) sites have a verified net-to-gross ratio (NTG) of 1.00.

Table 4. 2025 Annual Energy Savings by Measure

| Program Category | Savings Category | Ex Ante Gross Savings (Therms) | Verified Gross RR* | Verified Gross Savings (Therms) | NTG† | NPSO‡ | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Non-DAC | Advanced Thermostat - Manual Baseline | 141,301 | 100% | 141,301 | 0.95 | 1.048 | 140,680 |
|  | Advanced Thermostat - Programmable Baseline | 98,106 | 100% | 98,106 | 0.95 | 1.048 | 97,674 |
|  | Boilers, >95% AFUE <300 MBH | 22,622 | 100% | 22,622 | 0.84 | 1.048 | 19,914 |
|  | Combination Boilers, >95% AFUE <300 MBH | 14,946 | 101% | 15,065 | 0.84 | 1.048 | 13,262 |
|  | Furnace, >95% AFUE | 757,765 | 100% | 761,004 | 0.84 | 1.048 | 669,927 |
|  | Furnace, >97% AFUE | 791,169 | 100% | 794,551 | 0.86 | 1.048 | 716,113 |
|  | Outdoor Pool Covers | 3,167 | 100% | 3,167 | 0.84 | 1.048 | 2,788 |
|  | Tankless Water Heater | 28,134 | 98% | 27,600 | 0.84 | 1.048 | 24,296 |
| Non-DAC Subtotal |  | 1,857,211 | 100% | 1,863,415 |  |  | 1,684,654 |
| DAC | Advanced Thermostat - Manual Baseline | 5,566 | 100% | 5,566 | 1.00 | N/A | 5,566 |
|  | Advanced Thermostat - Programmable Baseline | 4,166 | 100% | 4,166 | 1.00 | N/A | 4,166 |
|  | Boilers, >95% AFUE <300 MBH | 669 | 100% | 669 | 1.00 | N/A | 669 |
|  | Combination Boilers, >95% AFUE <300 MBH | 626 | 101% | 631 | 1.00 | N/A | 631 |
|  | Furnace, >95% AFUE | 59,276 | 100% | 59,529 | 1.00 | N/A | 59,529 |
|  | Furnace, >97% AFUE | 40,100 | 100% | 40,271 | 1.00 | N/A | 40,271 |
|  | Outdoor Pool Covers | 792 | 100% | 792 | 1.00 | N/A | 792 |
|  | Tankless Water Heater | 1,168 | 98% | 1,146 | 1.00 | N/A | 1,146 |
| DAC Subtotal |  | 112,362 | 100% | 112,770 |  |  | 112,770 |
| Total |  | 1,969,573 | 100% | 1,976,184 |  |  | 1,797,424 |

† The program-level NTG values are calculated as Verified Net/Verified Gross. The program-level analysis typically used measure-level deemed NTG values, which are listed here: https://www.ilsag.info/evaluator-ntg-recommendations-for-2025.

‡ The Non-Disadvantaged Communities net savings were multiplied by a residential non-participant spillover (NPSO) factor of 1.048. Based on SAG Policy, for DAC projects, if deemed NTG is multiplied by the NPSO and the resulted NTG value is less than 1.00, the evaluation assigned a DAC NTG of 1.00. If the resulted NTG value is >1.00, evaluation used the >1.00 value for calculation of net savings impact (the NPSO in this case is left as N/A).

Source: Evaluation team analysis.

# Impact Analysis Findings and Recommendations

## Impact Parameter Estimates

Table 5 shows the unit therm savings and realization rate findings by measure from our review. The realization rate is the ratio of the verified savings to the ex ante savings. Following the table, we provide findings and recommendations, including discussion of all measures with realization rates above or below 100%. Appendix A provides a description of the impact analysis methodology.

Table 5. Verified Gross Savings Parameters

| Measure | Unit Basis | Ex Ante Gross (therms/unit) | Verified Gross (therms/unit) | Realization Rate | IL-TRM v13.0 Section† |
| --- | --- | --- | --- | --- | --- |
| Advanced Thermostat - Manual Baseline | Each | Varies | Varies | 100% | 5.3.16 |
| Advanced Thermostat - Programmable Baseline | Each | Varies | Varies | 100% | 5.3.16 |
| Boilers, >95% AFUE <300 MBH | Each | Varies | Varies | 100% | 5.3.6 |
| Combination Boilers, >95% AFUE <300 MBH | Each | Varies | Varies | 101% | 5.3.17 |
| Furnace, >95% AFUE | Each | Varies | Varies | 100% | 5.3.7 |
| Furnace, >97% AFUE | Each | Varies | Varies | 100% | 5.3.7 |
| Outdoor Pool Covers | Each | 395.92 | 395.92 | 100% | 5.4.10 |
| Tankless Water Heater | Each | Varies | Varies | 98% | 5.4.2 |

† State of Illinois Technical Reference Manual version 13.0 from http://www.ilsag.info/technical-reference-manual.html .

## Findings and Recommendations

Overall, the findings listed below had a minor impact on the savings, with the program-level realization rate being 100%.

Finding 1. All instances of Combination Boilers, >95% AFUE <300 MBH had a verified gross realization rate of 101%. It appears that the ex ante calculation used a household factor of 2.56 which is assigned to single-family participants in the TRM v12.0. In these instances, Guidehouse verified savings used a household factor of 2.62, which is assigned to single-family non-income qualified participants in the TRM v13.0.

Finding 2. All instances of Tankless Water Heater had a verified gross realization rate of 98%. It appears that the ex ante calculation used a household factor of 2.67 and 2.18, which is assigned to single-family all participants and multifamily all participants in the TRM, respectively. In these instances, Guidehouse verified savings assigned a household factor of 2.62 and 2.09, which is assigned to single-family non-income qualified and multifamily non-income qualified participants in the TRM.

Recommendation 1.  Ensure the household factor values align with the applicable TRM measure section to accurately calculate “GrossAnnualThermSavings” column in the “Measures” tab of tracking data.

# Appendix A. Impact Analysis Methodology

The evaluation verified gross savings were determined for each program measure by:

- Reviewing the savings algorithm inputs in the final 2025 tracking data for agreement with the IL-TRM v13.0 and IL-TRM Errata, where applicable.

- Validating the savings algorithm was applied correctly.

- Cross-checking per-unit savings values in the program tracking data with the verified values in Guidehouse’s calculations.

- Multiplying the verified per-unit savings value by the quantity reported in the tracking data. The team calculated verified net savings by multiplying the verified gross savings estimates by a NTG ratio. In Program Year 2024, NTG estimates used to calculate the net verified savings were based on past evaluation research and defined by a consensus process through the Illinois SAG.

- For DAC projects, if deemed NTG is multiplied by 1.048 Non-Participant Spillover factor (NPSO) and the resulted NTG value is less than 1.00, the evaluation assigned a DAC NTG of 1.00. If the resulted NTG value is >1.00, evaluation used the >1.00 value for calculation of net savings impact.

# Appendix B. Program Specific Inputs for the Illinois TRC

Table B-1 shows the Total Resource Cost (TRC) cost-effectiveness analysis inputs available at the time of producing this impact evaluation report. Additional required cost data (e.g., measure costs, program level incentive and non-incentive costs) are not included in this table and will be provided to the evaluation team later. Guidehouse will include annual and lifetime water savings and greenhouse gas reductions in the end of year summary report.

Table B-1. Verified Cost Effectiveness Inputs

| Savings Category | DAC Project* | Units | Quantity | Effective Useful Life | Early Replacement Flag | Ex Ante Gross Savings (Therms) | Verified Gross Savings (Therms) | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Advanced Thermostat - Manual Baseline | FALSE | Each | 1,412 | 11.00 | NO | 141,301 | 141,301 | 140,680 |
| Advanced Thermostat - Manual Baseline | TRUE | Each | 58 | 11.00 | NO | 5,566 | 5,566 | 5,566 |
| Advanced Thermostat - Programmable Baseline | FALSE | Each | 1,416 | 11.00 | NO | 98,106 | 98,106 | 97,674 |
| Advanced Thermostat - Programmable Baseline | TRUE | Each | 61 | 11.00 | NO | 4,166 | 4,166 | 4,166 |
| Boilers, >95% AFUE <300 MBH | FALSE | Each | 93 | 25.00 | YES | 22,622 | 22,622 | 19,914 |
| Boilers, >95% AFUE <300 MBH | TRUE | Each | 3 | 25.00 | YES | 669 | 669 | 669 |
| Combination Boilers, >95% AFUE <300 MBH | FALSE | Each | 68 | 21.50 | NO | 14,946 | 15,065 | 13,262 |
| Combination Boilers, >95% AFUE <300 MBH | TRUE | Each | 3 | 21.50 | NO | 626 | 631 | 631 |
| Furnace, >95% AFUE | FALSE | Each | 4,108 | 20.00 | YES | 757,765 | 761,004 | 669,927 |
| Furnace, >95% AFUE | TRUE | Each | 374 | 20.00 | YES | 59,276 | 59,529 | 59,529 |
| Furnace, >97% AFUE | FALSE | Each | 3,749 | 20.00 | YES | 791,169 | 794,551 | 716,113 |
| Furnace, >97% AFUE | TRUE | Each | 220 | 20.00 | YES | 40,100 | 40,271 | 40,271 |
| Outdoor Pool Covers | FALSE | Each | 8 | 6.00 | NO | 3,167 | 3,167 | 2,788 |
| Outdoor Pool Covers | TRUE | Each | 2 | 6.00 | NO | 792 | 792 | 792 |
| Tankless Water Heater | FALSE | Each | 364 | 20.00 | NO | 28,134 | 27,600 | 24,296 |
| Tankless Water Heater | TRUE | Each | 15 | 20.00 | NO | 1,168 | 1,146 | 1,146 |
| Total or Weighted Average |  |  |  | 18.91 |  | 1,969,573 | 1,976,184 | 1,797,424 |

Source: Evaluation team analysis.