# C&I and Public Sector Prescriptive Program Impact Evaluation Report

Energy Efficiency Plan: Program Year 2025

(01/01/2025-12/31/2025)

Prepared for:

Peoples Gas and North Shore Gas

DRAFT

March 27, 2026

Prepared by:

| Sophie Berne Guidehouse, Inc. |  |  |  |
| --- | --- | --- | --- |

Submitted to:

Peoples Gas

North Shore Gas

200 East Randolph Street

Chicago, IL 60601

Submitted by:

Guidehouse Inc.

167 N. Green Street

12th Floor

Chicago, IL 60607

Contact:

| Charles Maglione, Partner 202.481.7352 cmaglione@guidehouse.com | Jeff Erickson, Director 608.616.4962 jeff.erickson@guidehouse.com | Laura Agapay-Read, Associate Director 312.583.4178 laura.agapay.read@guidehouse.com |
| --- | --- | --- |

Charles Ampong, Associate Director

608.616.4922

charles.ampong@guidehouse.com

This report was prepared by Guidehouse for People Gas Light and Coke Company (PGL) and North Shore Gas (NSG) companies. The work presented in this report represents Guidehouse’s professional judgment based on the information available at the time this report was prepared. Use of this report by any other party for whatever purpose should not, and does not, absolve such party from using due diligence in verifying the report’s contents. Neither Guidehouse nor any of its subsidiaries or affiliates assumes any liability or duty of care to such parties and hereby disclaims any such liability.

Table of Contents

1	Introduction	1

2	Program Description	1

3	Program Savings Detail	3

4	Program Savings by Measure	4

5	Impact Analysis Findings and Recommendations	7

5.1	Impact Parameter Estimates	7

5.2	Findings and Recommendations	8

Appendix A. Impact Analysis Methodology	10

Appendix B. Program Specific Inputs for the Illinois TRC	11

Table of Tables, Figures, and Equations

Table 1. 2025 Volumetric Summary for PGL	1

Table 2. 2025 Installed Measure Quantities for PGL	2

Table 3. 2025 Volumetric Summary for NSG	2

Table 4. 2025 Installed Measure Quantities for NSG	3

Table 5. 2025 Annual Energy Savings Summary for PGL	3

Table 6. 2025 Annual Energy Savings Summary for NSG	4

Table 7. 2025 Annual Energy Savings by Measure for PGL	4

Table 8. 2025 Annual Energy Savings by Measure for NSG	6

Table 9. Verified Gross Savings Parameters	7

# Introduction

This report presents the results of the impact evaluation of the Peoples Gas (PGL) and North Shore Gas (NSG) 2025 Commercial and Industrial (C&I) and Public Sector Prescriptive programs. It presents a summary of the energy impacts for the total program and broken out by relevant measure and program structure details. Appendix A presents the impact analysis methodology. Appendix B presents the Total Resource Cost (TRC) cost-effectiveness analysis inputs. Program year 2025 covers January 1 to December 31, 2025.

# Program Description

The PGL and NSG comprehensive business programs combine offerings into program paths and allow eligible C&I and Public Sector customers access to any of the paths based on the needs of each customer. This report covers evaluation activities for measures installed and natural gas savings realized through the Prescriptive Rebate path, referred to as the Prescriptive program in this report. The program provides standardized incentives to cover part of the measure costs for existing commercial, industrial, and public sector buildings. The incentives focus on energy efficient measures, such as space heating systems, water heating systems, pipe and storage tank insulation, steam traps, and food service equipment.

The PGL program had 31 participants (16 C&I and 15 public sector participants) in 2025 and completed 246 projects (24 C&I and 222 public sector projects) as shown in the following table.

Table 1. 2025 Volumetric Summary for PGL

| Participation | Total |
| --- | --- |
| C&I Sector |  |
| Participants * | 16 |
| Installed Projects † | 24 |
| Measure Types Installed ‡ | 8 |
| Public Sector |  |
| Participants * | 15 |
| Installed Projects † | 222 |
| Measure Types Installed ‡ | 14 |
| Program 2025 Total |  |
| Participants * | 31 |
| Installed Projects † | 246 |
| Measure Types Installed ‡ | 15 |

* Participants are defined as the distinct count of Account Name.

† Installed Projects are defined as the distinct count of Work Order IDs, not including Steam Trap Test and Bonus measures.

‡ Measure Types Installed are the distinct count of verified measure names, not including Steam Trap Test and Bonus measures.

Source: Peoples Gas tracking data and evaluation team analysis.

Table 2 summarizes the PGL installed measure quantities that are the basis for verified energy savings.

Table 2. 2025 Installed Measure Quantities for PGL

| Program Category | Measure | Quantity Unit | Installed Quantity |
| --- | --- | --- | --- |
| C&I | Boiler Tune Up - Process | MBH | 32,600 |
| C&I | Boiler Tune Up - Space Heating | MBH | 42,434 |
| C&I | Prescriptive Change Steam Trap | Projects | 5 |
| C&I | Steam Pipe Insulation | Ln Ft. | 75 |
| C&I | Steam Pipe Insulation - Fitting | Ln Ft. | 3 |
| C&I | Steam Traps - HVAC Repair/Rep High | Unit | 5 |
| C&I | Steam Traps - HVAC Repair/Rep Low | Unit | 409 |
| C&I | Steam Traps - Industrial Rep | Unit | 438 |
| Public | Boiler Tune Up - Process | MBH | 90,200 |
| Public | Boiler Tune Up - Space Heating | MBH | 592,771 |
| Public | Demand Controlled Ventilation | Sq Ft | 232,148 |
| Public | DHW Pipe Insulation | Ln Ft | 410 |
| Public | High Efficiency Boiler | MBH | 21,000 |
| Public | HW Pipe Insulation | Ln Ft | 3,604 |
| Public | Linkageless Boiler Controls for Space Heating | MBH | 17,000 |
| Public | Prescriptive Change Steam Trap | Projects | 1 |
| Public | Steam Pipe Insulation | Ln Ft | 887 |
| Public | Steam Pipe Insulation - Fitting | Ln Ft | 125 |
| Public | Steam Pipe Insulation - Valve | Ln Ft | 83 |
| Public | Steam Traps - HVAC Repair/Rep Low | Unit | 1,551 |
| Public | Steam Traps - Industrial Rep | Unit | 59 |
| Public | Storage Water Heater (Large) | MBH | 390 |

Source: Peoples Gas tracking data and evaluation team analysis.

The NSG program had 12 participants (8 C&I and 4 public sector participants) in 2025 and completed 16 projects (11 C&I and 5 public sector projects) projects as shown in the following table.

Table 3. 2025 Volumetric Summary for NSG

| Participation | Total |
| --- | --- |
| C&I Sector |  |
| Participants * | 8 |
| Installed Projects † | 11 |
| Measure Types Installed ‡ | 4 |
| Public Sector |  |
| Participants * | 4 |
| Installed Projects † | 5 |
| Measure Types Installed ‡ | 4 |
| Program 2025 Total |  |
| Participants * | 12 |
| Installed Projects † | 16 |
| Measure Types Installed ‡ | 5 |

* Participants are defined as the distinct count of Account Name.

† Installed Projects are defined as the distinct count of Work Order IDs, not including Steam Trap Test and Bonus measures.

‡ Measure Types Installed are the distinct count of verified measure names, not including Steam Trap Test and Bonus measures.

Source: North Shore Gas tracking data and Guidehouse evaluation team analysis.

Table 4 summarizes the NSG installed measure quantities that are the basis for verified energy savings.

Table 4. 2025 Installed Measure Quantities for NSG

| Program Category | Measure | Quantity Unit | Installed Quantity |
| --- | --- | --- | --- |
| C&I | Boiler Tune Up - Process | MBH | 61,720 |
| C&I | Boiler Tune Up - Space Heating | MBH | 62,402 |
| C&I | Steam Traps - HVAC Repair/Rep Low | Each | 457 |
| C&I | Steam Traps - Industrial Rep | Each | 60 |
| Public | Boiler Tune Up - Process | MBH | 114,752 |
| Public | Boiler Tune Up - Space Heating | MBH | 41,400 |
| Public | Linkageless Boiler Controls for Space Heating | MBH | 5,000 |
| Public | Steam Traps - HVAC Repair/Rep Low | Each | 27 |

Source: North Shore Gas tracking data and Guidehouse evaluation team analysis.

# Program Savings Detail

Table 5 summarizes the energy savings the PGL Prescriptive Program achieved by path in 2025. Projects in disadvantaged communities (DAC) have a deemed net-to-gross ratio (NTG) of 1.00.

Table 5. 2025 Annual Energy Savings Summary for PGL

| Program Category | Ex Ante Gross Savings (Therms) | Verified Gross RR* | Verified Gross Savings (Therms) | NTG† | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- |
| C&I, Non-Disadvantaged Communities | 974,303 | 108% | 1,056,258 | 0.91 | 961,195 |
| C&I, Disadvantaged Communities | 282,808 | 115% | 325,657 | 1.00 | 325,657 |
| Public, Non-Disadvantaged Communities | 692,827 | 101% | 697,331 | 0.92 | 641,544 |
| Public, Disadvantaged Communities | 906,096 | 100% | 905,708 | 1.00 | 905,708 |
| Total or Weighted Average | 2,856,034 | 105% | 2,984,954 |  | 2,834,104 |

* Realization Rate (RR) is the ratio of verified gross savings to ex ante gross savings, based on evaluation research findings.

† The program path NTG value is calculated as Verified Net/Verified Gross. The program-level analysis typically used measure-level deemed NTG values (see Table 4), which are listed here: https://www.ilsag.info/evaluator-ntg-recommendations-for-2025.

Source: Evaluation team analysis.

Table 6 summarizes the energy savings the NSG Prescriptive Program achieved by path in 2025. Projects in disadvantaged communities (DAC) have a deemed net-to-gross ratio (NTG) of 1.00.

Table 6. 2025 Annual Energy Savings Summary for NSG

| Program Category | Ex Ante Gross Savings (Therms) | Verified Gross RR* | Verified Gross Savings (Therms) | NTG† | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- |
| C&I, Non-Disadvantaged Communities | 502,637 | 105% | 525,862 | 0.91 | 478,534 |
| C&I, Disadvantaged Communities | 182,501 | 115% | 210,237 | 1.00 | 210,237 |
| Public, Non-Disadvantaged Communities | 130,746 | 100% | 130,746 | 0.92 | 120,287 |
| Public, Disadvantaged Communities | 21,109 | 100% | 21,060 | 1.00 | 21,060 |
| Total or Weighted Average | 836,994 | 106% | 887,904 |  | 830,117 |

* Realization Rate (RR) is the ratio of verified gross savings to ex ante gross savings, based on evaluation research findings.

† The program path NTG value is calculated as Verified Net/Verified Gross. The program-level analysis typically used measure-level deemed NTG values (see Table 4), which are listed here: https://www.ilsag.info/evaluator-ntg-recommendations-for-2025.

Source: Evaluation team analysis.

# Program Savings by Measure

The PGL program includes 15 unique measures as shown in the following table. The Steam Trap and Boiler Tune Up measures contributed the most savings.

Table 7. 2025 Annual Energy Savings by Measure for PGL

| Program Category | Savings Category | Ex Ante Gross Savings (Therms) | Verified Gross RR* | Verified Gross Savings (Therms) | NTG† | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- |
| C&I, Non-Disadvantaged Communities | Boiler Tune Up - Process | 26,080 | 100% | 26,080 | 0.91 | 23,733 |
| C&I, Non-Disadvantaged Communities | Boiler Tune Up - Space Heating | 10,047 | 100% | 10,047 | 0.91 | 9,143 |
| NC&I, Non-Disadvantaged Communities | Prescriptive Change Steam Trap | 4,522 | 100% | 4,522 | 0.91 | 4,115 |
| C&I, Non-Disadvantaged Communities | Steam Pipe Insulation | 1,034 | 100% | 1,034 | 0.91 | 941 |
| C&I, Non-Disadvantaged Communities | Steam Pipe Insulation - Fitting | 48.50 | 100% | 48.50 | 0.91 | 44.14 |
| C&I, Non-Disadvantaged Communities | Steam Traps - HVAC Repair/Rep High | 1,798 | 100% | 1,802 | 0.91 | 1,640 |
| C&I, Non-Disadvantaged Communities | Steam Traps - HVAC Repair/Rep Low | 140,601 | 100% | 140,272 | 0.91 | 127,647 |
| C&I, Non-Disadvantaged Communities | Steam Traps - Industrial Rep | 790,172 | 110% | 872,452 | 0.91 | 793,932 |
| C&I, Non-Disadvantaged Communities Subtotal |  | 974,303 | 1.08 | 108% |  | 961,195 |
| C&I, Disadvantaged Communities | Prescriptive Change Steam Trap | -69,670 | 100% | -69,670 | 1.00 | -69,670 |
| C&I, Disadvantaged Communities | Steam Traps - HVAC Repair/Rep Low | 12,483 | 100% | 12,480 | 1.00 | 12,480 |
| C&I, Disadvantaged Communities | Steam Traps - Industrial Rep | 339,994 | 113% | 382,847 | 1.00 | 382,847 |
| C&I, Disadvantaged Communities Subtotal |  | 282,808 | 115% | 325,657 |  | 325,657 |
| C&I Subtotal |  | 1,257,111 | 110% | 1,381,915 |  | 1,286,852 |
| Public, Non-Disadvantaged Communities | Boiler Tune Up - Process | 16,875 | 100% | 16,875 | 0.92 | 15,525 |
| Public, Non-Disadvantaged Communities | Boiler Tune Up - Space Heating | 114,628 | 100% | 114,628 | 0.92 | 105,458 |
| Public, Non-Disadvantaged Communities | Demand Controlled Ventilation | 9,567 | 100% | 9,567 | 0.92 | 8,802 |
| Public, Non-Disadvantaged Communities | DHW Pipe Insulation | 127 | 100% | 127 | 0.92 | 117 |
| Public, Non-Disadvantaged Communities | High Efficiency Boiler | 14,373 | 100% | 14,373 | 0.92 | 13,223 |
| Public, Non-Disadvantaged Communities | HW Pipe Insulation | 15,988 | 100% | 15,988 | 0.92 | 14,709 |
| Public, Non-Disadvantaged Communities | Steam Traps - HVAC Repair/Rep Low | 451,611 | 100% | 450,549 | 0.92 | 414,505 |
| Public, Non-Disadvantaged Communities | Steam Traps - Industrial Rep | 69,657 | 108% | 75,224 | 0.92 | 69,206 |
| Public, Non-Disadvantaged Communities Subtotal |  | 692,827 | 1.01 | 101% |  | 641,544 |
| Public, Disadvantaged Communities | Boiler Tune Up - Process | 40,500 | 100% | 40,500 | 1.00 | 40,500 |
| Public, Disadvantaged Communities | Boiler Tune Up - Space Heating | 79,209 | 100% | 79,209 | 1.00 | 79,209 |
| Public, Disadvantaged Communities | Demand Controlled Ventilation | 7,148 | 100% | 7,148 | 1.00 | 7,148 |
| Public, Disadvantaged Communities | DHW Pipe Insulation | 705 | 100% | 705 | 1.00 | 705 |
| Public, Disadvantaged Communities | High Efficiency Boiler | 17,163 | 100% | 17,163 | 1.00 | 17,163 |
| Public, Disadvantaged Communities | Linkageless Boiler Controls for Space Heating | 8,353 | 100% | 8,353 | 1.00 | 8,353 |
| Public, Disadvantaged Communities | Prescriptive Change Steam Trap | 394 | 100% | 394 | 1.00 | 394 |
| Public, Disadvantaged Communities | Steam Pipe Insulation | 8,370 | 100% | 8,370 | 1.00 | 8,370 |
| Public, Disadvantaged Communities | Steam Pipe Insulation - Fitting | 997 | 100% | 997 | 1.00 | 997 |
| Public, Disadvantaged Communities | Steam Pipe Insulation - Valve | 2,637 | 100% | 2,637 | 1.00 | 2,637 |
| Public, Disadvantaged Communities | Steam Traps - HVAC Repair/Rep Low | 698,735 | 100% | 697,140 | 1.00 | 697,140 |
| Public, Disadvantaged Communities | Steam Traps - Industrial Rep | 41,614 | 103% | 42,822 | 1.00 | 42,822 |
| Public, Disadvantaged Communities | Storage Water Heater (Large) | 271 | 100% | 271 | 1.00 | 271 |
| Public, Disadvantaged Communities Subtotal |  | 906,096 | 100% | 905,708 |  | 905,708 |
| Public Subtotal |  | 1,598,922 | 100% | 1,603,039 |  | 1,547,253 |
| Total or Weighted Average |  | 2,856,034 | 105% | 2,984,954 |  | 2,834,104 |

* Realization Rate (RR) is the ratio of verified gross savings to ex ante gross savings, based on evaluation research findings.

† NTG, Net to Gross is the deemed value available on the SAG website: https://www.ilsag.info/evaluator-ntg-recommendations-for-2025.

Source: Evaluation team analysis.

The NSG program includes five unique measures as shown in the following table. The Steam Trap measures contributed the most savings.

Table 8. 2025 Annual Energy Savings by Measure for NSG

| Program Category | Savings Category | Ex Ante Gross Savings (Therms) | Verified Gross RR* | Verified Gross Savings (Therms) | NTG† | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- |
| C&I, Non-Disadvantaged Communities | Boiler Tune Up - Process | 63,123 | 100% | 63,123 | 0.91 | 57,442 |
| C&I, Non-Disadvantaged Communities | Boiler Tune Up - Space Heating | 29,550 | 100% | 29,550 | 0.91 | 26,891 |
| C&I, Non-Disadvantaged Communities | Steam Traps - HVAC Repair/Rep Low | 292,610 | 99% | 290,439 | 0.91 | 264,300 |
| C&I, Non-Disadvantaged Communities | Steam Traps - Industrial Rep | 117,354 | 122% | 142,749 | 0.91 | 129,902 |
| C&I, Non-Disadvantaged Communities Subtotal |  | 502,637 | 105% | 525,862 |  | 478,534 |
| C&I, Disadvantaged Communities | Steam Traps - HVAC Repair/Rep Low | 17,982 | 100% | 17,940 | 1.00 | 17,940 |
| C&I, Disadvantaged Communities | Steam Traps - Industrial Rep | 164,519 | 117% | 192,297 | 1.00 | 192,297 |
| C&I, Disadvantaged Communities Subtotal |  | 182,501 | 1.15 | 115% |  | 210,237 |
| C&I Subtotal |  | 685,139 | 1.07 | 107% |  | 688,771 |
| Public, Non-Disadvantaged Communities | Boiler Tune Up - Process | 117,361 | 100% | 117,361 | 0.92 | 107,972 |
| Public, Non-Disadvantaged Communities | Boiler Tune Up - Space Heating | 10,100 | 100% | 10,100 | 0.92 | 9,292 |
| Public, Non-Disadvantaged Communities | Linkageless Boiler Controls for Space Heating | 3,285 | 100% | 3,285 | 0.92 | 3,022 |
| Public, Non-Disadvantaged Communities Subtotal |  | 130,746 | 100% | 130,746 |  | 120,287 |
| Public, Disadvantaged Communities | Steam Traps - HVAC Repair/Rep Low | 21,109 | 100% | 21,060 | 1.00 | 21,060 |
| Public, Disadvantaged Communities Subtotal |  | 21,109 | 100% | 21,060 |  | 21,060 |
| Public Subtotal |  | 151,856 | 100% | 151,806 |  | 141,346 |
| Total or Weighted Average |  | 836,994 | 106% | 887,904 |  | 830,117 |

* Realization Rate (RR) is the ratio of verified gross savings to ex ante gross savings, based on evaluation research findings.

† NTG, Net to Gross is the deemed value available on the SAG website: https://www.ilsag.info/evaluator-ntg-recommendations-for-2025.

Source: Evaluation team analysis.

# Impact Analysis Findings and Recommendations

## Impact Parameter Estimates

Table 9 shows the unit therm savings and realization rate findings by measure from our review. The realization rate is the ratio of the verified savings to the ex ante savings. Following the table, we provide findings and recommendations, including discussion of all measures with realization rates above or below 100%. Appendix A provides a description of the impact analysis methodology.

Table 9. Verified Gross Savings Parameters

| Measure | Unit Basis | Ex Ante Gross (therms/unit) | Verified Gross (therms/unit) | Realization Rate | Data Source and TRM v13.0 Section † |
| --- | --- | --- | --- | --- | --- |
| Boiler Tune Up - Process | MBH | Varies | Varies | 100% | Section 4.4.3 |
| Boiler Tune Up - Space Heating | MBH | Varies | Varies | 100% | Section 4.4.2 |
| Demand Controlled Ventilation | Sq Ft | 0.072 | 0.072 | 100% | Section 4.4.19 |
| DHW Pipe Insulation | Ln Ft. | Varies | Varies | 100% | Section 4.4.14 |
| High Efficiency Boiler | MBH | Varies | Varies | 100% | Section 4.4.10 |
| HW Pipe Insulation | Ln Ft. | Varies | Varies | 100% | Section 4.4.14 |
| Linkageless Boiler Controls for Space Heating | MBH | Varies | Varies | 100% | Section 4.4.21 |
| Prescriptive Change Steam Trap | Projects | Custom Calculation | Custom Calculation | 100% | PTD*, MMDB |
| Steam Pipe Insulation | Ln Ft. | Varies | Varies | 100% | PTD, MMDB |
| Steam Pipe Insulation - Fitting | Ln Ft. | Varies | Varies | 100% | PTD, MMDB |
| Steam Pipe Insulation - Valve | Ln Ft. | Varies | Varies | 100% | PTD, MMDB |
| Steam Traps - HVAC Repair/Rep High | Each | Varies | Varies | 100% | Section 4.4.16, PTD, MMDB |
| Steam Traps - HVAC Repair/Rep Low | Each | Varies | Varies | 100% | Section 4.4.16, PTD, MMDB |
| Steam Traps - Industrial Rep | Each | Varies | Varies | 112% | Section 4.4.16, PTD, MMDB |
| Storage Water Heater (Large) | MBH | Varies | Varies | 100% | Section 4.3.1 |

* Program Tracking Data (PTD) provided by PGL NSG, extract dated January 30, 2026; PGL/NSG Master Measure Database (MMDB).

† State of Illinois Technical Reference Manual version 13.0 from http://www.ilsag.info/technical-reference-manual.html.

## Findings and Recommendations

Finding 1a. The evaluation team used the supplemental data file to obtain Steam Pressure PSIG input information for all Steam Trap measures. For measures that had a provided steam pressure value, verified savings used the value to calculate the steam loss per leaking trap (Sa) and temperature of saturated steam (T1), via the calculated average steam trap inlet pressure (P1), inputs. For measures that did not have a provided steam pressure value, verified savings used the Illinois Technical Reference Manual version 13.0 (IL TRM v13.0) to determine the Sa and T1, via the P1 calculation, values based on measure information. All other inputs came from the IL TRM v13.0 for both cases.

The two “HVAC Repair/Rep” measures with steam pressure values of 15 or above were assigned to the High category, both had steam pressure values provided. All other “HVAC Repair/Rep” measures were assigned to the Low category as they either had a steam pressure value of 7 or not provided. The evaluation team assumed a historic program evaluation default steam pressure value of 11.2, documented in the Master Measure Database (MMBD) and aligning with the IL TRM v13.0 “Industrial/Process Low Pressure: psig < 15” steam system, for those measures in the Low category without a steam pressure value provided.

All “Industrial/Process” measures either provided a steam pressure value or had details on steam pressure in the Measure Name tracking data column except for one instance. This was the “Industrial/Process” measure in WO-7130186, and the evaluation team assumed a historic program evaluation default steam pressure value of 16, aligning with the IL TRM v13.0 “Medium Pressure: 15 ≤ psig < 30” steam system.

Finding 1b. The evaluation team found two measures with ex ante savings that did not utilize the provided steam pressure values, WO-4636585 “HVAC Repair/Rep High – Audit” and WO-6032521 “Industrial/Process Audit - 15 <= psig < 30”. Verified savings followed suit, assuming a historic program evaluation default steam pressure of 16, aligning with the IL TRM v13.0 “Medium Pressure: 15 ≤ psig < 30” steam system, and using IL TRM v13.0 values to get a realization rate of 1.00.

Finding 1c. The evaluation team could not replicate ex ante savings for two measures. Verified savings for WO-7122677 “Industrial/Process Audit” used the provided steam pressure of 100, this resulted in a realization rate of 1.05. Verified savings for WO-7130186 “Industrial/Process Audit” used a default steam pressure of 16 as no steam pressure value was provided, this resulted in a realization rate of 0.84.

Recommendation 1. Ensure the tracking data provides all relevant steam trap information for measure savings verification. Tracking data information should support ex ante savings values.

Finding 2. The evaluation team observed ex ante savings used a formula to calculate the heat of vaporization of steam (Hv) input for all measures where a steam pressure value was provided, except for the three cases summarized in Findings 1b and 1c (WO-4636585, WO-6032521, and WO-7122677). The Interim Impact Review found this formula came from the MMBD, and in follow-up discussions Guidehouse stated verified savings will continue to use the IL TRM v13.0 to look up Hv values. This had varied impacts on measure savings realization rates.

Recommendation 2. The evaluation team recommends using the IL TRM v13.0 for Hv values to align with the methodology for other savings inputs. The TRM does not state that actual values can be used for this input.

Finding 3. The evaluation team used the supplemental data file “Building Type-Corrected” field as the project’s Building Type. When that field was blank, the evaluation team used the tracking data’s “Business Type” field for the project’s Building Type. When both fields were blank, the evaluation team assumed a Building Type of Unknown. This did not impact realization rates.

Recommendation 3. Ensure key columns of the tracking data, such as the project’s Building Type, are populated completely.

Finding 4. The evaluation team found the PGL program had Prescriptive Change Steam Trap measure category with one measure line item in tracking data reporting negative savings of 69,670 therms. We could not replicate the assumptions that produced these negative savings. We did not change the verified savings.

Recommendation 4. Produce adequate data for the Prescriptive Change Steam Trap measure to justify the savings claimed in the tracking data.

# Appendix A. Impact Analysis Methodology

Guidehouse determined verified gross savings for each program measure by:

Utilizing the provided tracking data: “Public Px 2025 Data for GH.xlsx” and “C&I Px 2025 Data for GH.xlsx”.

Utilizing a provided supplemental file to identify measures in a DAC designated site, project Building Type, and Steam Trap steam pressure: “EOY Data File for Business Programs 2025_DAC_Update Bldg Type.xlsx”.

Reviewing the savings algorithm inputs in the MMDB for agreement with the IL TRM v13.0 and IL TRM v13.0 errata memo or evaluation research for non-deemed measures or custom inputs.

Validating that the savings algorithm was applied correctly.

Cross-checking per-unit savings values in the tracking data with the verified values in the MMDB or Guidehouse’s calculations if the MMDB did not agree with the IL TRM v13.0 or IL TRM v13.0 errata memo.

Multiplying the verified per-unit savings value by the quantity reported in the program tracking data to substantiate the type and quantity of measures installed.

Verified gross realization rates are calculated by dividing the verified gross savings by the ex ante gross savings. We calculated verified net energy savings by multiplying the verified gross savings estimates by a deemed NTG. The 2025 NTG were based on past evaluation research and defined by a consensus process through SAG. Economically disadvantaged areas were identified by the “Dis-Advantaged Community” column of the supplemental data file, where a “0” represented a Non-Disadvantaged Community and a “1” represented a Disadvantaged Community. The evaluation team used a NTG of 1.00 for DAC projects’ verified savings when the “Dis-Advantaged Community” column had a value of “1”. The NTG of 1.00 is based on Illinois Policy Manual 3.0.

# Appendix B. Program Specific Inputs for the Illinois TRC

The table below shows the Total Resource Cost (TRC) cost-effectiveness analysis inputs available at the time of producing this impact evaluation report. Additional required cost data (e.g., measure costs, program level incentive and non-incentive costs) are not included in this table and will be provided to the evaluation team later. Guidehouse will include annual and lifetime water savings and greenhouse gas reductions in the end of year summary report.

Table B-1. Verified Cost Effectiveness Inputs - PGL

| Program Category | Savings Category | DAC Project | Units | Quantity | Effective Useful Life | Early Replacement Flag† | Verified Gross Annual Water Savings (Gallons) | Ex Ante Gross Savings (Therms) | Verified Gross Savings (Therms) | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C&I | Boiler Tune Up - Process | FALSE | MBH | 32,600 | 2.00 | NO |  | 26,080 | 26,080 | 23,733 |
| C&I | Boiler Tune Up - Space Heating | FALSE | MBH | 42,434 | 3.00 | NO |  | 10,047 | 10,047 | 9,143 |
| C&I | Prescriptive Change Steam Trap | FALSE | Projects | 2.00 | 6.00 | NO |  | 4,522 | 4,522 | 4,115 |
| C&I | Prescriptive Change Steam Trap | TRUE | Projects | 3.00 | 6.00 | NO |  | -69,670 | -69,670 | -69,670 |
| C&I | Steam Pipe Insulation | FALSE | Ln Ft. | 75.00 | 15.00 | NO |  | 1,034 | 1,034 | 941 |
| C&I | Steam Pipe Insulation - Fitting | FALSE | Ln Ft. | 3.00 | 15.00 | NO |  | 48.50 | 48.50 | 44.14 |
| C&I | Steam Traps - HVAC Repair/Rep High | FALSE | Each | 5.00 | 6.00 | NO | 15,713 | 1,798 | 1,802 | 1,640 |
| C&I | Steam Traps - HVAC Repair/Rep Low | FALSE | Each | 393 | 6.00 | NO | 1,202,795 | 140,601 | 140,272 | 127,647 |
| C&I | Steam Traps - HVAC Repair/Rep Low | TRUE | Each | 16.00 | 6.00 | NO | 107,010 | 12,483 | 12,480 | 12,480 |
| C&I | Steam Traps - Industrial Rep | FALSE | Each | 293 | 6.00 | NO | 7,116,805 | 790,172 | 872,452 | 793,932 |
| C&I | Steam Traps - Industrial Rep | TRUE | Each | 145 | 6.00 | NO | 3,484,569 | 339,994 | 382,847 | 382,847 |
| Public | Boiler Tune Up - Process | FALSE | MBH | 25,500 | 2.00 | NO |  | 16,875 | 16,875 | 15,525 |
| Public | Boiler Tune Up - Process | TRUE | MBH | 64,700 | 2.00 | NO |  | 40,500 | 40,500 | 40,500 |
| Public | Boiler Tune Up - Space Heating | FALSE | MBH | 348,464 | 3.00 | NO |  | 114,628 | 114,628 | 105,458 |
| Public | Boiler Tune Up - Space Heating | TRUE | MBH | 244,307 | 3.00 | NO |  | 79,209 | 79,209 | 79,209 |
| Public | Demand Controlled Ventilation | FALSE | Sq Ft | 132,873 | 10.00 | NO |  | 9,567 | 9,567 | 8,802 |
| Public | Demand Controlled Ventilation | TRUE | Sq Ft | 99,275 | 10.00 | NO |  | 7,148 | 7,148 | 7,148 |
| Public | DHW Pipe Insulation | FALSE | Ln Ft. | 35.00 | 15.00 | NO |  | 127 | 127 | 117 |
| Public | DHW Pipe Insulation | TRUE | Ln Ft. | 375 | 15.00 | NO |  | 705 | 705 | 705 |
| Public | High Efficiency Boiler | FALSE | MBH | 10,000 | 25.00 | NO |  | 14,373 | 14,373 | 13,223 |
| Public | High Efficiency Boiler | TRUE | MBH | 11,000 | 25.00 | NO |  | 17,163 | 17,163 | 17,163 |
| Public | HW Pipe Insulation | FALSE | Ln Ft. | 3,604 | 15.00 | NO |  | 15,988 | 15,988 | 14,709 |
| Public | Linkageless Boiler Controls for Space Heating | TRUE | MBH | 17,000 | 20.00 | NO |  | 8,353 | 8,353 | 8,353 |
| Public | Prescriptive Change Steam Trap | TRUE | Projects | 1.00 | 6.00 | NO |  | 394 | 394 | 394 |
| Public | Steam Pipe Insulation | TRUE | Ln Ft. | 887 | 15.00 | NO |  | 8,370 | 8,370 | 8,370 |
| Public | Steam Pipe Insulation - Fitting | TRUE | Ln Ft. | 125 | 15.00 | NO |  | 997 | 997 | 997 |
| Public | Steam Pipe Insulation - Valve | TRUE | Ln Ft. | 83.00 | 15.00 | NO |  | 2,637 | 2,637 | 2,637 |
| Public | Steam Traps - HVAC Repair/Rep Low | FALSE | Each | 601 | 6.00 | NO | 3,863,337 | 451,611 | 450,549 | 414,505 |
| Public | Steam Traps - HVAC Repair/Rep Low | TRUE | Each | 950 | 6.00 | NO | 5,977,793 | 698,735 | 697,140 | 697,140 |
| Public | Steam Traps - Industrial Rep | FALSE | Each | 36.00 | 6.00 | NO | 651,379 | 69,657 | 75,224 | 69,206 |
| Public | Steam Traps - Industrial Rep | TRUE | Each | 23.00 | 6.00 | NO | 357,037 | 41,614 | 42,822 | 42,822 |
| Public | Storage Water Heater (Large) | TRUE | MBH | 390 | 15.00 | NO |  | 271 | 271 | 271 |
| Total or Weighted Average |  |  |  |  |  |  | 22,776,439 | 2,856,034 | 2,984,954 | 2,834,104 |

Source: Evaluation team analysis.

Table B-2. Verified Cost Effectiveness Inputs – NSG

| Program Category | Savings Category | DAC Project | Units | Quantity | Effective Useful Life | Early Replacement Flag† | Verified Gross Annual Water Savings (Gallons) | Ex Ante Gross Savings (Therms) | Verified Gross Savings (Therms) | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C&I | Boiler Tune Up - Process | FALSE | MBH | 61,720 | 2.00 | NO |  | 63,123 | 63,123 | 57,442 |
| C&I | Boiler Tune Up - Space Heating | FALSE | MBH | 62,402 | 3.00 | NO |  | 29,550 | 29,550 | 26,891 |
| C&I | Steam Traps - HVAC Repair/Rep Low | FALSE | Each | 434 | 6.00 | NO | 2,902,653 | 292,610 | 290,439 | 264,300 |
| C&I | Steam Traps - HVAC Repair/Rep Low | TRUE | Each | 23.00 | 6.00 | NO | 153,827 | 17,982 | 17,940 | 17,940 |
| C&I | Steam Traps - Industrial Rep | FALSE | Each | 20.00 | 6.00 | NO | 1,050,572 | 117,354 | 142,749 | 129,902 |
| C&I | Steam Traps - Industrial Rep | TRUE | Each | 40.00 | 6.00 | NO | 1,724,147 | 164,519 | 192,297 | 192,297 |
| Public | Boiler Tune Up - Process | FALSE | MBH | 114,752 | 2.00 | NO |  | 117,361 | 117,361 | 107,972 |
| Public | Boiler Tune Up - Space Heating | FALSE | MBH | 41,400 | 3.00 | NO |  | 10,100 | 10,100 | 9,292 |
| Public | Linkageless Boiler Controls for Space Heating | FALSE | MBH | 5,000 | 20.00 | NO |  | 3,285 | 3,285 | 3,022 |
| Public | Steam Traps - HVAC Repair/Rep Low | TRUE | Each | 27.00 | 6.00 | NO | 180,580 | 21,109 | 21,060 | 21,060 |
| Total or Weighted Average |  |  |  |  |  |  | 6,011,778 | 836,994 | 887,904 | 830,117 |

Source: Evaluation team analysis

EOY Data File for Business Programs 2025_DAC_Update Bldg Type.xlsx.

EOY Data File for Business Programs 2025_DAC_Update Bldg Type.xlsx.

A deemed value. Available on the SAG website: https://www.ilsag.info/policy/net-to-gross-framework/evaluator-ntg-recommendations-for-2025/

EOY Data File for Business Programs 2025_DAC_Update Bldg Type.xlsx.

https://www.ilsag.info/wp-content/uploads/IL_EE_Policy_Manual_Version_3.0_Final_11-3-2023.pdf