# Business & Public Custom Program Impact Evaluation Report

Energy Efficiency Plan: Program Year 2025

(01/01/2025-12/31/2025)

Prepared for:

Peoples Gas and North Shore Gas

DRAFT

April 20, 2026

Prepared by:

| Eric Ogbe Guidehouse | Roger Hill InCA | Deborah Swarts InCA |  |
| --- | --- | --- | --- |

Submitted to:

Peoples Gas

North Shore Gas

200 East Randolph Street

Chicago, IL 60601

Submitted by:

Guidehouse Inc.

167 N. Green Street

12th Floor

Chicago, IL 60607

Contact:

| Charles Maglione, Partner 202.481.7352 cmaglione@guidehouse.com | Jeff Erickson, Director 608.616.4962 jeff.erickson@guidehouse.com | Laura Agapay-Read, Associate Director 312.583.4178 laura.agapay.read@guidehouse.com |
| --- | --- | --- |

Charles Ampong, Associate Director

608.616.4922

charles.ampong@guidehouse.com

This report was prepared by Guidehouse for People Gas Light and Coke Company (PGL)and North Shore Gas(NSG) companies. The work presented in this report represents Guidehouse’s professional judgment based on the information available at the time this report was prepared. Use of this report by any other party for whatever purpose should not, and does not, absolve such party from using due diligence in verifying the report’s contents. Neither Guidehouse nor any of its subsidiaries or affiliates assumes any liability or duty of care to such parties and hereby disclaims any such liability.

Table of Contents

1	Introduction	1

2	Program Description	1

3	Program Savings Detail	2

4	Program Savings by Measure	2

5	Impact Analysis Findings and Recommendations	3

5.1	Impact Parameter Estimates	3

5.2	Findings and Recommendations	4

Appendix A. Impact Analysis Methodology	5

Appendix B. Program Specific Inputs for the Illinois TRC	1

Table of Tables, Figures, and Equations

Table 1. 2025 Volumetric Summary for PGL	1

Table 2. 2025 Volumetric Summary for NSG	1

Table 3. 2025 Annual Energy Savings Summary for PGL	2

Table 4. 2025 Annual Energy Savings Summary for NSG	2

Table 5. 2025 Annual Energy Savings by Measure for PGL	3

Table 6. 2025 Annual Energy Savings by Measure for NSG	3

Table 7. Verified Gross Savings Parameters	4

Table 8. Profile of Gross Impact Sample for Custom Projects	6

Table 9. Gross Realization Rates and Relative Precision at 90% Confidence Level	6

Table 10. Sample Gross Therms and Verification Approach	7

Table 11. Sample Gross Realization Rates and Summary of Adjustments	8

Table 12. Verified Cost Effectiveness Inputs - PGL	9

Table 13. Verified Cost Effectiveness Inputs – NSG	10

# Introduction

This report presents the results of the impact evaluation of the Peoples Gas (PGL) and North Shore Gas (NSG) 2025 Custom program. It presents a summary of the energy impacts for the total program and broken out by relevant measure and program structure details. The appendix presents the impact analysis methodology and inputs for cost effectiveness analysis. Program year 2025 covers January 1 to December 31, 2025.

# Program Description

The Custom program provides PGL and NSG private sector commercial and industrial (C&I) and public sector (PS) customers with rebates on a custom basis; these are applications for measures not covered under the Prescriptive Rebate path. Typical market sectors for this program include larger customers in light and heavy manufacturing, hospitals, hotels, public sector facilities, and other process heating intensive businesses.

Custom rebates are on a dollar per therm basis, subject to payback and project cost limitations. PGL and NSG may revise eligible measures and incentives as driven by current market conditions, changes to codes and standards, technology, evaluation results, and program management knowledge.

The PGL Custom program had 10 participants in 2025 and completed 17 projects, as shown in Table 1.

Table 1. 2025 Volumetric Summary for PGL

| Participation | Private | Public | Total |
| --- | --- | --- | --- |
| Participants * | 8 | 2 | 10 |
| Installed Projects † | 10 | 7 | 17 |
| Measure Types Installed ‡ | 5 | 3 | 6 |

* Participants are defined as unique work order IDs

† Installed Projects are defined as unique retrofit for each participant

‡ Total Measure Types Installed is less than the sum of private and public due to the same Measure Types in different building sectors.

Source: Peoples Gas tracking data and evaluation team analysis.

The NSG Custom program had 6 participants in 2025 and completed 7 projects, as shown in Table 2.

Table 2. 2025 Volumetric Summary for NSG

| Participation | Private | Public | Total |
| --- | --- | --- | --- |
| Participants * | 5 | 1 | 6 |
| Installed Projects † | 6 | 1 | 7 |
| Measure Types Installed ‡ | 3 | 1 | 3 |

* Participants are defined as unique work order IDs

† Installed Projects are defined as unique retrofit for each participant

‡ Total Measure Types Installed is less than the sum of private and public due to the same Measure Types in different building sectors.

Source: North Shore Gas tracking data and Guidehouse evaluation team analysis.

# Program Savings Detail

Table 3 summarizes the energy savings the PGL Custom Program achieved in 2025, indicating which portion of the savings were realized from disadvantaged communities (DAC).

Table 3. 2025 Annual Energy Savings Summary for PGL

| Program Category | Ex Ante Gross Savings (Therms) | Verified Gross RR* | Verified Gross Savings (Therms) | NTG† | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- |
| Private, Non-DAC | 129,920 | 109% | 141,027 | 0.93 | 131,155 |
| Public, Non-DAC | 13,594 | 110% | 15,004 | 0.92 | 13,804 |
| Public, DAC | 30,108 | 110% | 33,200 | 1.00 | 33,200 |
| Total | 173,622 | 109% | 189,231 |  | 178,159 |

* Realization Rate (RR) is the ratio of verified gross savings to ex ante gross savings, based on evaluation research findings.

† NTG (Net to Gross) is the deemed value available on the SAG website: https://www.ilsag.info/evaluator-ntg-recommendations-for-2025/.

Source: Evaluation team analysis.

Table 4 summarizes the energy savings the NSG Custom Program achieved in 2025.

Table 4. 2025 Annual Energy Savings Summary for NSG

| Program Category | Ex Ante Gross Savings (Therms) | Verified Gross RR* | Verified Gross Savings (Therms) | NTG† | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- |
| Private, Non-DAC | 260,370 | 103% | 268,885 | 0.93 | 250,063 |
| Public, Non-DAC | 6,358 | 111% | 7,030 | 0.92 | 6,468 |
| Total | 266,728 | 103% | 275,915 |  | 256,531 |

* Realization Rate (RR) is the ratio of verified gross savings to ex ante gross savings, based on evaluation research findings.

† NTG (Net to Gross) is the deemed value available on the SAG website: https://www.ilsag.info/evaluator-ntg-recommendations-for-2025/.

Source: Evaluation team analysis.

# Program Savings by Measure

Table 5 and Table 6 present the custom measures processed through the Custom program in PY2025. The realization rate (RR) is the ratio of verified gross savings to ex ante gross savings, based on evaluation research findings for a sample of the Custom program projects. Realization rate findings for individual sampled projects are provided in the Appendix.

Table 5. 2025 Annual Energy Savings by Measure for PGL

| Program Category | Savings Category | Ex Ante Gross Savings (Therms) | Verified Gross RR* | Verified Gross Savings (Therms) | NTG† | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- |
| Private, Non-DAC | Boiler Condensate Recovery | 43,593 | 105% | 45,873 | 0.93 | 42,661 |
|  | Burner Replacement | 25,318 | 111% | 27,996 | 0.93 | 26,036 |
|  | Insulation | 51,934 | 110% | 57,238 | 0.93 | 52,969 |
|  | Space Conditioning Controls | 6,755 | 111% | 7,469 | 0.93 | 6,946 |
|  | Other | 2,321 | 106% | 2,452 | 0.93 | 2,280 |
| Private, Non-DAC Subtotal |  | 129,920 | 109% | 141,027 | 0.93 | 131,155 |
| Public, Non-DAC | Space Conditioning Controls | 13,594 | 110% | 15,004 | 0.92 | 13,804 |
| Public, Non-DAC Subtotal |  | 13,594 | 110% | 15,004 | 0.92 | 13,804 |
| Public - DAC | High Speed Door | 21,445 | 111% | 23,713 | 1.00 | 23,713 |
|  | Space Conditioning Controls | 6,806 | 111% | 7,525 | 1.00 | 7,525 |
|  | Insulation | 1,857 | 106% | 1,962 | 1.00 | 1,962 |
| Public, DAC Subtotal |  | 30,108 | 110% | 33,200 | 1.00 | 33,200 |
| Total |  | 173,622 | 109% | 189,231 | 0.94 | 178,159 |

* Realization Rate (RR) is the ratio of verified gross savings to ex ante gross savings, based on evaluation research findings.

† NTG (Net to Gross) is the deemed value available on the SAG website: https://www.ilsag.info/evaluator-ntg-recommendations-for-2025/.

Source: Evaluation team analysis.

Table 6. 2025 Annual Energy Savings by Measure for NSG

| Program Category | Savings Category | Ex Ante Gross Savings (Therms) | Verified Gross RR* | Verified Gross Savings (Therms) | NTG† | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- |
| Private, Non-DAC | Steam Chest Leak Replacement | 106,193 | 100% | 106,016 | 0.93 | 98,595 |
|  | Waste Heat Recovery | 73,016 | 99% | 76,835 | 0.93 | 71,457 |
|  | Insulation | 81,162 | 101% | 86,034 | 0.93 | 80,012 |
| Private, Non-DAC Subtotal |  | 260,370 | 100% | 268,885 | 0.80 | 250,063 |
| Public, Non-DAC | Insulation | 6,357 | 111% | 7,030 | 0.92 | 6,467 |
| Public, Non-DAC Subtotal |  | 6,357 | 111% | 7,030 | 0.92 | 6,467 |
| Total |  | 266,728 | 103% | 275,915 |  | 256,531 |

* Realization Rate (RR) is the ratio of verified gross savings to ex ante gross savings, based on evaluation research findings.

† NTG (Net to Gross) is the deemed value available on the SAG website: https://www.ilsag.info/evaluator-ntg-recommendations-for-2025/.

Source: Evaluation team analysis.

# Impact Analysis Findings and Recommendations

## Impact Parameter Estimates

Table 7 shows the unit therm savings and realization rate findings by measure from our review. The realization rate is the ratio of the verified savings to the ex ante savings. Following the table, we provide findings and recommendations, including discussion of all measures with realization rates above or below 100%. Appendix A provides a description of the impact analysis methodology.

Table 7. Verified Gross Savings Parameters

| Measure | Unit Basis | Ex Ante Gross (therms/unit) | Verified Gross (therms/unit) | Realization Rate | Data Source(s) |
| --- | --- | --- | --- | --- | --- |
| PGL Custom | Project | Varies | Varies | 109% | Illinois Technical Reference Manual (IL-TRM) v13.0†, Project Files, Utility Data |
| NSG Custom | Project | Varies | Varies | 103% | Illinois Technical Reference Manual (IL-TRM) v13.0†, Project Files, Utility Data |

† State of Illinois Technical Reference Manual version 13.0 from http://www.ilsag.info/technical-reference-manual.html.

‡ Project files and monthly billing data provided by PGL NSG. Where conducted, on-site or telephone interview data collected by Guidehouse.

## Findings and Recommendations

General findings and recommendations are presented below. The detailed realization rates and evaluation finding for individual sampled projects are provided in Appendix A.

Finding 1. For WO-5984069, a Waste Heat Recovery project, the evaluation team found that TMY3 weather data was used to estimate hours of operation instead of the required TMYx dataset in accordance with Illinois TRM guideline. Updating the analysis to use the TMYx dataset resulted in a Realization Rate (RR) of 144%.

Recommendation 1.  Follow the Illinois TRM guidelines and use TMYx weather data for savings calculations.

Finding 2. For WO-4297269, a Boiler Condensate Recovery project, the evaluation team found that the ex ante savings in the tracking data were entered from a wrong database as the final ex ante savings. Ex ante estimate is the pre-approval estimate of 43,593 therms instead of the final approval of 39,513 therms.

Finding 3. For WO-5921741, a Space Conditioning Controls project, the evaluation team could not find how the tracking data ex ante savings were calculated in the project documentations. The evaluation team performed a regression model using additional billing data for the site, excluding summer months with minimal gas usage. The regression results for both baseline and ex post were normalized with TMYx 2007-2021 dataset. The analysis resulted in a RR of 188%.

Finding 4. For WO-6622742, a Process Insulation project, the evaluation team found that the boiler efficiency used was incorrect.

Recommendation 2.  The evaluation team recommend strengthening data quality control procedures to ensure savings calculation inputs and final savings submissions are properly verified.

# Appendix A. Impact Analysis Methodology

The evaluation sample design used a stratified random sample stratified by gross therm savings. The strata were chosen to include approximately one third of the savings in the top three strata. The fourth strata was for small projects. Fifteen out of the population of 24 projects were selected for analysis based on this design. Table 8 shows a profile of the sample selection.

Table 8. Profile of Gross Impact Sample for Custom Projects

|  | Population Summary |  |  |  | Sample Summary |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Program | Sampling Strata | Number of Projects (N) | Ex Ante Gross Savings | Percent of Ex Ante Gross | N | Ex Ante Gross Savings | Sampled % of Population |
|  |  |  | (Therms) |  |  | (Therms) | (% Therms) |
|  | 1 | 1 | 106,193 | 24% | 1 | 106,193 | 100% |
| PGL NSG Custom | 2 | 5 | 186,039 | 42% | 5 | 186,039 | 100% |
|  | 3 | 12 | 139,552 | 32% | 9 | 115,124 | 82% |
|  | 4 | 6 | 8,567 | 2% | 0 | 0 | 0% |
| TOTAL |  | 24 | 440,350 | 100% | 15 | 407,355 | 93% |

Source: Evaluation team analysis.

Engineering Review of Project Files

For each selected project, the evaluation did an in-depth application review to assess the engineering methods, parameters and assumptions used to generate all ex ante impact estimates. For each measure in the sampled project, evaluation engineers estimated ex post gross savings based on their review of documentation and engineering analysis.

To support this review, the evaluation reviewed project documentation in electronic format for each sampled project. Documentation included some or all scanned files of hardcopy application forms and supporting documentation from the applicant (invoices, measure specification sheets, and vendor proposals), pre-inspection reports and photos, post inspection reports and photos, and calculation spreadsheets.

Table 9 shows the sample and all the roll up gross realization rates and the mean precision estimates at a 90% confidence level.

Table 9. Gross Realization Rates and Relative Precision at 90% Confidence Level

| Program Sector | Sampling Strata |  | Relative Precision | Mean RR | Standard Error |
| --- | --- | --- | --- | --- | --- |
|  |  |  | + or - % |  |  |
| PGL NSG Business Custom | 1 |  | 0% | 100% | 0.00 |
|  | 2 |  | 0% | 105% | 0.00 |
|  | 3 |  | 6% | 111% | 0.03 |
|  | 4 |  | NA | NA | NA |
| Custom Total RR (90/10) |  |  | 9% | 106% | 0.03 |

Source: Evaluation team analysis.

Table 10 provides a summary of the Business Custom Program sample selection and verification.

Table 10. Sample Gross Therms and Verification Approach

| Project ID | Ex Ante Gross Savings (Therms) | Strata | Verification Approach | Measure |
| --- | --- | --- | --- | --- |
| WO-6622742 | 3,433 | 3 | File Review | Insulation |
| WO-7200348 | 6,755 | 3 | File Review | Space Conditioning Controls |
| WO-4297224 | 6,790 | 3 | File Review | Space Conditioning Controls |
| WO-6781000 | 21,445 | 3 | File Review | High Speed Door |
| WO-5984069 | 31,104 | 2 | File Review | Waste Heat Recovery |
| WO-6149553 | 41,912 | 2 | File Review | Waste Heat Recovery |
| WO-4297269 | 43,593 | 2 | File Review | Boiler Condensate Recovery |
| WO-6232126 | 23,164 | 3 | File Review | Insulation |
| WO-6014800 | 36,832 | 2 | File Review | Insulation |
| WO-7999106 | 32,598 | 2 | File Review | Insulation |
| WO-6961435 | 15,177 | 3 | File Review | Insulation |
| WO-4911636 | 2,773 | 3 | File Review | Space Conditioning Controls |
| WO-5887462 | 106,193 | 1 | File Review | Steam Chest Leak Replacement |
| WO-4514894 | 25,318 | 3 | File Review | Burner Replacement |
| WO-5921741 | 10,269 | 3 | File Review | Space Conditioning Controls |

Source: Evaluation team analysis.

Table 11 provides a summary of verification results and adjustments for the PGL and NSG sampled Custom projects.

Table 11. Sample Gross Realization Rates and Summary of Adjustments

| Project ID | Measure | Program | Strata | Realization Rate | Comments |
| --- | --- | --- | --- | --- | --- |
| WO-6622742 | Insulation | PG C&I Custom | 3 | 102% | Boiler efficiency discrepancy |
| WO-7200348 | Space Conditioning Controls | PG C&I Custom | 3 | 100% |  |
| WO-4297224 | Space Conditioning Controls | PG Public Custom | 3 | 100% |  |
| WO-6781000 | High Speed Door | PG Public Custom | 3 | 100% |  |
| WO-5984069 | Waste Heat Recovery | NSG C&I Custom | 2 | 144% | TMY3 dataset updated to TMYx |
| WO-6149553 | Waste Heat Recovery | NSG C&I Custom | 2 | 100% |  |
| WO-4297269 | Boiler Condensate Recovery | PG C&I Custom | 2 | 91% | Wrong calculation tab used as final savings |
| WO-6232126 | Insulation | PG C&I Custom | 3 | 100% |  |
| WO-6014800 | Insulation | NSG C&I Custom | 2 | 100% |  |
| WO-7999106 | Insulation | NSG C&I Custom | 2 | 100% |  |
| WO-6961435 | Insulation | PG C&I Custom | 3 | 100% |  |
| WO-4911636 | Space Conditioning Controls | PG Public Custom | 3 | 100% |  |
| WO-5887462 | Steam Chest Leak Replacement | NSG C&I Custom | 1 | 100% |  |
| WO-4514894 | Burner Replacement | PG C&I Custom | 3 | 112% | Billing analysis |
| WO-5921741 | Space Conditioning Controls | PG Public Custom | 3 | 188% | Billing analysis and usage of TMYx dataset |

Source: Evaluation team analysis.

# Appendix B. Program Specific Inputs for the Illinois TRC

The Table 12 and Table 13 below show the Total Resource Cost (TRC) cost-effectiveness analysis inputs available at the time of producing this impact evaluation report. Additional required cost data (e.g., measure costs, program level incentive and non-incentive costs) are not included in this table and will be provided to the evaluation team later. Guidehouse will include annual and lifetime water savings and greenhouse gas reductions in the end of year summary report.

Table 12. Verified Cost Effectiveness Inputs - PGL

| Program Category | Program Path | Savings Category | Units | Quantity | Effective Useful Life | Early Replacement Flag† | Ex Ante Gross Savings (Therms) | Verified Gross Savings (Therms) | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Private | Custom | Boiler Condensate Recovery | Project | 1.00 | 15.00 | NO | 43,593 | 45,873 | 42,661 |
|  |  | Burner Replacement | Project | 1.00 | 15.00 | NO | 25,318 | 27,996 | 26,036 |
|  |  | Insulation | Project | 6.00 | 15.00 | NO | 51,934 | 57,238 | 53,231 |
|  |  | Space Conditioning Controls | Project | 1.00 | 15.00 | NO | 6,755 | 7,469 | 6,946 |
|  |  | Other | Project | 1.00 | 15.00 | NO | 2,321 | 2,452 | 2,280 |
| Public | Custom | Space Conditioning Controls | Project | 3.00 | 15.00 | NO | 13,594 | 15,004 | 13,804 |
|  |  | High Speed Door | Project | 1.00 | 16.00 | NO | 21,445 | 23,713 | 23,713 |
|  |  | Space Conditioning Controls | Project | 2.00 | 15.00 | NO | 6,806 | 7,525 | 7,525 |
|  |  | Insulation | Project | 1.00 | 15.00 | NO | 1,857 | 1,962 | 1,962 |
| Total or Weighted Average |  |  |  |  | 15.00 |  | 173,622 | 189,231 | 178,159 |

Source: Evaluation team analysis.

Table 13. Verified Cost Effectiveness Inputs – NSG

| Program Category | Program Path | Savings Category | Units | Quantity | Effective Useful Life | Early Replacement Flag† | Ex Ante Gross Savings (Therms) | Verified Gross Savings (Therms) | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Private | Custom | Steam Chest Leak Replacement | Project | 1.00 | 15.00 | NO | 106,193 | 106,016 | 98,595 |
|  |  | Waste Heat Recovery | Project | 2.00 | 15.00 | NO | 73,016 | 76,835 | 71,457 |
|  |  | Insulation | Project | 3.00 | 15.00 | NO | 81,162 | 86,034 | 80,012 |
| Public | Custom | Insulation | Project | 1.00 | 15.00 | NO | 6,357 | 7,030 | 6,467 |
| Total or Weighted Average |  |  |  |  | 15.00 |  | 266,728 | 275,915 | 256,531 |

Source: Evaluation team analysis.