# Energy Education Outreach Program Impact Evaluation Report

Energy Efficiency Plan: Program Year 2025

(01/01/2025-12/31/2025)

Prepared for:

Peoples Gas and North Shore Gas

DRAFT

March 19, 2026

Prepared by:

| Kapil Mallarappu EcoMetric | Fahman Khan EcoMetric | Mike Frischmann EcoMetric |  |
| --- | --- | --- | --- |

Submitted to:

Peoples Gas

North Shore Gas

200 East Randolph Street

Chicago, IL 60601

Submitted by:

Guidehouse Inc.

167 N. Green Street

12th Floor

Chicago, IL 60607

Contact:

| Charles Maglione, Partner 202.481.7352 cmaglione@guidehouse.com | Jeff Erickson, Director 608.616.4962 jeff.erickson@guidehouse.com | Laura Agapay-Read, Associate Director 312.583.4178 laura.agapay.read@guidehouse.com |
| --- | --- | --- |

Charles Ampong, Associate Director

608.446.3172

charles.ampong@guidehouse.com

This report was prepared by Guidehouse for People Gas Light and Coke Company (PGL)and North Shore Gas(NSG) companies. The work presented in this report represents Guidehouse’s professional judgment based on the information available at the time this report was prepared. Use of this report by any other party for whatever purpose should not, and does not, absolve such party from using due diligence in verifying the report’s contents. Neither Guidehouse nor any of its subsidiaries or affiliates assumes any liability or duty of care to such parties and hereby disclaims any such liability.

Table of Contents

1	Introduction	1

2	Program Description	1

3	Program Savings Detail	4

4	Program Savings by Measure	5

5	Impact Analysis Findings and Recommendations	9

5.1	Impact Parameter Estimates	9

5.2	Findings and Recommendations	12

Appendix A. Impact Analysis Methodology	18

Appendix B. Program Specific Inputs for the Illinois TRC	20

Table of Tables, Figures, and Equations

Table 1. 2025 Volumetric Summary for PGL	1

Table 2. 2025 Installed Measure Quantities for PGL	1

Table 3. 2025 Volumetric Summary for NSG	3

Table 4. 2025 Installed Measure Quantities for NSG	3

Table 5. 2025 Annual Energy Savings Summary for PGL	4

Table 6. 2025 Annual Energy Savings Summary for NSG	5

Table 7. 2025 Annual Energy Savings by Measure for PGL	5

Table 8. 2025 Annual Energy Savings by Measure for NSG	7

Table 9. Verified Gross Savings Parameters	9

Table A-1. Impact Parameters	18

Table B-1. Verified Cost Effectiveness Inputs - PGL	20

Table B-2. Verified Cost Effectiveness Inputs – NSG	23

# Introduction

This report presents the results of the impact evaluation of the Peoples Gas (PGL) and North Shore Gas (NSG) 2025 Energy Education Outreach (EEO) Kits Program. It presents a summary of the energy impacts for the total program and broken out by relevant measure and program structure details. Appendix A presents the impact analysis methodology. Appendix B shows the Total Resource Cost (TRC) cost-effectiveness analysis inputs.  Program year 2025 covers January 1 to December 31, 2025.

# Program Description

The EEO Kits program is offered jointly to schools served by PGL or NSG and ComEd. PGL and NSG claimed all the gas savings, while ComEd claimed all the electricity savings from kits sent to the joint utility customers. The EEO Kits program has three paths, Income Eligible, Market Rate and Market Rate B Kits. Income Eligible and Market Rate kits consist of five water saving measures including faucet aerators, showerheads, shower timers, temperature setback cards, and three air sealing measures including door sweeps, window insulation kits, and weatherstripping rolls. Market Rate Kit B includes four additional air sealing measures: rope caulk, V-Seal Weatherstripping Roll, and switch and outlet gaskets. Income eligible kits are distributed at schools that are located in low-income communities.

The program had 11,080 participants in 2025 and distributed 100,061 measures, as shown in Table 1.

Table 1. 2025 Volumetric Summary for PGL

| Participation | Income Eligible | Market Rate | Market Rate B | Total |
| --- | --- | --- | --- | --- |
| Participants* | 3,796 | 7,253 | 31 | 11,080 |
| Distributed Measures † | 65,277 | 34,164 | 620 | 100,061 |
| Measure Types Installed ‡ | 8 | 8 | 12 | 28 |

* Participants are defined as number of kits distributed
† Distributed Measures is the total quantity of measures distributed

‡ Measure types installed are the distinct count of PGL measure names

Source: Peoples Gas tracking data and evaluation team analysis.

Table 2 summarizes the installed measure quantities that are the basis for verified energy savings.

Table 2. 2025 Installed Measure Quantities for PGL

| Program Category | Program Path | Measure | Quantity Unit | Installed Quantity |
| --- | --- | --- | --- | --- |
| Non-Disadvantaged Communities | Market Rate | Low-Flow Showerhead (1.5 GPM) MR | Each | 2,784 |
|  |  | Kitchen Aerator (1.5 GPM) MR | Each | 2,784 |
|  |  | Bathroom Aerator (1.0 GPM) MR | Each | 5,568 |
|  |  | Shower Timer MR | Each | 2,784 |
|  |  | Temperature Setback Card MR | Each | 2,784 |
|  |  | Self-Adhesive Door Sweep MR | Each | 2,784 |
|  |  | Window Insultation Kit (5-pack) MR | Each | 2,784 |
|  |  | Closed-Cell Foam Weatherstripping Roll (17') MR | Ln Ft | 2,784 |
| Non-Disadvantaged Communities | Market Rate B | Closed-Foam Weatherstripping Roll (17') MRB | Each | 62 |
|  |  | Rope Caulk (30') MRB | Each | 62 |
|  |  | Self-Adhesive Door Sweep MRB | Each | 31 |
|  |  | Low-Flow Showerhead (1.5 GPM) MRB | Each | 31 |
|  |  | V-Seal Weatherstripping Roll (17') MRB | Each | 31 |
|  |  | Outlet Gasket MRB | Each | 124 |
|  |  | Switch Gasket MRB | Each | 124 |
|  |  | Bathroom Aerator (1 GPM) MRB | Each | 31 |
|  |  | Kitchen Aerator (1.5 GPM) MRB | Ln Ft | 31 |
|  |  | Temperature Setback Card MRB | Each | 31 |
|  |  | Shower Timer MRB | Each | 31 |
|  |  | Window Insulation Kit (Pack of 5) MRB | Each | 31 |
| Disadvantaged Communities | Market Rate | Low-Flow Showerhead (1.5 GPM) MR DAC | Each | 1,012 |
|  |  | Kitchen Aerator (1.5 GPM) MR DAC | Each | 1,012 |
|  |  | Bathroom Aerator (1.0 GPM) MR DAC | Each | 2,024 |
|  |  | Shower Timer MR DAC | Each | 1,012 |
|  |  | Temperature Setback Card MR DAC | Each | 1,012 |
|  |  | Self-Adhesive Door Sweep MR DAC | Each | 1,012 |
|  |  | Window Insultation Kit (5-pack) MR DAC | Each | 1,012 |
|  |  | Closed-Cell Foam Weatherstripping Roll (17') MR DAC | Ln Ft | 1,012 |
|  | Income Eligible | Low-Flow Showerhead (1.5 GPM) IE | Each | 7,253 |
|  |  | Kitchen Aerator (1.5 GPM) IE | Each | 7,253 |
|  |  | Bathroom Aerator (1.0 GPM) IE | Each | 14,506 |
|  |  | Shower Timer IE | Each | 7,253 |
|  |  | Temperature Setback Card IE | Each | 7,253 |
|  |  | Self-Adhesive Door Sweep IE | Each | 7,253 |
|  |  | Window Insultation Kit (5-pack) IE | Each | 7,253 |
|  |  | Closed-Cell Foam Weatherstripping Roll (17') IE | Ln Ft | 7,253 |

Source: Peoples Gas tracking data and evaluation team analysis.

The NSG program had 2,416 participants in 2025 and distributed 21,744 measures, as shown in Table 3.

Table 3. 2025 Volumetric Summary for NSG

| Participation | Income Eligible | Market Rate | Total |
| --- | --- | --- | --- |
| Participants | 897 | 1,519 | 2,416 |
| Distributed Measures † | 8,073 | 13,671 | 21,744 |
| Measure Types Installed ‡ | 8 | 8 | 16 |

* Participants are defined as number of kits distributed
† Distributed Measures is the total quantity of measures distributed

‡ Measure types installed are the distinct count of PGL measure names

Source: Peoples Gas tracking data and evaluation team analysis.

Table 4 summarizes the installed measure quantities that are the basis for verified energy savings.

Table 4. 2025 Installed Measure Quantities for NSG

| Program Category | Program Path | Measure | Quantity Unit | Installed Quantity |
| --- | --- | --- | --- | --- |
| Non-Disadvantaged Communities | Market Rate | Low-Flow Showerhead (1.5 GPM) MR | Unit | 1,456 |
|  |  | Kitchen Aerator (1.5 GPM) MR | Unit | 1,456 |
|  |  | Bathroom Aerator (1.0 GPM) MR | Unit | 2,912 |
|  |  | Shower Timer MR | Unit | 1,456 |
|  |  | Temperature Setback Card MR | Unit | 1,456 |
|  |  | Self-Adhesive Door Sweep MR | Unit | 1,456 |
|  |  | Window Insultation Kit (5-pack) MR | Unit | 1,456 |
|  |  | Closed-Cell Foam Weatherstripping Roll (17') MR | Ln Ft | 1,456 |
| Disadvantaged Communities | Market Rate | Low-Flow Showerhead (1.5 GPM) MR DAC | Unit | 63 |
|  |  | Kitchen Aerator (1.5 GPM) MR DAC | Unit | 63 |
|  |  | Bathroom Aerator (1.0 GPM) MR DAC | Unit | 126 |
|  |  | Shower Timer MR DAC | Unit | 63 |
|  |  | Temperature Setback Card MR DAC | Unit | 63 |
|  |  | Self-Adhesive Door Sweep MR DAC | Unit | 63 |
|  |  | Window Insultation Kit (5-pack) MR DAC | Unit | 63 |
|  |  | Closed-Cell Foam Weatherstripping Roll (17') MR DAC | Ln Ft | 63 |
|  | Income Eligible | Low-Flow Showerhead (1.5 GPM) IE | Unit | 897 |
|  |  | Kitchen Aerator (1.5 GPM) IE | Unit | 897 |
|  |  | Bathroom Aerator (1.0 GPM) IE | Unit | 1,794 |
|  |  | Shower Timer IE | Unit | 897 |
|  |  | Temperature Setback Card IE | Unit | 897 |
|  |  | Self-Adhesive Door Sweep IE | Unit | 897 |
|  |  | Window Insultation Kit (5-pack) IE | Unit | 897 |
|  |  | Closed-Cell Foam Weatherstripping Roll (17') IE | Ln Ft | 897 |

Source: North Shore Gas tracking data and Guidehouse evaluation team analysis.

# Program Savings Detail

Table 5 summarizes the energy savings the PGL EEO Program achieved by paths Income Eligible, Market Rate and Market Rate B in 2025.

Table 5. 2025 Annual Energy Savings Summary for PGL

| Program Category | Program Path | Ex Ante Gross Savings (Therms) | Verified Gross RR* | Verified Gross Savings (Therms) | NTG† | NSPO‡ | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Non-Disadvantaged Communities | Market Rate | 71,346 | 100% | 71,345 | 1.00 | 1.083 | 77,267 |
|  | Market Rate B | 1,399 | 100% | 1,399 | 1.00 | 1.083 | 1,515 |
| Non-DAC Subtotal |  | 72,745 |  | 72,745 |  |  | 78,782 |
| Disadvantaged Communities | Market Rate | 25,934 | 100% | 25,934 | 1.00 | 1.083 | 28,087 |
| DAC Subtotal |  | 25,934 |  | 25,934 |  |  | 28,087 |
|  | Income Eligible | 178,656 | 100% | 178,656 | 1.00 | N/A | 178,656 |
| Total |  | 277,335 | 100% | 277,335 |  |  | 285,525 |

* Realization Rate (RR) is the ratio of verified gross savings to ex ante gross savings, based on evaluation research findings.

† NTG, Net to Gross is the deemed value available on the SAG website: https://www.ilsag.info/evaluator-ntg-recommendations-for-2025/.

‡ The market rate net savings were multiplied by a residential non-participant spillover (NPSO) factor of 1.048. Based on SAG Policy, for Disadvantaged communities (DAC projects), if deemed NTG is multiplied by the NPSO and the resulted NTG value is less than 1.00, the evaluation assigned a DAC NTG of 1.00. If the resulted NTG value is >1.00, evaluation used the >1.00 value for calculation of net savings impact.

Source: Evaluation team analysis.

Table 6 summarizes the energy savings the NSG EEO Program achieved by paths Income Eligible and Market Rate in 2025.

Table 6. 2025 Annual Energy Savings Summary for NSG

| Program Category | Program Path | Ex Ante Gross Savings (Therms) | Verified Gross RR* | Verified Gross Savings (Therms) | NTG† | NSPO‡ | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Non-Disadvantaged Communities | Market Rate | 36,742 | 100% | 36,742 | 1.00 | 1.083 | 39,792 |
| Disadvantaged Communities | Market Rate | 1,590 | 100% | 1,590 | 1.00 | 1.083 | 1,722 |
|  | Income Eligible | 22,264 | 100% | 22,264 | 1.00 | N/A | 22,264 |
| Total |  | 60,596 | 100% | 60,595 |  |  | 63,777 |

* Realization Rate (RR) is the ratio of verified gross savings to ex ante gross savings, based on evaluation research findings.

† NTG, Net to Gross is the deemed value available on the SAG website: https://www.ilsag.info/evaluator-ntg-recommendations-for-2025/.

‡ The market rate net savings were multiplied by a residential non-participant spillover (NPSO) factor of 1.083. Based on SAG Policy, for Disadvantaged communities (DAC projects), if deemed NTG is multiplied by the NPSO and the resulted NTG value is less than 1.00, the evaluation assigned a DAC NTG of 1.00. If the resulted NTG value is >1.00, evaluation used the >1.00 value for calculation of net savings impact.

Source: Evaluation team analysis.

# Program Savings by Measure

The PGL program includes eight measures under the Market Rate and Income Eligible paths and twelve measures in the Market Rate B Kit, as shown in Table 7. The Window Insulation Kit (5-pack) and Low Flow Showerheads measures contributed the most savings for the Market Rate and Income Eligible paths. The Rope Caulk (30') and Closed-Foam Weatherstripping Roll (17') MRB measures contributed the most savings for the Market Rate B path.

Table 7. 2025 Annual Energy Savings by Measure for PGL

| Program Category | Program Path | Savings Category | Ex Ante Gross Savings (Therms) | Verified Gross RR* | Verified Gross Savings (Therms) | NTG† | NSPO‡ | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Non-Disadvantaged Communities | Market Rate | Low-Flow Showerhead (1.5 GPM) MR | 17,982 | 100% | 17,982 | 1.00 | 1.083 | 19,474 |
|  |  | Kitchen Aerator (1.5 GPM) MR | 2,391 | 100% | 2,391 | 1.00 | 1.083 | 2,590 |
|  |  | Bathroom Aerator (1.0 GPM) MR | 2,890 | 100% | 2,890 | 1.00 | 1.083 | 3,130 |
|  |  | Shower Timer MR | 11,247 | 100% | 11,247 | 1.00 | 1.083 | 12,181 |
|  |  | Temperature Setback Card MR | 379 | 100% | 379 | 1.00 | 1.083 | 410 |
|  |  | Self-Adhesive Door Sweep MR | 9,087 | 100% | 9,087 | 1.00 | 1.083 | 9,841 |
|  |  | Window Insultation Kit (5-pack) MR | 17,066 | 100% | 17,066 | 1.00 | 1.083 | 18,482 |
|  |  | Closed-Cell Foam Weatherstripping Roll (17') MR | 10,304 | 100% | 10,304 | 1.00 | 1.083 | 11,159 |
| Non-Disadvantaged Communities | Market Rate B | Closed-Foam Weatherstripping Roll (17') MRB | 229 | 100% | 229 | 1.00 | 1.083 | 249 |
|  |  | Rope Caulk (30') MRB | 346 | 100% | 346 | 1.00 | 1.083 | 375 |
|  |  | Self-Adhesive Door Sweep MRB | 101 | 100% | 101 | 1.00 | 1.083 | 110 |
|  |  | Low-Flow Showerhead (1.5 GPM) MRB | 200 | 100% | 200 | 1.00 | 1.083 | 217 |
|  |  | V-Seal Weatherstripping Roll (17') MRB | 115 | 100% | 115 | 1.00 | 1.083 | 124 |
|  |  | Outlet Gasket MRB | 21 | 100% | 21 | 1.00 | 1.083 | 23 |
|  |  | Switch Gasket MRB | 21 | 100% | 21 | 1.00 | 1.083 | 23 |
|  |  | Bathroom Aerator (1 GPM) MRB | 16 | 100% | 16 | 1.00 | 1.083 | 17 |
|  |  | Kitchen Aerator (1.5 GPM) MRB | 27 | 100% | 27 | 1.00 | 1.083 | 29 |
|  |  | Temperature Setback Card MRB | 7 | 100% | 7 | 1.00 | 1.083 | 8 |
|  |  | Shower Timer MRB | 125 | 100% | 125 | 1.00 | 1.083 | 136 |
|  |  | Window Insulation Kit (Pack of 5) MRB | 190 | 100% | 190 | 1.00 | 1.083 | 206 |
| Disadvantaged Communities | Market Rate | Low-Flow Showerhead (1.5 GPM) MR DAC | 6,537 | 100% | 6,537 | 1.00 | 1.083 | 7,079 |
|  |  | Kitchen Aerator (1.5 GPM) MR DAC | 869 | 100% | 869 | 1.00 | 1.083 | 941 |
|  |  | Bathroom Aerator (1.0 GPM) MR DAC | 1,050 | 100% | 1,050 | 1.00 | 1.083 | 1,138 |
|  |  | Shower Timer MR DAC | 4,088 | 100% | 4,088 | 1.00 | 1.083 | 4,428 |
|  |  | Temperature Setback Card MR DAC | 138 | 100% | 138 | 1.00 | 1.083 | 149 |
|  |  | Self-Adhesive Door Sweep MR DAC | 3,303 | 100% | 3,303 | 1.00 | 1.083 | 3,577 |
|  |  | Window Insultation Kit (5-pack) MR DAC | 6,204 | 100% | 6,204 | 1.00 | 1.083 | 6,718 |
|  |  | Closed-Cell Foam Weatherstripping Roll (17') MR DAC | 3,745 | 100% | 3,745 | 1.00 | 1.083 | 4,056 |
|  | Income Eligible | Low-Flow Showerhead (1.5 GPM) IE | 49,930 | 100% | 49,930 | 1.00 | N/A | 49,930 |
|  |  | Kitchen Aerator (1.5 GPM) IE | 6,644 | 100% | 6,644 | 1.00 | N/A | 6,644 |
|  |  | Bathroom Aerator (1.0 GPM) IE | 8,022 | 100% | 8,022 | 1.00 | N/A | 8,022 |
|  |  | Shower Timer IE | 26,836 | 100% | 26,836 | 1.00 | N/A | 26,836 |
|  |  | Temperature Setback Card IE | 986 | 100% | 986 | 1.00 | N/A | 986 |
|  |  | Self-Adhesive Door Sweep IE | 21,498 | 100% | 21,498 | 1.00 | N/A | 21,498 |
|  |  | Window Insultation Kit (5-pack) IE | 40,363 | 100% | 40,363 | 1.00 | N/A | 40,363 |
|  |  | Closed-Cell Foam Weatherstripping Roll (17') IE | 24,377 | 100% | 24,377 | 1.00 | N/A | 24,377 |
| Total |  |  | 277,335 | 100% | 277,335 |  |  | 285,525 |

* Realization Rate (RR) is the ratio of verified gross savings to ex ante gross savings, based on evaluation research findings.

† NTG, Net to Gross is the deemed value available on the SAG website: https://www.ilsag.info/evaluator-ntg-recommendations-for-2025/.

‡ The market rate net savings were multiplied by a residential non-participant spillover (NPSO) factor of 1.083. Based on SAG Policy, for Disadvantaged communities (DAC projects), if deemed NTG is multiplied by the NPSO and the resulted NTG value is less than 1.00, the evaluation assigned a DAC NTG of 1.00. If the resulted NTG value is >1.00, evaluation used the >1.00 value for calculation of net savings impact.

Source: Evaluation team analysis.

The NSG program includes eight measures under the Market Rate and Income Eligible paths, as shown in the following table. The Window Insulation Kit (5-pack) and Low Flow Showerheads measures contributed the most savings for each path.

Table 8. 2025 Annual Energy Savings by Measure for NSG

| Program Category | Program Path | Savings Category | Ex Ante Gross Savings (Therms) | Verified Gross RR* | Verified Gross Savings (Therms) | NTG† | NSPO‡ | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Non-Disadvantaged Communities | Market Rate | Low-Flow Showerhead (1.5 GPM) MR | 9,404 | 100% | 9,404 | 1.00 | 1.083 | 10,185 |
|  |  | Kitchen Aerator (1.5 GPM) MR | 1,251 | 100% | 1,251 | 1.00 | 1.083 | 1,355 |
|  |  | Bathroom Aerator (1.0 GPM) MR | 1,511 | 100% | 1,511 | 1.00 | 1.083 | 1,637 |
|  |  | Shower Timer MR | 5,311 | 100% | 5,311 | 1.00 | 1.083 | 5,752 |
|  |  | Temperature Setback Card MR | 198 | 100% | 198 | 1.00 | 1.083 | 214 |
|  |  | Self-Adhesive Door Sweep MR | 4,752 | 100% | 4,752 | 1.00 | 1.083 | 5,147 |
|  |  | Window Insultation Kit (5-pack) MR | 8,925 | 100% | 8,925 | 1.00 | 1.083 | 9,666 |
|  |  | Closed-Cell Foam Weatherstripping Roll (17') MR | 5,389 | 100% | 5,389 | 1.00 | 1.083 | 5,836 |
| Disadvantaged Communities | Market Rate | Low-Flow Showerhead (1.5 GPM) MR DAC | 407 | 100% | 407 | 1.00 | 1.083 | 441 |
|  |  | Kitchen Aerator (1.5 GPM) MR DAC | 54 | 100% | 54 | 1.00 | 1.083 | 59 |
|  |  | Bathroom Aerator (1.0 GPM) MR DAC | 65 | 100% | 65 | 1.00 | 1.083 | 71 |
|  |  | Shower Timer MR DAC | 230 | 100% | 230 | 1.00 | 1.083 | 249 |
|  |  | Temperature Setback Card MR DAC | 9 | 100% | 9 | 1.00 | 1.083 | 9 |
|  |  | Self-Adhesive Door Sweep MR DAC | 206 | 100% | 206 | 1.00 | 1.083 | 223 |
|  |  | Window Insultation Kit (5-pack) MR DAC | 386 | 100% | 386 | 1.00 | 1.083 | 418 |
|  |  | Closed-Cell Foam Weatherstripping Roll (17') MR DAC | 233 | 100% | 233 | 1.00 | 1.083 | 253 |
|  | Income Eligible | Low-Flow Showerhead (1.5 GPM) IE | 6,175 | 100% | 6,175 | 1.00 | N/A | 6,175 |
|  |  | Kitchen Aerator (1.5 GPM) IE | 822 | 100% | 822 | 1.00 | N/A | 822 |
|  |  | Bathroom Aerator (1.0 GPM) IE | 992 | 100% | 992 | 1.00 | N/A | 992 |
|  |  | Shower Timer IE | 3,488 | 100% | 3,488 | 1.00 | N/A | 3,488 |
|  |  | Temperature Setback Card IE | 122 | 100% | 122 | 1.00 | N/A | 122 |
|  |  | Self-Adhesive Door Sweep IE | 2,659 | 100% | 2,659 | 1.00 | N/A | 2,659 |
|  |  | Window Insultation Kit (5-pack) IE | 4,992 | 100% | 4,992 | 1.00 | N/A | 4,992 |
|  |  | Closed-Cell Foam Weatherstripping Roll (17') IE | 3,015 | 100% | 3,015 | 1.00 | N/A | 3,015 |
| Total |  |  | 60,596 | 100% | 60,595 |  |  | 63,777 |

* Realization Rate (RR) is the ratio of verified gross savings to ex ante gross savings, based on evaluation research findings.

† NTG, Net to Gross is the deemed value available on the SAG website: https://www.ilsag.info/evaluator-ntg-recommendations-for-2025/.

‡ The market rate net savings were multiplied by a residential non-participant spillover (NPSO) factor of 1.083. Based on SAG Policy, for Disadvantaged communities (DAC projects), if deemed NTG is multiplied by the NPSO and the resulted NTG value is less than 1.00, the evaluation assigned a DAC NTG of 1.00. If the resulted NTG value is >1.00, evaluation used the >1.00 value for calculation of net savings impact.

Source: Evaluation team analysis.

# Impact Analysis Findings and Recommendations

## Impact Parameter Estimates

Table 9 shows the unit therm savings and realization rate findings by measure from our review. The realization rate is the ratio of the verified savings to the ex ante savings. Following the table, we provide findings and recommendations, including discussion of all measures with realization rates above or below 100%. Appendix A provides a description of the impact analysis methodology.

Table 9. Verified Gross Savings Parameters

| Measure | Unit Basis | Ex Ante Gross (therms/unit) | Verified Gross (therms/unit) | Realization Rate | Illinois TRM, v13.0 Section |
| --- | --- | --- | --- | --- | --- |
| PGL EEO Kits Market Rate |  |  |  |  |  |
| Low-Flow Showerhead (1.5 GPM) MR | Unit | 6.46 | 6.46 | 1.00 | 5.4.5 |
| Kitchen Aerator (1.5 GPM) MR | Unit | 0.86 | 0.86 | 1.00 | 5.4.4 |
| Bathroom Aerator (1.0 GPM) MR | Unit | 0.52 | 0.52 | 1.00 | 5.4.4 |
| Shower Timer MR | Unit | 4.04 | 4.04 | 1.00 | 5.4.9 |
| Temperature Setback Card MR | Unit | 0.14 | 0.14 | 1.00 | 5.4.6 |
| Self-Adhesive Door Sweep MR | Unit | 3.26 | 3.26 | 1.00 | 5.6.1 |
| Window Insultation Kit (5-pack) MR | Unit | 6.13 | 6.13 | 1.00 | 5.6.1 |
| Closed-Cell Foam Weatherstripping Roll (17') MR | Ln Ft | 3.70 | 3.70 | 1.00 | 5.6.1 |
| PGL EEO Kits Market Rate B |  |  |  |  |  |
| Closed-Foam Weatherstripping Roll (17') MRB | Unit | 3.70 | 3.70 | 1.00 | 5.4.5 |
| Rope Caulk (30') MRB | Unit | 5.58 | 5.58 | 1.00 | 5.4.4 |
| Self-Adhesive Door Sweep MRB | Unit | 3.26 | 3.26 | 1.00 | 5.4.4 |
| Low-Flow Showerhead (1.5 GPM) MRB | Unit | 6.46 | 6.46 | 1.00 | 5.4.9 |
| V-Seal Weatherstripping Roll (17') MRB | Unit | 3.70 | 3.70 | 1.00 | 5.4.6 |
| Outlet Gasket MRB | Unit | 0.17 | 0.17 | 1.00 | 5.6.1 |
| Switch Gasket MRB | Unit | 0.17 | 0.17 | 1.00 | 5.6.1 |
| Bathroom Aerator (1 GPM) MRB | Unit | 0.52 | 0.52 | 1.00 | 5.6.1 |
| Kitchen Aerator (1.5 GPM) MRB | Ln Ft | 0.86 | 0.86 | 1.00 | 5.6.1 |
| Temperature Setback Card MRB | Unit | 0.24 | 0.24 | 1.00 | 5.6.1 |
| Shower Timer MRB | Unit | 4.04 | 4.04 | 1.00 | 5.6.1 |
| Window Insulation Kit (Pack of 5) MRB | Unit | 6.13 | 6.13 | 1.00 | 5.6.1 |
| PGL EEO Kits Income Eligible |  |  |  |  |  |
| Low-Flow Showerhead (1.5 GPM) IE | Unit | 6.88 | 6.88 | 1.00 | 5.4.5 |
| Kitchen Aerator (1.5 GPM) IE | Unit | 0.92 | 0.92 | 1.00 | 5.4.4 |
| Bathroom Aerator (1.0 GPM) IE | Unit | 0.55 | 0.55 | 1.00 | 5.4.4 |
| Shower Timer IE | Unit | 3.70 | 3.70 | 1.00 | 5.4.9 |
| Temperature Setback Card IE | Unit | 0.14 | 0.14 | 1.00 | 5.4.6 |
| Self-Adhesive Door Sweep IE | Unit | 2.96 | 2.96 | 1.00 | 5.6.1 |
| Window Insultation Kit (5-pack) IE | Unit | 5.57 | 5.57 | 1.00 | 5.6.1 |
| Closed-Cell Foam Weatherstripping Roll (17') IE | Ln Ft | 3.36 | 3.36 | 1.00 | 5.6.1 |
| PGL EEO Kits DAC |  |  |  |  |  |
| Low-Flow Showerhead (1.5 GPM) MR DAC | Unit | 6.46 | 6.46 | 1.00 | 5.4.5 |
| Kitchen Aerator (1.5 GPM) MR DAC | Unit | 0.86 | 0.86 | 1.00 | 5.4.4 |
| Bathroom Aerator (1.0 GPM) MR DAC | Unit | 0.52 | 0.52 | 1.00 | 5.4.4 |
| Shower Timer MR DAC | Unit | 4.04 | 4.04 | 1.00 | 5.4.9 |
| Temperature Setback Card MR DAC | Unit | 0.14 | 0.14 | 1.00 | 5.4.6 |
| Self-Adhesive Door Sweep MR DAC | Unit | 3.26 | 3.26 | 1.00 | 5.6.1 |
| Window Insultation Kit (5-pack) MR DAC | Unit | 6.13 | 6.13 | 1.00 | 5.6.1 |
| Closed-Cell Foam Weatherstripping Roll (17') MR DAC | Ln Ft | 3.70 | 3.70 | 1.00 | 5.6.1 |
| NSG EEO Kits Market Rate |  |  |  |  |  |
| Low-Flow Showerhead (1.5 GPM) MR | Unit | 6.46 | 6.46 | 1.00 | 5.4.5 |
| Kitchen Aerator (1.5 GPM) MR | Unit | 0.86 | 0.86 | 1.00 | 5.4.4 |
| Bathroom Aerator (1.0 GPM) MR | Unit | 0.52 | 0.52 | 1.00 | 5.4.4 |
| Shower Timer MR | Unit | 3.65 | 3.65 | 1.00 | 5.4.9 |
| Temperature Setback Card MR | Unit | 0.14 | 0.14 | 1.00 | 5.4.6 |
| Self-Adhesive Door Sweep MR | Unit | 3.26 | 3.26 | 1.00 | 5.6.1 |
| Window Insultation Kit (5-pack) MR | Unit | 6.13 | 6.13 | 1.00 | 5.6.1 |
| Closed-Cell Foam Weatherstripping Roll (17') MR | Ln Ft | 3.70 | 3.70 | 1.00 | 5.6.1 |
| NSG EEO Kits Income Eligible |  |  |  |  |  |
| Low-Flow Showerhead (1.5 GPM) IE | Unit | 6.88 | 6.88 | 1.00 | 5.4.5 |
| Kitchen Aerator (1.5 GPM) IE | Unit | 0.92 | 0.92 | 1.00 | 5.4.4 |
| Bathroom Aerator (1.0 GPM) IE | Unit | 0.55 | 0.55 | 1.00 | 5.4.4 |
| Shower Timer IE | Unit | 3.89 | 3.89 | 1.00 | 5.4.9 |
| Temperature Setback Card IE | Unit | 0.14 | 0.14 | 1.00 | 5.4.6 |
| Self-Adhesive Door Sweep IE | Unit | 2.96 | 2.96 | 1.00 | 5.6.1 |
| Window Insultation Kit (5-pack) IE | Unit | 5.57 | 5.57 | 1.00 | 5.6.1 |
| Closed-Cell Foam Weatherstripping Roll (17') IE | Ln Ft | 3.36 | 3.36 | 1.00 | 5.6.1 |
| NSG EEO Kits DAC |  |  |  |  |  |
| Low-Flow Showerhead (1.5 GPM) MR DAC | Unit | 6.46 | 6.46 | 1.00 | 5.4.5 |
| Kitchen Aerator (1.5 GPM) MR DAC | Unit | 0.86 | 0.86 | 1.00 | 5.4.4 |
| Bathroom Aerator (1.0 GPM) MR DAC | Unit | 0.52 | 0.52 | 1.00 | 5.4.4 |
| Shower Timer MR DAC | Unit | 3.65 | 3.65 | 1.00 | 5.4.9 |
| Temperature Setback Card MR DAC | Unit | 0.14 | 0.14 | 1.00 | 5.4.6 |
| Self-Adhesive Door Sweep MR DAC | Unit | 3.26 | 3.26 | 1.00 | 5.6.1 |
| Window Insultation Kit (5-pack) MR DAC | Unit | 6.13 | 6.13 | 1.00 | 5.6.1 |
| Closed-Cell Foam Weatherstripping Roll (17') MR DAC | Ln Ft | 3.70 | 3.70 | 1.00 | 5.6.1 |

† State of Illinois Technical Reference Manual version 13.0 from http://www.ilsag.info/technical-reference-manual.html.

Source: Program tracking data and evaluation team analysis.

## Findings and Recommendations

Ex ante savings were calculated using IL-TRM v13.0 algorithms and assumptions, with a 100% realization rate observed. The evaluation team, however, noticed that savings were reported inconsistently in different fields of the tracking data. The following findings outline these discrepancies.

Finding 1. For both PGL and NSG Income Eligible kits, savings reported in the gross therms field appear to represent total gross therm savings (i.e., per-kit savings multiplied by the number of kits distributed). However, the total gross therms field reports double the savings shown in the gross therms field and the utility scorecard. Because the savings reported in the gross therms field align with the savings claimed in the utility scorecard, the evaluation team used the gross therms field as the basis for ex ante savings.

Finding 2. For both PGL and NSG Market Rate kits, the total net therms field appears to represent total ex ante gross therm savings. Therefore, the evaluation team used the ex-ante total net therms field in place of the total gross therm savings while calculating verified gross savings. PGL Market Rate Kit B was an exception where the total gross therms are reported correctly.

Recommendation 1.  Ensure that the total gross therm savings in the program tracking data are calculated and reported as the product of per-kit gross therms and number of kits distributed.

# Appendix A. Impact Analysis Methodology

The evaluation team verified per-unit savings for each program measure using the impact algorithm sources found in the Illinois Statewide Technical Reference Manual v13.0 (IL-TRM). Table A-1 presents the key parameters and the references used in the verified gross and net savings calculations.

Table A-1. Impact Parameters

| Gross Savings Input Parameters | Deemed Input Parameters | Illinois TRM, v13.0, Section |
| --- | --- | --- |
| Low-Flow Showerhead (1.5 GPM) | GPM_base, GPM_low, L_base, L_low, Showerheads per household (SPH), showers per capita per day (SPCD), EPG_gas, %FossilDHW, Household, ISR, %SF, %MF | 5.4.5 |
| Kitchen Aerator (1.5 GPM) | GPM_base, GPM_low, L_base, L_low, faucets per household (FPH), drain factor (DF), EPG_gas, %FossilDHW, Household, ISR, %SF, %MF | 5.4.4 |
| Bathroom Aerator (1.0 GPM) | GPM_base, GPM_low, L_base, L_low, faucets per household (FPH), drain factor (DF), EPG_gas, %FossilDHW, Household, ISR, %SF, %MF | 5.4.4 |
| Shower Timer | L_base, Showers per capita per day (SPCD), GPH, EPG_gas, %FossilDHW, GPM, L_timer, Household, Usage Factor, %SF, %MF | 5.4.9 |
| Temperature Setback Card | Overall heat transfer coefficient of tank, Surface area of storage tank, Tpre,Tpost, ISR, RE_gas | 5.4.6 |
| Switch Gasket | ADJ_RxAirsealing, ISR, Δthermsgasket, %FossilHeat, %SF, %MF | 5.6.1 |
| Outlet Gasket | ADJ_RxAirsealing, ISR, Δthermsgasket, %FossilHeat, %SF, %MF | 5.6.1 |
| V-Seal Weatherstripping Roll (17') | ADJRxAirsealing, ISR, %FossilHeat, Δtherms_wx, %SF, %MF | 5.6.1 |
| Closed-Cell Foam Weatherstripping Roll (17') | ADJRxAirsealing, ISR, %FossilHeat, Δtherms_wx, %SF, %MF | 5.6.1 |
| Window Insultation Kit (5-pack) | ADJRxAirsealing, ISR, %FossilHeat, Δtherms_windows, %SF, %MF | 5.6.1 |
| Rope Caulk (30') | ADJRxAirsealing, ISR, %FossilHeat, Δtherms_sealing, %SF, %MF | 5.6.1 |
| Self-Adhesive Door Sweep | ADJRxAirsealing, ISR, %FossilHeat, Δtherms_sweep, %SF, %MF | 5.6.1 |

# Appendix B. Program Specific Inputs for the Illinois TRC

The tables below show the Total Resource Cost (TRC) cost-effectiveness analysis inputs available at the time of producing this impact evaluation report. Additional required cost data (e.g., measure costs, program level incentive and non-incentive costs) are not included in these tables and will be provided to the evaluation team later. Guidehouse will include annual and lifetime water savings and greenhouse gas reductions in the end of year summary report.

Table B-1. Verified Cost Effectiveness Inputs - PGL

| Program Category | Program Path | Savings Category | DAC Project | Units | Quantity | Effective Useful Life | Early Replacement Flag† | Verified Gross Annual Water Savings (Gallons) | Ex Ante Gross Savings (Therms) | Verified Gross Savings (Therms) | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Non-Disadvantaged Communities | Market Rate | Low-Flow Showerhead (1.5 GPM) MR | FALSE | Each | 2,784 | 10.0 | NO | 3,476,704 | 17,982 | 17,982 | 19,474 |
|  |  | Kitchen Aerator (1.5 GPM) MR | FALSE | Each | 2,784 | 10.0 | NO | 553,278 | 2,391 | 2,391 | 2,590 |
|  |  | Bathroom Aerator (1.0 GPM) MR | FALSE | Each | 5,568 | 10.0 | NO | 796,146 | 2,890 | 2,890 | 3,130 |
|  |  | Shower Timer MR | FALSE | Each | 2,784 | 2.0 | NO | 1,976,142 | 11,247 | 11,247 | 12,181 |
|  |  | Temperature Setback Card MR | FALSE | Each | 2,784 | 2.0 | NO | 0 | 379 | 379 | 410 |
|  |  | Self-Adhesive Door Sweep MR | FALSE | Each | 2,784 | 20.0 | NO | 0 | 9,087 | 9,087 | 9,841 |
|  |  | Window Insultation Kit (5-pack) MR | FALSE | Each | 2,784 | 1.0 | NO | 0 | 17,066 | 17,066 | 18,482 |
|  |  | Closed-Cell Foam Weatherstripping Roll (17') MR | FALSE | Ln Ft | 2,784 | 20.0 | NO | 0 | 10,304 | 10,304 | 11,159 |
| Non-Disadvantaged Communities | Market Rate B | Closed-Foam Weatherstripping Roll (17') MRB | FALSE | Each | 62 | 10.0 | NO | 0 | 229 | 229 | 249 |
|  |  | Rope Caulk (30') MRB | FALSE | Each | 62 | 10.0 | NO | 0 | 346 | 346 | 375 |
|  |  | Self-Adhesive Door Sweep MRB | FALSE | Each | 31 | 10.0 | NO | 0 | 101 | 101 | 110 |
|  |  | Low-Flow Showerhead (1.5 GPM) MRB | FALSE | Each | 31 | 2.0 | NO | 0 | 200 | 200 | 217 |
|  |  | V-Seal Weatherstripping Roll (17') MRB | FALSE | Each | 31 | 2.0 | NO | 0 | 115 | 115 | 124 |
|  |  | Outlet Gasket MRB | FALSE | Each | 124 | 20.0 | NO | 0 | 21 | 21 | 23 |
|  |  | Switch Gasket MRB | FALSE | Each | 124 | 20.0 | NO | 0 | 21 | 21 | 23 |
|  |  | Bathroom Aerator (1 GPM) MRB | FALSE | Each | 31 | 20.0 | NO | 0 | 16 | 16 | 17 |
|  |  | Kitchen Aerator (1.5 GPM) MRB | FALSE | Ln Ft | 31 | 20.0 | NO | 0 | 27 | 27 | 29 |
|  |  | Temperature Setback Card MRB | FALSE | Each | 31 | 1.0 | NO | 0 | 7 | 7 | 8 |
|  |  | Shower Timer MRB | FALSE | Each | 31 | 20.0 | NO | 0 | 125 | 125 | 136 |
|  |  | Window Insulation Kit (Pack of 5) MRB | FALSE | Each | 31 | 20.0 | NO | 0 | 190 | 190 | 206 |
| Disadvantaged Communities | Market Rate | Low-Flow Showerhead (1.5 GPM) MR DAC | TRUE | Each | 1,012 | 10.0 | NO | 1,263,802 | 6,537 | 6,537 | 7,079 |
|  |  | Kitchen Aerator (1.5 GPM) MR DAC | TRUE | Each | 1,012 | 10.0 | NO | 201,120 | 869 | 869 | 941 |
|  |  | Bathroom Aerator (1.0 GPM) MR DAC | TRUE | Each | 2,024 | 10.0 | NO | 289,404 | 1,050 | 1,050 | 1,138 |
|  |  | Shower Timer MR DAC | TRUE | Each | 1,012 | 2.0 | NO | 718,339 | 4,088 | 4,088 | 4,428 |
|  |  | Temperature Setback Card MR DAC | TRUE | Each | 1,012 | 2.0 | NO | 0 | 138 | 138 | 149 |
|  |  | Self-Adhesive Door Sweep MR DAC | TRUE | Each | 1,012 | 20.0 | NO | 0 | 3,303 | 3,303 | 3,577 |
|  |  | Window Insultation Kit (5-pack) MR DAC | TRUE | Each | 1,012 | 1.0 | NO | 0 | 6,204 | 6,204 | 6,718 |
|  |  | Closed-Cell Foam Weatherstripping Roll (17') MR DAC | TRUE | Ln Ft | 1,012 | 20.0 | NO | 0 | 3,745 | 3,745 | 4,056 |
|  | Income Eligible | Low-Flow Showerhead (1.5 GPM) IE | FALSE | Each | 7,253 | 10.0 | NO | 9,654,077 | 49,930 | 49,930 | 49,930 |
|  |  | Kitchen Aerator (1.5 GPM) IE | FALSE | Each | 7,253 | 10.0 | NO | 1,536,338 | 6,644 | 6,644 | 6,644 |
|  |  | Bathroom Aerator (1.0 GPM) IE | FALSE | Each | 14,506 | 10.0 | NO | 2,210,729 | 8,022 | 8,022 | 8,022 |
|  |  | Shower Timer IE | FALSE | Each | 7,253 | 2.0 | NO | 5,487,337 | 26,836 | 26,836 | 26,836 |
|  |  | Temperature Setback Card IE | FALSE | Each | 7,253 | 2.0 | NO | 0 | 986 | 986 | 986 |
|  |  | Self-Adhesive Door Sweep IE | FALSE | Each | 7,253 | 20.0 | NO | 0 | 21,498 | 21,498 | 21,498 |
|  |  | Window Insultation Kit (5-pack) IE | FALSE | Each | 7,253 | 1.0 | NO | 0 | 40,363 | 40,363 | 40,363 |
|  |  | Closed-Cell Foam Weatherstripping Roll (17') IE | FALSE | Ln Ft | 7,253 | 20.0 | NO | 0 | 24,377 | 24,377 | 24,377 |
| Total or Weighted Average |  |  |  |  |  | 9.3 |  | 28,163,414 | 277,335 | 277,335 | 285,525 |

Source: Evaluation team analysis.

Table B-2. Verified Cost Effectiveness Inputs – NSG

| Program Category | Program Path | Savings Category | DAC Project | Units | Quantity | Effective Useful Life | Early Replacement Flag† | Verified Gross Annual Water Savings (Gallons) | Ex Ante Gross Savings (Therms) | Verified Gross Savings (Therms) | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Non-Disadvantaged Communities | Market Rate | Low-Flow Showerhead (1.5 GPM) MR | FALSE | Each | 1,456 | 10.0 | NO | 1,818,276 | 9,404 | 9,404 | 10,185 |
|  |  | Kitchen Aerator (1.5 GPM) MR | FALSE | Each | 1,456 | 10.0 | NO | 289,358 | 1,251 | 1,251 | 1,355 |
|  |  | Bathroom Aerator (1.0 GPM) MR | FALSE | Each | 2,912 | 10.0 | NO | 416,375 | 1,511 | 1,511 | 1,637 |
|  |  | Shower Timer MR | FALSE | Each | 1,456 | 2.0 | NO | 1,033,499 | 5,311 | 5,311 | 5,752 |
|  |  | Temperature Setback Card MR | FALSE | Each | 1,456 | 2.0 | NO | 0 | 198 | 198 | 214 |
|  |  | Self-Adhesive Door Sweep MR | FALSE | Each | 1,456 | 20.0 | NO | 0 | 4,752 | 4,752 | 5,147 |
|  |  | Window Insultation Kit (5-pack) MR | FALSE | Each | 1,456 | 1.0 | NO | 0 | 8,925 | 8,925 | 9,666 |
|  |  | Closed-Cell Foam Weatherstripping Roll (17') MR | FALSE | Ln Ft | 1,456 | 20.0 | NO | 0 | 5,389 | 5,389 | 5,836 |
| Disadvantaged Communities | Market Rate | Low-Flow Showerhead (1.5 GPM) MR DAC | TRUE | Each | 63 | 10.0 | NO | 78,675 | 407 | 407 | 441 |
|  |  | Kitchen Aerator (1.5 GPM) MR DAC | TRUE | Each | 63 | 10.0 | NO | 12,520 | 54 | 54 | 59 |
|  |  | Bathroom Aerator (1.0 GPM) MR DAC | TRUE | Each | 126 | 10.0 | NO | 18,016 | 65 | 65 | 71 |
|  |  | Shower Timer MR DAC | TRUE | Each | 63 | 2.0 | NO | 44,719 | 230 | 230 | 249 |
|  |  | Temperature Setback Card MR DAC | TRUE | Each | 63 | 2.0 | NO | 0 | 9 | 9 | 9 |
|  |  | Self-Adhesive Door Sweep MR DAC | TRUE | Each | 63 | 20.0 | NO | 0 | 206 | 206 | 223 |
|  |  | Window Insultation Kit (5-pack) MR DAC | TRUE | Each | 63 | 1.0 | NO | 0 | 386 | 386 | 418 |
|  |  | Closed-Cell Foam Weatherstripping Roll (17') MR DAC | TRUE | Ln Ft | 63 | 20.0 | NO | 0 | 233 | 233 | 253 |
|  | Income Eligible | Low-Flow Showerhead (1.5 GPM) IE | FALSE | Each | 897 | 10.0 | NO | 1,193,948 | 6,175 | 6,175 | 6,175 |
|  |  | Kitchen Aerator (1.5 GPM) IE | FALSE | Each | 897 | 10.0 | NO | 190,003 | 822 | 822 | 822 |
|  |  | Bathroom Aerator (1.0 GPM) IE | FALSE | Each | 1,794 | 10.0 | NO | 273,407 | 992 | 992 | 992 |
|  |  | Shower Timer IE | FALSE | Each | 897 | 2.0 | NO | 678,635 | 3,488 | 3,488 | 3,488 |
|  |  | Temperature Setback Card IE | FALSE | Each | 897 | 2.0 | NO | 0 | 122 | 122 | 122 |
|  |  | Self-Adhesive Door Sweep IE | FALSE | Each | 897 | 20.0 | NO | 0 | 2,659 | 2,659 | 2,659 |
|  |  | Window Insultation Kit (5-pack) IE | FALSE | Each | 897 | 1.0 | NO | 0 | 4,992 | 4,992 | 4,992 |
|  |  | Closed-Cell Foam Weatherstripping Roll (17') IE | FALSE | Ln Ft | 897 | 20.0 | NO | 0 | 3,015 | 3,015 | 3,015 |
| Total or Weighted Average |  |  |  |  |  | 9.3 |  | 6,047,434 | 60,596 | 60,595 | 63,777 |

Source: Evaluation team analysis.