# Home Energy Rebates Program Impact Evaluation Report

Energy Efficiency Plan: Program Year 2025

(01/01/2025-12/31/2025)

Prepared for:

Peoples Gas and North Shore Gas

DRAFT

March 27, 2026

Prepared by:

| Robert Hodge Guidehouse | Julia Kaplan Guidehouse |  |  |
| --- | --- | --- | --- |

Submitted to:

Peoples Gas

North Shore Gas

200 East Randolph Street

Chicago, IL 60601

Submitted by:

Guidehouse Inc.

167 N. Green Street

12th Floor

Chicago, IL 60607

Contact:

| Charles Maglione, Partner 202.481.7352 cmaglione@guidehouse.com | Jeff Erickson, Director 608.616.4962 jeff.erickson@guidehouse.com | Laura Agapay-Read, Associate Director 312.583.4178 laura.agapay.read@guidehouse.com |
| --- | --- | --- |

Charles Ampong, Associate Director

608.616.4922

charles.ampong@guidehouse.com

This report was prepared by Guidehouse for People Gas Light and Coke Company (PGL)and North Shore Gas(NSG) companies. The work presented in this report represents Guidehouse’s professional judgment based on the information available at the time this report was prepared. Use of this report by any other party for whatever purpose should not, and does not, absolve such party from using due diligence in verifying the report’s contents. Neither Guidehouse nor any of its subsidiaries or affiliates assumes any liability or duty of care to such parties and hereby disclaims any such liability.

Table of Contents

1	Introduction	1

2	Program Description	1

3	Program Savings Detail	3

4	Program Savings by Measure	4

5	Impact Analysis Findings and Recommendations	7

5.1	Impact Parameter Estimates	7

5.2	Findings and Recommendations	8

Appendix A. Impact Analysis Methodology	9

Appendix B. Program Specific Inputs for the Illinois TRC	10

Table of Tables, Figures, and Equations

Table 1. 2025 Volumetric Summary for PGL	1

Table 2. 2025 Installed Measure Quantities for PGL	1

Table 3. 2025 Volumetric Summary for NSG	2

Table 4. 2025 Installed Measure Quantities for NSG	2

Table 5. 2025 Annual Energy Savings Summary for PGL	3

Table 6. 2025 Annual Energy Savings Summary for NSG	3

Table 7. 2025 Annual Energy Savings by Measure for PGL	4

Table 8. 2025 Annual Energy Savings by Measure for NSG	6

Table 9. Verified Gross Savings Parameters	7

# Introduction

This report presents the results of the impact evaluation of the Peoples Gas (PGL) and North Shore Gas (NSG) 2025 Home Energy Rebates program. It presents a summary of the energy impacts for the total program and broken out by relevant measure and program structure details. The appendix presents the impact analysis methodology. Program year 2025 covers January 1 to December 31, 2025.

# Program Description

The Home Energy Rebate Program provides rebates to PGL and NSG residential customers who make certain energy-saving improvements to their homes. The program is composed of two distinct offerings: 1) replacement of HVAC and other equipment and 2) weatherization (Wx) improvements. The first offers incentives for customer-installed or contractor-installed energy efficient furnaces, boilers, water heaters, and programmable thermostats. The second offers incentives to trade ally-installed home insulation measures, such as air sealing and duct sealing. The program also works with ComEd to process rebates for advanced thermostats, and PGL and NSG reimbursed ComEd for the gas portion (“ComEd Marketplace”), removing the need for participants to complete separate rebate applications for natural gas and electricity.

The PGL program had 1,812 participants in 2025 and completed 1,959 projects as shown in the following table.

Table 1. 2025 Volumetric Summary for PGL

| Participation | ComEd Marketplace | HVAC | Weatherization | Total |
| --- | --- | --- | --- | --- |
| Participants * | 718 | 903 | 191 | 1,812 |
| Installed Projects † | 720 | 1,103 | 136 | 1,959 |
| Measure Types Installed ‡ | 1 | 9 | 6 | 16 |

* Participants are defined as the number of distinct account numbers with realized gas savings.

† Installed Projects represent the unique count of work orders completed.

‡ Measure types are defined as the number of distinct measure IDs with realized gas savings.

Source: Peoples Gas tracking data and evaluation team analysis.

Table 2 summarizes the installed measure quantities that are the basis for verified energy savings for PGL.

Table 2. 2025 Installed Measure Quantities for PGL

| Program Path | Measure | Quantity Unit | Installed Quantity |
| --- | --- | --- | --- |
| ComEd Marketplace | Smart Thermostat | Unit | 720 |
| HVAC | Boiler - DHW Two-In-One >=95% AFUE | Unit | 15 |
|  | Boiler - HW <300MBtu, >=95% AFUE | Unit | 25 |
|  | Boiler - Steam <=300MBH, >=82.5% AFUE | Unit | 1 |
|  | Furnace >95% <97% Afue | Unit | 398 |
|  | Furnace >= 97% Afue | Unit | 328 |
|  | Indirect Water Heater, >=88% Afue | Unit | 2 |
|  | Programmable Thermostat-Baseline Manual | Unit | 18 |
|  | Smart Thermostat | Unit | 360 |
|  | Tankless Water Heater>=0.95 UEF | Unit | 58 |
| Weatherization | Air Sealing with Attic Insulation | CFM* | 50,454 |
|  | Air Sealing | CFM* | 98,504 |
|  | Attic Insulation | SQFT | 122,217 |
|  | Duct Sealing | CFM* | 4,905 |
|  | Foundation Insulation | SQFT | 872 |
|  | Wall Insulation | SQFT | 3,733 |

Source: Peoples Gas tracking data and evaluation team analysis.

* CFM denotes cubic feet per minute.

The NSG program had 1,192 participants in 2025 and completed 1,275 projects as shown in the following table.

Table 3. 2025 Volumetric Summary for NSG

| Participation | ComEd Marketplace | HVAC | Weatherization | Total |
| --- | --- | --- | --- | --- |
| Participants * | 389 | 703 | 100 | 1,192 |
| Installed Projects † | 392 | 819 | 64 | 1,275 |
| Measure Types Installed ‡ | 1 | 10 | 6 | 17 |

* Participants are defined as the number of distinct account numbers with realized gas savings.

† Installed Projects represent the unique count of work orders completed.

‡ Measure types are defined as the number of distinct measure IDs with realized gas savings.

Source: North Shore Gas tracking data and Guidehouse evaluation team analysis.

Table 4 summarizes the installed measure quantities that are the basis for verified energy savings for NSG.

Table 4. 2025 Installed Measure Quantities for NSG

| Program Path | Measure | Quantity Unit | Installed Quantity |
| --- | --- | --- | --- |
| ComEd Marketplace | Smart Thermostat | Unit | 392 |
| HVAC | Boiler - DHW Two-In-One >=95% AFUE | Unit | 5 |
|  | Boiler - HW <300MBtu, 90 to 94.9% AFUE | Unit | 3 |
|  | Boiler - HW <300MBtu, >=95% AFUE | Unit | 9 |
|  | Boiler - Steam <=300MBH, >=82.5% AFUE | Unit | 3 |
|  | Furnace >95% <97% Afue | Unit | 275 |
|  | Furnace >= 97% Afue | Unit | 443 |
|  | Indirect Water Heater, >=88% Afue | Unit | 1 |
|  | Programmable Thermostat-Baseline Manual | Unit | 6 |
|  | Smart Thermostat | Unit | 176 |
|  | Tankless Water Heater>=0.95 UEF | Unit | 36 |
| Weatherization | Air Sealing with Attic Insulation | CFM* | 32,067 |
|  | Air Sealing | CFM* | 59,029 |
|  | Attic Insulation | SQFT | 74,320 |
|  | Duct Sealing | CFM* | 3,692 |
|  | Foundation Insulation | SQFT | 789 |
|  | Wall Insulation | SQFT | 150 |

Source: North Shore Gas tracking data and Guidehouse evaluation team analysis.

* CFM denotes cubic feet per minute.

# Program Savings Detail

Table 5 summarizes the energy savings the PGL Home Energy Rebates Program achieved by path in 2025.

Table 5. 2025 Annual Energy Savings Summary for PGL

| Program Category | Program Path | Ex Ante Gross Savings (Therms) | Verified Gross RR* | Verified Gross Savings (Therms) | NTG† | NPSO‡ | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Non-Disadvantaged Communities | ComEd Marketplace | 40,385 | 1.01 | 40,657 | 0.95 | 1.083 | 41,829 |
|  | HVAC | 164,898 | 1.00 | 164,853 | 0.76 | 1.083 | 135,436 |
|  | Weatherization | 24,293 | 1.00 | 24,292 | 0.88 | 1.083 | 23,193 |
| Non-DAC Subtotal |  | 229,575 | 1.00 | 229,802 |  |  | 200,458 |
| Disadvantaged Communities | ComEd Marketplace | 12,326 | 1.00 | 12,326 | 0.95 | 1.083 | 12,682 |
|  | HVAC | 36,075 | 1.00 | 36,075 | 1.00 | N/A | 36,075 |
|  | Weatherization | 7,242 | 1.00 | 7,242 | 1.00 | N/A | 7,242 |
| DAC Subtotal |  | 55,643 | 1.00 | 55,644 |  |  | 55,999 |
| Total |  | 285,219 | 1.00 | 285,445 |  |  | 256,458 |

* Realization Rate (RR) is the ratio of verified gross savings to ex ante gross savings, based on evaluation research findings.

† The program-level NTG value is calculated as Verified Net/Verified Gross. The program-level analysis typically used measure-level deemed NTG values (see Table 4), which are listed here: https://www.ilsag.info/evaluator-ntg-recommendations-for-2025.

‡ The market rate net savings were multiplied by a residential non-participant spillover (NPSO) factor of 1.083. Based on SAG Policy, if deemed NTG is multiplied by the NPSO and the resulted NTG value is less than 1.00, the evaluation assigned NTG of 1.00 to projects in DAC zones (NPSO is shown as “N/A”). If the resulted NTG value is >1.00, the evaluation used the >1.00 value for calculation of net savings impact.

Source: Evaluation team analysis.

Table 6 summarizes the energy savings the NSG Home Energy Rebates Program achieved by path in 2025.

Table 6. 2025 Annual Energy Savings Summary for NSG

| Program Category | Program Path | Ex Ante Gross Savings (Therms) | Verified Gross RR* | Verified Gross Savings (Therms) | NTG† | NSPO‡ | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Non-Disadvantaged Communities | ComEd Marketplace | 29,497 | 1.00 | 29,497 | 0.95 | 1.083 | 30,348 |
|  | HVAC | 179,144 | 1.00 | 179,145 | 0.75 | 1.083 | 145,778 |
|  | Weatherization | 18,690 | 1.00 | 18,690 | 0.88 | 1.083 | 17,848 |
| Non-DAC Subtotal |  | 227,332 | 1.00 | 227,333 |  |  | 193,974 |
| Disadvantaged Communities | ComEd Marketplace | 2,218 | 1.00 | 2,218 | 0.95 | 1.083 | 2,282 |
|  | HVAC | 5,257 | 1.00 | 5,241 | 1.00 | N/A | 5,241 |
|  | Weatherization | 690 | 1.00 | 690 | 1.00 | N/A | 690 |
| DAC Subtotal |  | 8,165 | 1.00 | 8,149 |  |  | 8,213 |
| Total |  | 235,497 | 1.00 | 235,482 |  |  | 202,187 |

* Realization Rate (RR) is the ratio of verified gross savings to ex ante gross savings, based on evaluation research findings.

† The program-level NTG value is calculated as Verified Net/Verified Gross. The program-level analysis typically used measure-level deemed NTG values (see Table 4), which are listed here: https://www.ilsag.info/evaluator-ntg-recommendations-for-2025.

‡ The market rate net savings were multiplied by a residential non-participant spillover (NPSO) factor of 1.083. Based on SAG Policy, if deemed NTG is multiplied by the NPSO and the resulted NTG value is less than 1.00, the evaluation assigned NTG of 1.00 to projects in DAC zones (NPSO is shown as “N/A”). If the resulted NTG value is >1.00, the evaluation used the >1.00 value for calculation of net savings impact.

Source: Evaluation team analysis.

# Program Savings by Measure

The program includes 16 measures as shown in the following table. The Furnaces and Smart Thermostats contributed the most savings.

Table 7. 2025 Annual Energy Savings by Measure for PGL

| Program Category | Program Path | Savings Category | Ex Ante Gross Savings (Therms) | Verified Gross RR* | Verified Gross Savings (Therms) | NTG† | NSPO‡ | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Non-Disadvantaged Communities | ComEd Marketplace | Smart Thermostat | 40,385 | 1.01 | 40,657 | 0.95 | 1.083 | 41,829 |
|  | HVAC | Boiler - DHW Two-In-One >=95% AFUE | 3,149 | 0.99 | 3,103 | 0.74 | 1.083 | 2,487 |
|  |  | Boiler - HW <300MBtu, >=95% AFUE | 3,757 | 1.00 | 3,757 | 0.74 | 1.083 | 3,011 |
|  |  | Furnace >95% <97% Afue | 66,891 | 1.00 | 66,892 | 0.74 | 1.083 | 53,609 |
|  |  | Furnace >= 97% Afue | 66,192 | 1.00 | 66,192 | 0.74 | 1.083 | 53,047 |
|  |  | Indirect Water Heater, >=88% Afue | 120 | 1.00 | 120 | 0.74 | 1.083 | 96.09 |
|  |  | Programmable Thermostat-Baseline Manual | 419 | 1.00 | 419 | 0.74 | 1.083 | 336 |
|  |  | Smart Thermostat | 20,433 | 1.00 | 20,434 | 0.89 | 1.083 | 19,695 |
|  |  | Tankless Water Heater>=0.95 UEF | 3,936 | 1.00 | 3,936 | 0.74 | 1.083 | 3,154 |
|  | Weatherization | Air Sealing with Attic Insulation | 2,596 | 1.00 | 2,596 | 0.88 | 1.083 | 2,474 |
|  |  | Air Sealing | 6,112 | 1.00 | 6,112 | 0.88 | 1.083 | 5,825 |
|  |  | Attic Insulation | 12,440 | 1.00 | 12,440 | 0.89 | 1.083 | 11,990 |
|  |  | Duct Sealing | 2,724 | 1.00 | 2,724 | 0.87 | 1.083 | 2,567 |
|  |  | Foundation Insulation | 47 | 1.00 | 47 | 0.74 | 1.083 | 37 |
|  |  | Wall Insulation | 374 | 1.00 | 374 | 0.74 | 1.083 | 300 |
| Non-DAC Subtotal |  |  | 229,575 | 1.00 | 229,802 |  |  | 200,458 |
| Disadvantaged Communities | ComEd Marketplace | Smart Thermostat | 12,326 | 1.00 | 12,326 | 0.95 | 1.083 | 12,682 |
|  | HVAC | Boiler - DHW Two-In-One >=95% AFUE | 787 | 1.00 | 787 | 1.00 | N/A | 787 |
|  |  | Boiler - HW <300MBtu, >=95% AFUE | 1,403 | 1.00 | 1,403 | 1.00 | N/A | 1,403 |
|  |  | Boiler - Steam <=300MBH, >=82.5% AFUE | 37 | 1.00 | 37 | 1.00 | N/A | 37 |
|  |  | Furnace >95% <97% Afue | 16,566 | 1.00 | 16,566 | 1.00 | N/A | 16,566 |
|  |  | Furnace >= 97% Afue | 11,905 | 1.00 | 11,905 | 1.00 | N/A | 11,905 |
|  |  | Programmable Thermostat-Baseline Manual | 150 | 1.00 | 150 | 1.00 | N/A | 150 |
|  |  | Smart Thermostat | 4,956 | 1.00 | 4,956 | 1.00 | N/A | 4,956 |
|  |  | Tankless Water Heater>=0.95 UEF | 271 | 1.00 | 271 | 1.00 | N/A | 271 |
|  | Weatherization | Air Sealing with Attic Insulation | 543 | 1.00 | 543 | 1.00 | N/A | 543 |
|  |  | Air Sealing | 1,952 | 1.00 | 1,952 | 1.00 | N/A | 1,952 |
|  |  | Attic Insulation | 3,877 | 1.00 | 3,877 | 1.00 | N/A | 3,877 |
|  |  | Duct Sealing | 808 | 1.00 | 808 | 1.00 | N/A | 808 |
|  |  | Foundation Insulation | 9 | 1.00 | 9 | 1.00 | N/A | 9 |
|  |  | Wall Insulation | 53 | 1.00 | 53 | 1.00 | N/A | 53 |
| DAC Subtotal |  |  | 55,643 | 1.00 | 55,644 |  |  | 55,999 |
| Total |  |  | 285,219 | 1.00 | 285,445 |  |  | 256,458 |

Source: Evaluation team analysis.

Table 8. 2025 Annual Energy Savings by Measure for NSG

| Program Category | Program Path | Savings Category | Ex Ante Gross Savings (Therms) | Verified Gross RR* | Verified Gross Savings (Therms) | NTG† | NSPO‡ | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Non-Disadvantaged Communities | ComEd Marketplace | Smart Thermostat | 29,497 | 1.00 | 29,497 | 0.95 | 1.083 | 30,348 |
|  | HVAC | Boiler - DHW Two-In-One >=95% AFUE | 1,050 | 1.00 | 1,050 | 0.74 | 1.083 | 841 |
|  |  | Boiler - HW <300MBtu, 90 to 94.9% AFUE | 511 | 1.00 | 511 | 0.74 | 1.083 | 409 |
|  |  | Boiler - HW <300MBtu, >=95% AFUE | 2,484 | 1.00 | 2,484 | 0.74 | 1.083 | 1,990 |
|  |  | Boiler - Steam <=300MBH, >=82.5% AFUE | 55 | 1.00 | 55 | 0.74 | 1.083 | 44 |
|  |  | Furnace >95% <97% Afue | 54,729 | 1.00 | 54,730 | 0.74 | 1.083 | 43,862 |
|  |  | Furnace >= 97% Afue | 103,812 | 1.00 | 103,811 | 0.74 | 1.083 | 83,196 |
|  |  | Indirect Water Heater, >=88% Afue | 67 | 1.00 | 67 | 0.74 | 1.083 | 53 |
|  |  | Programmable Thermostat-Baseline Manual | 225 | 1.00 | 225 | 0.74 | 1.083 | 180 |
|  |  | Smart Thermostat | 13,588 | 1.00 | 13,589 | 0.89 | 1.083 | 13,098 |
|  |  | Tankless Water Heater>=0.95 UEF | 2,625 | 1.00 | 2,625 | 0.74 | 1.083 | 2,103 |
|  | Weatherization | Air Sealing with Attic Insulation | 1,995 | 1.00 | 1,995 | 0.88 | 1.083 | 1,901 |
|  |  | Air Sealing | 4,567 | 1.00 | 4,567 | 0.88 | 1.083 | 4,352 |
|  |  | Attic Insulation | 8,044 | 1.00 | 8,044 | 0.89 | 1.083 | 7,754 |
|  |  | Duct Sealing | 4,028 | 1.00 | 4,028 | 0.87 | 1.083 | 3,795 |
|  |  | Foundation Insulation | 49 | 1.00 | 49 | 0.74 | 1.083 | 40 |
|  |  | Wall Insulation | 7 | 1.00 | 7 | 0.74 | 1.083 | 5 |
| Non-DAC Subtotal |  |  | 227,332 | 1.00 | 227,333 |  |  | 193,974 |
| Disadvantaged Communities | ComEd Marketplace | Smart Thermostat | 2,218 | 1.00 | 2,218 | 0.95 | 1.083 | 2,282 |
|  | HVAC | Boiler - DHW Two-In-One >=95% AFUE | 262 | 0.94 | 247 | 1.00 | N/A | 247 |
|  |  | Furnace >95% <97% Afue | 2,936 | 1.00 | 2,936 | 1.00 | N/A | 2,936 |
|  |  | Furnace >= 97% Afue | 1,667 | 1.00 | 1,667 | 1.00 | N/A | 1,667 |
|  |  | Smart Thermostat | 317 | 1.00 | 317 | 1.00 | N/A | 317 |
|  |  | Tankless Water Heater>=0.95 UEF | 75 | 1.00 | 75 | 1.00 | N/A | 75 |
|  | Weatherization | Air Sealing | 266 | 1.00 | 266 | 1.00 | N/A | 266 |
|  |  | Attic Insulation | 424 | 1.00 | 424 | 1.00 | N/A | 424 |
| DAC Subtotal |  |  | 8,165 | 1.00 | 8,149 |  |  | 8,213 |
| Total |  |  | 235,497 | 1.00 | 235,482 |  |  | 202,187 |

Source: Evaluation team analysis.

# Impact Analysis Findings and Recommendations

## Impact Parameter Estimates

Table 9 shows the unit therm savings and realization rate findings by measure from our review. The realization rate is the ratio of the verified savings to the ex ante savings. Following the table, we provide findings and recommendations, including discussion of all measures with realization rates above or below 100%. Appendix A provides a description of the impact analysis methodology.

Table 9. Verified Gross Savings Parameters

| Measure | Unit Basis | Ex Ante Gross (therms/unit) | Verified Gross (therms/unit) | Realization Rate | Il TRM v13.0 Section |
| --- | --- | --- | --- | --- | --- |
| Air Sealing with Attic Insulation | CFM | 0.06 | 0.06 | 100% | 5.6.1 |
| Air Sealing | CFM | 0.08 | 0.08 | 100% | 5.6.1 |
| Attic Insulation | SQFT | Varies | Varies | 100% | 5.6.5 |
| Boiler - DHW Two-In-One >=95% AFUE | Unit | 262.45 | 259.38 | 99% | 5.3.17 |
| Boiler - HW <300MBtu, 90 to 94.9% AFUE | Unit | 170.26 | 170.26 | 100% | 5.3.6 |
| Boiler - HW <300MBtu, >=95% AFUE | Unit | Varies | Varies | 100% | 5.3.6 |
| Boiler - Steam <=300MBH, >=82.5% AFUE | Unit | 18.36 | 18.36 | 100% | 5.3.6 |
| Duct Sealing | CFM | Varies | Varies | 100% | 5.3.4 |
| Foundation Insulation | SQFT | 0.06 | 0.06 | 100% | 5.6.6 |
| Furnace >95% <97% AFUE | Unit | 209.69 | 209.69 | 100% | 5.3.7 |
| Furnace >= 97% AFUE | Unit | 238.10 | 238.10 | 100% | 5.3.7 |
| Indirect Water Heater, >=88% AFUE | Unit | Varies | Varies | 100% | 5.4.2 |
| Programmable Thermostat-Baseline Manual | Unit | Varies | Varies | 100% | 5.3.11 |
| Smart Thermostat | Unit | Varies | Varies | 100% | 5.3.16 |
| Tankless Water Heater>=0.95 UEF | Unit | Varies | Varies | 100% | 5.4.2 |
| Wall Insulation | SQFT | Varies | Varies | 100% | 5.3.4 |

* State of Illinois Technical Reference Manual version 13.0 from http://www.ilsag.info/technical-reference-manual.html.

## Findings and Recommendations

Overall, the findings listed below had a minor impact on the savings, with the program-level realization rate being 100%.

Finding 1. Four Gas High Efficiency Combination Boiler records for multi-family units had a single-family household factor of 2.62 used in savings calculations. The affected project IDs are a0PUp00000RmD6fMAF, a0PUp00000GZukTMAT, a0PUp00000MJ2JBMA1, and a0PUp00000VD8QwMA.

Recommendation 1.  Guidehouse recommends selecting household factors based on unit type, consistent with IL TRM v13.

Finding 2. Four Smart Thermostat records from the ComEd Marketplace program path reported zero savings and were not identified as duplicate or unclaimed measures. Guidehouse included savings for these units in this evaluation. The affected work order IDs are COMED_97117, COMED_97845, COMED_97868, and COMED_SI_699256.

Recommendation 2.  Savings flags and claim indicators for Smart Thermostat records should be reviewed to ensure they accurately reflect eligibility and savings status. Correctly marking these records will reduce the potential for misclassification in reported results.

# Appendix A. Impact Analysis Methodology

The evaluation verified gross savings were determined for each program measure by:

Reviewing the savings algorithm inputs in the final 2025 tracking data for agreement with the IL-TRM v13.0 and IL-TRM Errata, where applicable.

Validating the savings algorithm was applied correctly.

Cross-checking per-unit savings values in the program tracking data with the verified values in Guidehouse’s calculations.

Multiplying the verified per-unit savings value by the quantity reported in the tracking data. The team calculated verified net savings by multiplying the verified gross savings estimates by an NTG ratio. In Program Year 2024, NTG estimates used to calculate the net verified savings were based on past evaluation research and defined by a consensus process through the Illinois SAG.

The market rate net savings were multiplied by a residential non-participant spillover (NPSO) factor of 1.083. Based on SAG Policy, if deemed NTG is multiplied by the NPSO and the resulted NTG value is less than 1.00, the evaluation assigned NTG of 1.00 to projects in DAC zones (NPSO is shown as “N/A”). If the resulted NTG value is >1.00, the evaluation used the >1.00 value for calculation of net savings impact.

# Appendix B. Program Specific Inputs for the Illinois TRC

The table below shows the Total Resource Cost (TRC) cost-effectiveness analysis inputs available at the time of producing this impact evaluation report. Additional required cost data (e.g., measure costs, program level incentive and non-incentive costs) are not included in this table and will be provided to the evaluation team later. Guidehouse will include annual and lifetime water savings and greenhouse gas reductions in the end of year summary report.

Table B-1. Verified Cost Effectiveness Inputs - PGL

| Program Path | Savings Category |  | DAC Project |  | Units |  | Quantity | Effective Useful Life | Early Replacement Flag† | Ex Ante Gross Savings (Therms) | Verified Gross Savings (Therms) | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ComEd Marketplace | Smart Thermostat |  | FALSE |  | Unit |  | 554 | 11.00 | NO | 40,385 | 40,657 | 41,829 |
|  | Smart Thermostat |  | TRUE |  | Unit |  | 166 | 11.00 | NO | 12,326 | 12,326 | 12,682 |
| HVAC | Boiler - DHW Two-In-One >=95% AFUE |  | FALSE |  | Unit |  | 12.00 | 21.50 | YES | 3,149 | 3,103 | 2,487 |
|  | Boiler - DHW Two-In-One >=95% AFUE |  | TRUE |  | Unit |  | 3.00 | 21.50 | YES | 787 | 787 | 787 |
|  | Boiler - HW <300MBtu, >=95% AFUE |  | FALSE |  | Unit |  | 18.00 | 25.00 | YES | 3,757 | 3,757 | 3,011 |
|  | Boiler - HW <300MBtu, >=95% AFUE |  | TRUE |  | Unit |  | 7.00 | 25.00 | YES | 1,403 | 1,403 | 1,403 |
|  | Boiler - Steam <=300MBH, >=82.5% AFUE |  | TRUE |  | Unit |  | 1.00 | 25.00 | YES | 36.71 | 36.71 | 36.71 |
|  | Furnace >95% <97% Afue |  | FALSE |  | Unit |  | 319 | 20.00 | YES | 66,891 | 66,892 | 53,609 |
|  | Furnace >95% <97% Afue |  | TRUE |  | Unit |  | 79.00 | 20.00 | YES | 16,566 | 16,566 | 16,566 |
|  | Furnace >= 97% Afue |  | FALSE |  | Unit |  | 278 | 20.00 | YES | 66,192 | 66,192 | 53,047 |
|  | Furnace >= 97% Afue |  | TRUE |  | Unit |  | 50.00 | 20.00 | YES | 11,905 | 11,905 | 11,905 |
|  | Indirect Water Heater, >=88% Afue |  | FALSE |  | Unit |  | 2.00 | 14.50 | NO | 120 | 120 | 96.09 |
|  | Programmable Thermostat-Baseline Manual |  | FALSE |  | Unit |  | 13.00 | 16.00 | NO | 419 | 419 | 336 |
|  | Programmable Thermostat-Baseline Manual |  | TRUE |  | Unit |  | 5.00 | 16.00 | NO | 150 | 150 | 150 |
|  | Smart Thermostat |  | FALSE |  | Unit |  | 295 | 11.00 | NO | 20,433 | 20,434 | 19,695 |
|  | Smart Thermostat |  | TRUE |  | Unit |  | 65.00 | 11.00 | NO | 4,956 | 4,956 | 4,956 |
|  | Tankless Water Heater>=0.95 UEF |  | FALSE |  | Unit |  | 54.00 | 20.00 | NO | 3,936 | 3,936 | 3,154 |
|  | Tankless Water Heater>=0.95 UEF |  | TRUE |  | Unit |  | 4.00 | 20.00 | NO | 271 | 271 | 271 |
| Weatherization | Air Sealing with Attic Insulation |  | FALSE |  | CFM |  | 41,729 | 20.00 | NO | 2,596 | 2,596 | 2,474 |
|  | Air Sealing with Attic Insulation |  | TRUE |  | CFM |  | 8,725 | 20.00 | NO | 543 | 543 | 543 |
|  | Air Sealing |  | FALSE |  | CFM |  | 74,661 | 20.00 | NO | 6,112 | 6,112 | 5,825 |
|  | Air Sealing |  | TRUE |  | CFM |  | 23,843 | 20.00 | NO | 1,952 | 1,952 | 1,952 |
|  | Attic Insulation |  | FALSE |  | SQFT |  | 92,966 | 30.00 | NO | 12,440 | 12,440 | 11,990 |
|  | Attic Insulation |  | TRUE |  | SQFT |  | 29,251 | 30.00 | NO | 3,877 | 3,877 | 3,877 |
|  | Duct Sealing |  | FALSE |  | SQFT |  | 4,093 | 20.00 | NO | 2,724 | 2,724 | 2,567 |
|  | Duct Sealing |  | TRUE |  | SQFT |  | 812 | 20.00 | NO | 808 | 808 | 808 |
|  | Foundation Insulation |  | FALSE |  | SQFT |  | 732 | 30.00 | NO | 46.66 | 46.67 | 37.40 |
|  | Foundation Insulation |  | TRUE |  | SQFT |  | 140 | 30.00 | NO | 9.22 | 9.22 | 9.22 |
|  | Wall Insulation |  | FALSE |  | SQFT |  | 3,293 | 30.00 | NO | 374 | 374 | 300 |
|  | Wall Insulation |  | TRUE |  | SQFT |  | 440 | 30.00 | NO | 53.33 | 53.33 | 53.33 |
| Total or Weighted Average |  |  |  |  |  |  |  | 18.22 |  | 285,219 | 285,445 | 256,458 |

Source: Evaluation team analysis.

Table B2. Verified Cost Effectiveness Inputs – NSG

| Program Path | Savings Category | DAC Project | Units | Quantity | Effective Useful Life | Early Replacement Flag† | Ex Ante Gross Savings (Therms) | Verified Gross Savings (Therms) | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ComEd Marketplace | Smart Thermostat | FALSE | Unit | 366 | 11.00 | NO | 29,497 | 29,497 | 30,348 |
|  | Smart Thermostat | TRUE | Unit | 26.00 | 11.00 | NO | 2,218 | 2,218 | 2,282 |
| HVAC | Boiler - DHW Two-In-One >=95% AFUE | FALSE | Unit | 4.00 | 21.50 | YES | 1,050 | 1,050 | 841 |
|  | Boiler - DHW Two-In-One >=95% AFUE | TRUE | Unit | 1.00 | 21.50 | YES | 262 | 247 | 247 |
|  | Boiler - HW <300MBtu, 90 to 94.9% AFUE | FALSE | Unit | 3.00 | 25.00 | YES | 511 | 511 | 409 |
|  | Boiler - HW <300MBtu, >=95% AFUE | FALSE | Unit | 9.00 | 25.00 | YES | 2,484 | 2,484 | 1,990 |
|  | Boiler - Steam <=300MBH, >=82.5% AFUE | FALSE | Unit | 3.00 | 25.00 | YES | 55.08 | 55.07 | 44.13 |
|  | Furnace >95% <97% Afue | FALSE | Unit | 261 | 20.00 | YES | 54,729 | 54,730 | 43,862 |
|  | Furnace >95% <97% Afue | TRUE | Unit | 14.00 | 20.00 | YES | 2,936 | 2,936 | 2,936 |
|  | Furnace >= 97% Afue | FALSE | Unit | 436 | 20.00 | YES | 103,812 | 103,811 | 83,196 |
|  | Furnace >= 97% Afue | TRUE | Unit | 7.00 | 20.00 | YES | 1,667 | 1,667 | 1,667 |
|  | Indirect Water Heater, >=88% Afue | FALSE | Unit | 1.00 | 14.50 | NO | 66.70 | 66.70 | 53.45 |
|  | Programmable Thermostat-Baseline Manual | FALSE | Unit | 6.00 | 16.00 | NO | 225 | 225 | 180 |
|  | Smart Thermostat | FALSE | Unit | 172 | 11.00 | NO | 13,588 | 13,589 | 13,098 |
|  | Smart Thermostat | TRUE | Unit | 4.00 | 11.00 | NO | 317 | 317 | 317 |
|  | Tankless Water Heater>=0.95 UEF | FALSE | Unit | 35.00 | 20.00 | NO | 2,625 | 2,625 | 2,103 |
|  | Tankless Water Heater>=0.95 UEF | TRUE | Unit | 1.00 | 20.00 | NO | 75.43 | 75.43 | 75.43 |
| Weatherization | Air Sealing with Attic Insulation | FALSE | CFM | 32,067 | 20.00 | NO | 1,995 | 1,995 | 1,901 |
|  | Air Sealing | FALSE | CFM | 55,784 | 20.00 | NO | 4,567 | 4,567 | 4,352 |
|  | Air Sealing | TRUE | CFM | 3,245 | 20.00 | NO | 266 | 266 | 266 |
|  | Attic Insulation | FALSE | SQFT | 71,290 | 30.00 | NO | 8,044 | 8,044 | 7,754 |
|  | Attic Insulation | TRUE | SQFT | 3,030 | 30.00 | NO | 424 | 424 | 424 |
|  | Duct Sealing | FALSE | CFM | 3,692 | 20.00 | NO | 4,028 | 4,028 | 3,795 |
|  | Foundation Insulation | FALSE | SQFT | 789 | 30.00 | NO | 49.41 | 49.42 | 39.61 |
|  | Wall Insulation | FALSE | SQFT | 150 | 30.00 | NO | 6.63 | 6.63 | 5.31 |
| Total or Weighted Average |  |  |  |  | 18.69 |  | 235,497 | 235,482 | 202,187 |

Source: Evaluation team analysis.