# Income Eligible Multi-Family and Public Housing Impact Evaluation Report

Energy Efficiency Plan: Program Year 2025

(01/01/2025-12/31/2025)

Prepared for:

Peoples Gas and North Shore Gas

DRAFT

March 13, 2026

Prepared by:

| Swapnil Lotake EcoMetric | Vishal Jagdale EcoMetric | Fahman Khan EcoMetric | Mike Frischmann EcoMetric |
| --- | --- | --- | --- |

Submitted to:

Peoples Gas

North Shore Gas

200 East Randolph Street

Chicago, IL 60601

Submitted by:

Guidehouse Inc.

167 N. Green Street

12th Floor

Chicago, IL 60607

Contact:

| Ed Balbis, Partner 561.644.9407 ebalbis@guidehouse.com | Jeff Erickson, Director 608.616.4962 jeff.erickson@guidehouse.com | Laura Agapay-Read, Associate Director 312.583.4178 laura.agapay.read@guidehouse.com |
| --- | --- | --- |

Charles Ampong, Associate Director

608.446.3172

charles.ampong@guidehouse.com

This report was prepared by Guidehouse for People Gas Light and Coke Company (PGL)and North Shore Gas(NSG) companies. The work presented in this report represents Guidehouse’s professional judgment based on the information available at the time this report was prepared. Use of this report by any other party for whatever purpose should not, and does not, absolve such party from using due diligence in verifying the report’s contents. Neither Guidehouse nor any of its subsidiaries or affiliates assumes any liability or duty of care to such parties, and hereby disclaims any such liability.

Table of Contents

1	Introduction	1

2	Multi-Family Income Eligible	1

2.1	Program Description	1

2.2	Program Savings Detail	4

2.3	Program Savings by Measure	4

2.4	Impact Analysis Findings and Recommendations	6

2.4.1	Impact Parameter Estimates	6

2.4.2	Findings and Recommendations	7

3	Public Housing Energy Savings Program	9

3.1	Program Description	9

3.2	Program Savings Detail	10

3.3	Program Savings by Measure	10

3.4	Impact Analysis Findings and Recommendations	11

3.4.1	Impact Parameter Estimates	11

3.4.2	Findings and Recommendations	11

Appendix A. Impact Analysis Methodology	12

Appendix B. Program Specific Inputs for the Illinois TRC	13

Table of Tables, Figures, and Equations

Table 1. 2025 Volumetric Summary for PGL – Income Eligible	1

Table 2. 2025 Installed Measure Quantities for PGL – Income Eligible	2

Table 3. 2025 Volumetric Summary for NSG – Income Eligible	3

Table 4. 2025 Installed Measure Quantities for NSG – Income Eligible	3

Table 5. 2025 Annual Energy Savings Summary for PGL – Income Eligible	4

Table 6. 2025 Annual Energy Savings Summary for NSG – Income Eligible	4

Table 7. 2025 Annual Energy Savings by Measure for PGL – Income Eligible	5

Table 8. 2025 Annual Energy Savings by Measure for NSG – Income Eligible	6

Table 9. Verified Gross Savings Parameters – Income Eligible	7

Table 10. Volumetric Summary for PGL – Public Housing	10

Table 11. Installed Measure Quantities for PGL – Public Housing	10

Table 12. Volumetric Summary for NSG – Public Housing	11

Table 13. Installed Measure Quantities for NSG – Public Housing	11

Table 14. Annual Energy Savings Summary for PGL – Public Housing	11

Table 15. Annual Energy Savings Summary for NSG – Public Housing	12

Table 16. Annual Energy Savings by Measure for PGL – Public Housing	12

Table 17. Annual Energy Savings by Measure for NSG – Public Housing	12

Table 18. Verified Gross Savings Parameters – Public Housing	13

# Introduction

This report presents the results of the impact evaluation of the Peoples Gas (PGL) and North Shore Gas (NSG) 2025 Income Eligible Multi-Family (MFIE) and the Public Housing (PHA) programs. It presents a summary of the energy impacts for the total program and is broken out by relevant measure and program structure details. Appendix A presents the impact analysis methodology. Appendix B presents the Total Resource Cost (TRC) cost-effectiveness analysis inputs. Program year 2025 covers January 1 to December 31, 2025.

# Multi-Family Income Eligible

## Program Description

The MFIE program provides energy efficiency upgrades to customers in the PGL and NSG territories with incomes below 80% of the Area Median Income (AMI) level as defined by the geography or location of the building and/or the subsidy status and rent levels of the building. The programs offer assessments, direct installation of energy efficiency measures, rebates for replacement of inefficient equipment, technical assistance, and educational information.

There are two program paths offered to multi-family income eligible customers.

- The Income Eligible Multi-Family Savings (Income Eligible) path, jointly implemented with ComEd.

- The Income Eligible Illinois Home Weatherization Assistance Program (IHWAP) path, jointly implemented with ComEd and Peoples Gas.

The PGL program had 459 participants in 2025 and completed 5,301 projects as shown in the following table.

Table 1. 2025 Volumetric Summary for PGL – Income Eligible

| Participation | Income Eligible | IHWAP | Total |
| --- | --- | --- | --- |
| Participants * | 457 | 2 | 459 |
| Installed Projects † | 5,295 | 6 | 5,301 |
| Measure Types Installed ‡ | 61 | 8 | 69 |

* Participants are defined as distinct count of gas account numbers.

† Installed Projects are defined as distinct count of project ID.

‡ Measure Types Installed are the distinct count of PGL measure names.

Source: Peoples Gas tracking data and evaluation team analysis.

Table 2 summarizes the installed measure quantities that are the basis for verified energy savings.

Table 2. 2025 Installed Measure Quantities for PGL – Income Eligible

| Program Path | Measure | Quantity Unit | Installed Quantity |
| --- | --- | --- | --- |
| Income Eligible | Advanced Thermostat | Unit | 10 |
|  | Air Sealing | Unit | 840 |
|  | Air Sealing | Linear feet | 9582 |
|  | Assessments/No Savings | Unit | 34865 |
|  | Assessments/No Savings | Linear feet | 556 |
|  | Assessments/No Savings | Square feet | 2996 |
|  | Attic Insulation | Square feet | 7,06,537 |
|  | Boiler Reset Controls | MBH Input | 2 |
|  | Boiler Tune Up | MBH Input | 98 |
|  | Controls for Domestic Hot Water | Living Units | 267 |
|  | Covers and Gap Sealers for Room AC | Unit | 1,125 |
|  | Duct Sealing | Linear feet | 396 |
|  | Exhaust Damper | MBH Input | 7,461 |
|  | Faucet Aerator – Bathroom (IU) | Unit | 1,800 |
|  | Faucet Aerator – Kitchen (CA) | Unit | 5 |
|  | Faucet Aerator – Kitchen (IU) | Unit | 3,061 |
|  | Furnace Filter | Unit | 434 |
|  | High Efficiency Boiler | MBH Input | 13 |
|  | High Efficiency Furnace | MBH Input | 1,291 |
|  | Low Flow Showerheads (CA) | Unit | 6 |
|  | Low Flow Showerheads (IU) | Unit | 3,453 |
|  | Pipe Insulation | Unit | 1632 |
|  | Pipe Insulation | Linear feet | 31192 |
|  | Programmable Thermostats | Unit | 454 |
|  | Reprogrammable Thermostat | Unit | 71 |
|  | Shower Timer | Unit | 4,750 |
|  | Steam Boiler Average Controls | Living Units | 757 |
|  | Steam Traps | Unit | 2,938 |
|  | Wall Insulation | Square feet | 252 |
| IHWAP | Air Sealing | Unit | 16,805 |
|  | Custom | Unit | 105 |
|  | Faucet Aerator – Bathroom (IU) | Unit | 72 |
|  | Faucet Aerator – Kitchen (IU) | Unit | 79 |
|  | Showerhead (IU) | Unit | 93 |

Source: Peoples Gas tracking data and evaluation team analysis.

The NSG program had 3 participants in 2025 and completed 4 projects as shown in the following table.

Table 3. 2025 Volumetric Summary for NSG – Income Eligible

| Participation | Income Eligible |
| --- | --- |
| Participants * | 3 |
| Installed Projects † | 4 |
| Measure Types Installed ‡ | 7 |

* Participants are defined as distinct count of gas account numbers.

† Installed Projects are defined as distinct count of project ID.

‡ Measure Types Installed are the distinct count of NSG measure names

Source: North Shore Gas tracking data and Guidehouse evaluation team analysis.

Table 4 summarizes the installed measure quantities that are the basis for verified energy savings.

Table 4. 2025 Installed Measure Quantities for NSG – Income Eligible

| Program Path | Measure | Quantity Unit | Installed Quantity |
| --- | --- | --- | --- |
| Income Eligible | Assessments/No Savings | Each | 5,016 |
|  | Attic Insulation | Square feet | 84,134 |
|  | Pipe Insulation | Linear feet | 613 |
|  | Wall Insulation | Square feet | 7,560 |

Source: North Shore Gas tracking data and Guidehouse evaluation team analysis

## Program Savings Detail

Table 5 summarizes the energy savings the PGL Multi-Family Income Eligible Program achieved by path in 2025.

Table 5. 2025 Annual Energy Savings Summary for PGL – Income Eligible

| Program Path | Ex Ante Gross Savings (Therms) | Verified Gross RR* | Verified Gross Savings (Therms | NTG† | NSPO | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- |
| Income Eligible | 1,600,826 | 102% | 1,626,361 | 1.00 | N/A | 1,626,361 |
| IHWAP | 50,220 | 58% | 29,282 | 1.00 | N/A | 29,282 |
| Total | 1,651,046 | 100% | 1,655,644 | 1.00 |  | 1,655,644 |

* Realization Rate (RR) is the ratio of verified gross savings to ex ante gross savings, based on evaluation research findings.

† NTG, Net to Gross is the deemed value available on the SAG website: https://www.ilsag.info/evaluator-ntg-recommendations-for-2025/

Source: Evaluation team analysis.

Table 6 summarizes the energy savings the NSG Multi-Family Income Eligible Program achieved by path in 2025.

Table 6. 2025 Annual Energy Savings Summary for NSG – Income Eligible

| Program Path | Ex Ante Gross Savings (Therms) | Verified Gross RR* | Verified Gross Savings (Therms | NTG† | NSPO | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- |
| Income Eligible | 32,849 | 100% | 32,982 | 1.00 | N/A | 32,982 |

* Realization Rate (RR) is the ratio of verified gross savings to ex ante gross savings, based on evaluation research findings.

† NTG, Net to Gross is the deemed value available on the SAG website: https://www.ilsag.info/evaluator-ntg-recommendations-for-2025/.

Source: Evaluation team analysis.

## Program Savings by Measure

The program includes 31 measures as shown in the following table. Steam Traps and Attic Insulation measures contributed the most savings.

Table 7. 2025 Annual Energy Savings by Measure for PGL – Income Eligible

| Program Path | Savings Category |  | Ex Ante Gross Savings (Therms) | Verified Gross RR* | Verified Gross Savings (Therms) | NTG | NSPO‡ | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Income Eligible | Advanced Thermostat |  | 565 | 100% | 565 | 1.00 | N/A | 565 |
|  | Air Sealing |  | 10,505 | 100% | 10,505 | 1.00 | N/A | 10,505 |
|  | Attic Insulation |  | 423,366 | - | 423,369 | 1.00 | N/A | 423,369 |
|  | Boiler Reset Controls |  | 3,495 | 100% | 3,495 | 1.00 | N/A | 3,495 |
|  | Boiler Tune Up |  | 111,689 | 100% | 111,689 | 1.00 | N/A | 111,689 |
|  | Controls for Domestic Hot Water |  | 16,741 | 100% | 16,741 | 1.00 | N/A | 16,741 |
|  | Covers and Gap Sealers for Room AC |  | 12,262 | 100% | 12,262 | 1.00 | N/A | 12,262 |
|  | Duct Sealing |  | 399 | 100% | 399 | 1.00 | N/A | 399 |
|  | Exhaust Damper |  | 1,230 | 100% | 1,230 | 1.00 | N/A | 1,230 |
|  | Faucet Aerator - Bathroom (IU) |  | 3,599 | 100% | 3,599 | 1.00 | N/A | 3,599 |
|  | Faucet Aerator - Kitchen (CA) |  | 41 | 100% | 41 | 1.00 | N/A | 41 |
|  | Faucet Aerator - Kitchen (IU) |  | 30,310 | 100% | 30,310 | 1.00 | N/A | 30,310 |
|  | Furnace Filter |  | 4,362 | 100% | 4,362 | 1.00 | N/A | 4,362 |
|  | High Efficiency Boiler |  | 19,986 | 100% | 19,986 | 1.00 | N/A | 19,986 |
|  | High Efficiency Furnace |  | 2,781 | 107% | 2,966 | 1.00 | N/A | 2,966 |
|  | Low Flow Showerheads (CA) |  | 130 | 100% | 130 | 1.00 | N/A | 130 |
|  | Low Flow Showerheads (IU) |  | 46,993 | 100% | 46,968 | 1.00 | N/A | 46,968 |
|  | Pipe Insulation |  | 301,275 | 100% | 301,265 | 1.00 | N/A | 301,265 |
|  | Programmable Thermostats |  | 18,388 | 100% | 18,388 | 1.00 | N/A | 18,388 |
|  | Reprogrammable Thermostat |  | 2,914 | 100% | 2,914 | 1.00 | N/A | 2,914 |
|  | Shower Timer |  | 19,736 | 100% | 19,737 | 1.00 | N/A | 19,737 |
|  | Steam Boiler Average Controls |  | 46,244 | 100% | 46,244 | 1.00 | N/A | 46,244 |
|  | Steam Traps |  | 523,789 | 105% | 549,166 | 1.00 | N/A | 549,166 |
|  | Wall Insulation |  | 27 | 118% | 32 | 1.00 | N/A | 32 |
|  | Income Eligible Subtotal |  | 1,600,826 | 102% | 1,626,361 | 1.00 |  | 1,626,361 |
| IHWAP | Air Sealing |  | 8,544 | 100% | 8,544 | 1.00 | N/A | 8,544 |
|  | Custom |  | 20,462 | 95% | 19,402 | 1.00 | N/A | 19,402 |
|  | Faucet Aerator - Bathroom (IU) |  | 5,884 | 2% | 144 | 1.00 | N/A | 144 |
|  | Faucet Aerator - Kitchen (IU) |  | 782 | 100% | 782 | 1.00 | N/A | 782 |
|  | Showerhead (IU) |  | 14,548 | 3% | 410 | 1.00 | N/A | 410 |
|  | IHWAP Subtotal |  | 50,220 | 58% | 29,282 | 1.00 |  | 29,282 |
| Total |  |  | 1,651,046 | 100% | 1,655,644 | 1.00 |  | 1,655,644 |

* Realization Rate (RR) is the ratio of verified gross savings to ex ante gross savings, based on evaluation research findings.

† NTG, Net to Gross is the deemed value available on the SAG website: https://www.ilsag.info/evaluator-ntg-recommendations-for-2025/.

Source: Evaluation team analysis.

Table 8. 2025 Annual Energy Savings by Measure for NSG – Income Eligible

| Program Path | Savings Category |  | Ex Ante Gross Savings (Therms) | Verified Gross RR* | Verified Gross Savings (Therms) | NTG† | NSPO | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Income Eligible | Attic Insulation |  | 28,743 | 100% | 28,743 | 1.00 | N/A | 28,743 |
|  | Pipe Insulation |  | 3,347 | 100% | 3,347 | 1.00 | N/A | 3,347 |
|  | Wall Insulation |  | 758 | 118% | 891 | 1.00 | N/A | 891 |
| Total |  |  | 32,849 | 100% | 32,982 | 1.00 |  | 32,982 |

* Realization Rate (RR) is the ratio of verified gross savings to ex ante gross savings, based on evaluation research findings.

† NTG, Net to Gross is the deemed value available on the SAG website: https://www.ilsag.info/evaluator-ntg-recommendations-for-2025/.

Source: Evaluation team analysis.

## Impact Analysis Findings and Recommendations

### Impact Parameter Estimates

Table 9 shows the unit Therm savings and realization rate findings by measure from our review. The realization rate is the ratio of the verified savings to the ex ante savings. Following the table, we provide findings and recommendations, including a discussion of all measures with realization rates above or below 100%. Appendix A provides a description of the impact analysis methodology.

Table 9. Verified Gross Savings Parameters – Income Eligible

| Measure | Unit Basis | Ex Ante Gross (therms/unit) | Verified Gross (therms/unit) | Realization Rate | Illinois TRM, v13.0†, |
| --- | --- | --- | --- | --- | --- |
| Advanced Thermostat | Each | Varies | Varies | 100% | 5.3.16 |
| Air Sealing | Each/Linear feet | Varies | Varies | 100% | 5.6.1 |
| Attic Insulation | Square feet | Varies | Varies | 100% | 5.6.5 |
| Boiler Reset Controls | MBH Input | Varies | Varies | 100% | 4.4.4 |
| Boiler Tune Up | MBH Input | Varies | Varies | 100% | 4.4.2 |
| Controls for Domestic Hot Water | Living Units | 62.7 | 62.7 | 100% | 4.3.8 |
| Covers and Gap Sealers for Room AC | Each | Varies | Varies | 100% | 4.4.38 |
| Custom | Each | Varies | Varies | 95% | 5.3.4 |
| Duct Sealing | Linear feet | 1.0067 | 1.0067 | 100% | 4.4.23 |
| Exhaust Damper | MBH Input | Varies | Varies | 100% | 5.4.4 |
| Faucet Aerator – Bathroom (IU) | Each | Varies | Varies | 39% | 4.3.2 |
| Faucet Aerator – Kitchen (CA) | Each | 8.150 | 8.150 | 100% | 5.4.4 |
| Faucet Aerator – Kitchen (IU) | Each | 9.9 | 9.9 | 100% | 5.3.21 |
| Furnace Filter | Each | 10.05 | 10.05 | 100% | 4.4.10 |
| High Efficiency Boiler | MBH Input | Varies | Varies | 100% | 5.3.7 |
| High Efficiency Furnace | MBH Input | 2.154 | 2.298 | 107% | 4.3.3 |
| Low Flow Showerheads (CA) | Each | 21.63 | 21.63 | 100% | 5.4.5 |
| Low Flow Showerheads (IU) | Each | 13.605 | Varies | 100% | 4.4.14 |
| Pipe Insulation | Each/Linear feet | Varies | Varies | 100% | 5.3.11 |
| Programmable Thermostats | Each | 40.502 | 40.502 | 100% | 5.4.4 |
| Reprogrammable Thermostat | Each | Varies | Varies | 100% | 5.4.9 |
| Shower Timer | Each | Varies | Varies | 100% | 5.4.9 |
| Steam Boiler Average Controls | Living Units | 61.08 | 61.08 | 100% | 4.4.36 |
| Steam Traps | Each | Varies | Varies | 105% | 4.4.36 |
| Wall Insulation | Square feet | Varies | Varies | 118% | 4.4.16 |

† State of Illinois Technical Reference Manual version 13.0 from http://www.ilsag.info/technical-reference-manual.html.
Source: Program tracking data and evaluation team analysis.

### Findings and Recommendations

The evaluation team developed findings and recommendations based on the 2025 evaluation, organized by path type in the following sections.

#### Income Eligible

Finding 1. For all instances of Furnace >95 AFUE-Joint MF measure, ex ante calculations used Baseline Furnace Annual Fuel Utilization Efficiency Rating (AFUEbase) as 81%. The evaluation team updated AFUEbase to 80%, consistent with section 5.3.7 of IL-TRM v13.0. This measure accounts for 0.17% of Income Eligible component’s reported gross Therms savings and the evaluation adjustment resulted in a realization rate of 107%.

Recommendation 1.  Use the baseline furnace AFUE from Section 5.3.7 of IL-TRM v13.0.

Finding 2. For one measure instance of High-Rise - F&T/Inverted Bucket Steam Traps - HVAC - Audit-Joint MF (WO-7402555), ex ante calculations applied an 80% Boiler Efficiency applicable to boilers in commercial and industrial buildings. The evaluation team updated the Boiler Efficiency to 64.8%, applicable to boilers in multifamily buildings, consistent with section 4.4.16 of IL-TRM v13.0. This measure accounts for 33% of Income Eligible component’s reported gross Therms savings and the evaluation adjustments from Findings 2 and 3 resulted in a realization rate of 105%.

Recommendation 2.   Use the 64.8% boiler efficiency, rather than 80%, to ensure alignment with IL‑TRM v13.0, where 64.8% is specified for multifamily low‑pressure steam boilers

Finding 3. For both instances of Wall Insulation-Joint MF and one instance of High-Rise – Radiator Steam Traps – HVAC – Audit-Joint MF measures, the evaluation could not replicate ex ante savings. The evaluation team calculated verified savings using inputs reported in tracking data and default values from IL-TRM v13.0. Steam Trap and Wall Insulation measures account for 33% and 0.002% of Income Eligible component’s reported gross Therms savings and the evaluation adjustments from Findings 2 and 3 resulted in a realization rate of 105% and 118%, respectively.

Recommendation 3. Ensure that inputs used in savings calculations align with inputs reported in the tracking data.

#### Illinois Home Weatherization Assistance Program (IHWAP)

Finding 4. For one instance of Bathroom Aerator-IHWAP-Joint MF and Showerhead-IHWAP-Joint MF measures (WO-7080492), the tracking data reported quantities of 77 and 69, respectively. However, the reported total savings in the tracking data correspond to a quantity of 2,940 for the Bathroom Aerator and 3,281 for the Showerhead. The evaluation team used the quantities reported in the tracking data to calculate total verified savings. Bathroom Aerator and Showerhead measures account for 12% and 29% of IHWAP component’s reported gross Therms savings, respectively and the evaluation adjustment resulted in a realization rate of 2%.

Recommendation 4. Ensure that measure quantities reported in the tracking data are used to calculate total savings.

Finding 5. For two measure instances of Custom Measure-IHWAP-Joint MF (WO-7077355 and WO-7521699), ex ante calculations used Equivalent Full Load Hours (EFLH) as 1,661 hours, which is an average value for MF Mid-Rise and MF High-Rise building types. The evaluation team determined that the building is a High-Rise building based on the project location. Hence, the team updated EFLH to 1,540 hours, corresponding to MF High-Rise building type. This measure accounts for 41% of IHWAP component’s reported gross Therms savings and the evaluation adjustments resulted in a realization rate of 95%.

Recommendation 5.  Use EFLH values consistent with the actual building type as provided in the tracking data.

# Public Housing Energy Savings Program

## Program Description

The Public Housing Energy Savings Program (PHES) works with Public Housing Authorities (PHAs) in ComEd, Nicor Gas, PGL, and NSG territories to achieve electricity and natural gas savings. The PHA is the program participant, though the residents of the properties are directly affected by the program through in-unit and common area upgrades. Gas savings opportunities for NSG included Boiler Tune Up. Gas savings opportunities for PGL included Covers and Gap Sealers for Room AC.

The PGL PHES program had 3 participants in 2025, completed 3 projects, and installed one measure type, as shown in Table 10.

Table 10. Volumetric Summary for PGL – Public Housing

| Participation | Public Housing |
| --- | --- |
| Participants * | 3 |
| Installed Projects † | 3 |
| Measure Types Installed ‡ | 1 |

* Participants are defined as distinct count of account numbers.

† Installed Projects are defined as distinct count of project IDs.

‡ Measure Types Installed are the distinct count of PGL measure names.

Source: Peoples Gas tracking data and evaluation team analysis.

Table 11 summarizes the installed measure quantities that are the basis for verified energy savings for PGL.

Table 11. Installed Measure Quantities for PGL – Public Housing

| Program Category | Program Path | Measure | Quantity Unit | Installed Quantity |
| --- | --- | --- | --- | --- |
| Private | Public Housing | Covers and Gap Sealers for Room AC | Each | 1,514 |

Source: Peoples Gas tracking data and evaluation team analysis.

The NSG PHES program had 1 participant in 2025, completed 1 project, and installed 1 measure type, as shown in Table 12.

Table 12. Volumetric Summary for NSG – Public Housing

| Participation | Public Housing |
| --- | --- |
| Participants * | 1 |
| Installed Projects † | 1 |
| Measure Types Installed ‡ | 1 |

* Participants are defined as distinct count of account numbers.

† Installed Projects are defined as distinct count of project IDs.

‡ Measure Types Installed are the distinct count of NSG measure names.

Source: North Shore Gas tracking data and Guidehouse evaluation team analysis.

Table 13 summarizes the installed measure quantities that are the basis for verified energy savings for NSG.

Table 13. Installed Measure Quantities for NSG – Public Housing

| Program Category | Program Path | Measure | Quantity Unit | Installed Quantity |
| --- | --- | --- | --- | --- |
| Public | Public Housing | Boiler Tune Up | MBH Input | 6 |

Source: North Shore Gas tracking data and Guidehouse evaluation team analysis.

## Program Savings Detail

Table 14 summarizes the energy savings the PGL PHES program achieved in 2025.

Table 14. Annual Energy Savings Summary for PGL – Public Housing

| Program Path | Ex Ante Gross Savings (Therms) | Verified Gross RR* | Verified Gross Savings (Therms) | NTG† | NSPO | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- |
| Public Housing | 9,173 | 100% | 9,173 | 1.00 | N/A | 9,173 |

* Realization Rate (RR) is the ratio of verified gross savings to ex ante gross savings, based on evaluation research findings.

† NTG, Net to Gross is the deemed value available on the SAG website: https://www.ilsag.info/evaluator-ntg-recommendations-for-2025/.

Source: Evaluation team analysis.

Table 15 summarizes the energy savings the NSG PHES program achieved in 2025.

Table 15. Annual Energy Savings Summary for NSG – Public Housing

| Program Path | Ex Ante Gross Savings (Therms) | Verified Gross RR* | Verified Gross Savings (Therms) | NTG† | NSPO | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- |
| Public Housing | 1,117 | 100% | 1,117 | 1.00 | N/A | 1,117 |

* Realization Rate (RR) is the ratio of verified gross savings to ex ante gross savings, based on evaluation research findings.

† NTG, Net to Gross is the deemed value available on the SAG website: https://www.ilsag.info/evaluator-ntg-recommendations-for-2025/.

Source: Evaluation team analysis.

## Program Savings by Measure

The PGL PHES program includes one measure, as shown in Table 16.

Table 16. Annual Energy Savings by Measure for PGL – Public Housing

| Program Path | Savings Category | Ex Ante Gross Savings (Therms) | Verified Gross RR* | Verified Gross Savings (Therms) | NTG† | NSPO | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Public Housing | Covers and Gap Sealers for Room AC | 9,173 | 100% | 9,173 | 1.00 | N/A | 9,173 |

* Realization Rate (RR) is the ratio of verified gross savings to ex ante gross savings, based on evaluation research findings.

† NTG, Net to Gross is the deemed value available on the SAG website: https://www.ilsag.info/evaluator-ntg-recommendations-for-2025/.

Source: Evaluation team analysis.

The NSG PHES program includes one measure, as shown in Table 17.

Table 17. Annual Energy Savings by Measure for NSG – Public Housing

| Program Path | Savings Category | Ex Ante Gross Savings (Therms) | Verified Gross RR* | Verified Gross Savings (Therms) | NTG† | NSPO | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Public Housing | Boiler Tune Up | 1,117 | 100% | 1,117 | 1.00 | N/A | 1,117 |

* Realization Rate (RR) is the ratio of verified gross savings to ex ante gross savings, based on evaluation research findings.

† NTG, Net to Gross is the deemed value available on the SAG website: https://www.ilsag.info/evaluator-ntg-recommendations-for-2025/.

Source: Evaluation team analysis.

## Impact Analysis Findings and Recommendations

### Impact Parameter Estimates

Table 18 shows the per unit Therm savings and realization rate findings by measure from our review. The realization rate is the ratio of the verified savings to the ex ante savings. Appendix A provides a description of the impact analysis methodology.

Table 18. Verified Gross Savings Parameters – Public Housing

| Measure | Unit Basis | Ex Ante Gross (therms/unit) | Verified Gross (therms/unit) | Realization Rate | Illinois TRM, v13.0†, Section |
| --- | --- | --- | --- | --- | --- |
| Boiler Tune Up | MBH Input | Varies | Varies | 100% | 4.4.2 |
| Covers and Gap Sealers for Room AC | Each | Varies | Varies | 100% | 4.4.38 |

† State of Illinois Technical Reference Manual version 13.0 from http://www.ilsag.info/technical-reference-manual.html.
Source: Program tracking data and evaluation team analysis.

### Findings and Recommendations

There are no findings and recommendations for the Public Housing program. The ex ante calculations are consistent with the IL-TRM v13.0.

# Appendix A. Impact Analysis Methodology

The evaluation team used the same impact methodology for each component. Verified gross savings were determined for each program measure by:

- Reviewing the savings algorithm inputs in the measure workbook for agreement with the IL-TRM v13.0 and IL-TRM Errata, where applicable.

- Validating the savings algorithm was applied correctly.

- Cross-checking per-unit savings values in the program tracking data with the verified values in the measure workbook or Guidehouse’s calculations if the workbook did not agree with the IL-TRM v13.0.

- Multiplying the verified per-unit savings value by the quantity reported in the tracking data. The team calculated verified net savings by multiplying the verified gross savings estimates by an NTG ratio. In Program Year 2025, NTG estimates used to calculate the net verified savings were based on past evaluation research and defined by a consensus process through the Illinois SAG.

- Guidehouse sourced methodologies and assumptions from the IL-TRM v13.0 and the final 2025 tracking data.

# Appendix B. Program Specific Inputs for the Illinois TRC

The tables below shows the Total Resource Cost (TRC) cost-effectiveness analysis inputs available at the time of producing this impact evaluation report. Additional required cost data (e.g., measure costs, program level incentive and non-incentive costs) are not included in these tables and will be provided to the evaluation team later. Guidehouse will include annual and lifetime water savings and greenhouse gas reductions in the end of year summary report.

Table B-1. Verified Cost Effectiveness Inputs – PGL (Income Eligible)

| Program Path | Savings Category | DAC Project* |  | Units |  | Quantity |  | Effective Useful Life |  | Early Replacement Flag |  | Verified Gross Annual Water Savings (Gallons) |  | Ex Ante Gross Savings (Therms) |  | Verified Gross Savings (Therms) |  | Verified Net Savings (Therms) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Income Eligible | Advanced Thermostat | FALSE |  | Each |  | 10 |  | 11.0 |  | NO |  | - |  | 565 |  | 565 |  | 565 |  |
|  | Air Sealing | FALSE |  | Each |  | 840 |  | 20.0 |  | NO |  | - |  | 6,519 |  | 6,519 |  | 6,519 |  |
|  | Air Sealing | FALSE |  | Linear Feet |  | 9,582 |  | 20.0 |  | NO |  | - |  | 3,986 |  | 3,986 |  | 3,986 |  |
|  | Attic Insulation | FALSE |  | Each |  | 706,537 |  | 30.0 |  | NO |  | - |  | 423,366 |  | 423,369 |  | 423,369 |  |
|  | Boiler Reset Controls | FALSE |  | MBH Input |  | 2 |  | 16.0 |  | NO |  | - |  | 3,495 |  | 3,495 |  | 3,495 |  |
|  | Boiler Tune Up | FALSE |  | MBH Input |  | 98 |  | 3.0 |  | NO |  | - |  | 111,689 |  | 111,689 |  | 111,689 |  |
|  | Controls for Domestic Hot Water | FALSE |  | Living Units |  | 267 |  | 15.0 |  | NO |  | - |  | 16,741 |  | 16,741 |  | 16,741 |  |
|  | Covers and Gap Sealers for Room AC | FALSE |  | Each |  | 1,125 |  | 5.0 |  | NO |  | - |  | 12,262 |  | 12,262 |  | 12,262 |  |
|  | Duct Sealing | FALSE |  | Linear feet |  | 396 |  | 20.0 |  | NO |  | - |  | 399 |  | 399 |  | 399 |  |
|  | Exhaust Damper | FALSE |  | MBH Input |  | 7,461 |  | 15.0 |  | NO |  | - |  | 1,230 |  | 1,230 |  | 1,230 |  |
|  | Faucet Aerator - Bathroom (IU) | FALSE |  | Each |  | 1,800 |  | 10.0 |  | NO |  | 817,932 |  | 3,599 |  | 3,599 |  | 3,599 |  |
|  | Faucet Aerator - Kitchen (CA) | FALSE |  | Each |  | 5 |  | 10.0 |  | NO |  | 7,689 |  | 41 |  | 41 |  | 41 |  |
|  | Faucet Aerator - Kitchen (IU) | FALSE |  | Each |  | 3,061 |  | 10.0 |  | NO |  | 5,718,836 |  | 30,310 |  | 30,310 |  | 30,310 |  |
|  | Furnace Filter | FALSE |  | Each |  | 434 |  | 3.0 |  | NO |  | - |  | 4,362 |  | 4,362 |  | 4,362 |  |
|  | High Efficiency Boiler | FALSE |  | MBH Input |  | 13 |  | 25.0 |  | NO |  | - |  | 19,986 |  | 19,986 |  | 19,986 |  |
|  | High Efficiency Furnace | FALSE |  | MBH Input |  | 1,291 |  | 20.0 |  | NO |  | - |  | 2,781 |  | 2,966 |  | 2,966 |  |
|  | Low Flow Showerheads (CA) | FALSE |  | Each |  | 6 |  | 10.0 |  | NO |  | 20,605 |  | 130 |  | 130 |  | 130 |  |
|  | Low Flow Showerheads (IU) | FALSE |  | Each |  | 3,453 |  | 10.0 |  | NO |  | 7,457,207 |  | 46,993 |  | 46,968 |  | 46,968 |  |
|  | Pipe Insulation | FALSE |  | Each |  | 1632 |  | 10.0 |  | NO |  | - |  | 44,659 |  | 44,653 |  | 44,653 |  |
|  | Pipe Insulation | FALSE |  | Linear feet |  | 31192 |  | 10.0 |  | NO |  | - |  | 256,616 |  | 256,612 |  | 256,612 |  |
|  | Programmable Thermostats | FALSE |  | Each |  | 454 |  | 16.0 |  | NO |  | - |  | 18,388 |  | 18,388 |  | 18,388 |  |
|  | Reprogrammable Thermostat | FALSE |  | Each |  | 71 |  | 2.0 |  | NO |  | - |  | 2,914 |  | 2,914 |  | 2,914 |  |
|  | Shower Timer | FALSE |  | Each |  | 4,750 |  | 2.0 |  | NO |  | 19,737 |  | 19,736 |  | 19,737 |  | 19,737 |  |
|  | Steam Boiler Average Controls | FALSE |  | Living Units |  | 757 |  | 20.0 |  | NO |  | - |  | 46,244 |  | 46,244 |  | 46,244 |  |
|  | Steam Traps | FALSE |  | Each |  | 2,938 |  | 6.0 |  | NO |  | 3,879,684 |  | 523,789 |  | 549,166 |  | 549,166 |  |
|  | Wall Insulation | FALSE |  | Square feet |  | 252 |  | 20.0 |  | NO |  | - |  | 27 |  | 32 |  | 32 |  |
| IHWAP | Air Sealing | FALSE |  | Each |  | 16,805 |  | 20.0 |  | NO |  | - |  | 8,544 |  | 8,544 |  | 8,544 |  |
|  | Custom | FALSE |  | Each |  | 105 |  | 22.8 |  | NO |  | - |  | 20,462 |  | 19,402 |  | 19,402 |  |
|  | Faucet Aerator - Bathroom (IU) | FALSE |  | Each |  | 72 |  | 10.0 |  | NO |  | 32,717 |  | 5,884 |  | 144 |  | 144 |  |
|  | Faucet Aerator - Kitchen (IU) | FALSE |  | Each |  | 79 |  | 10.0 |  | NO |  | 147,595 |  | 782 |  | 782 |  | 782 |  |
|  | Showerhead (IU) | FALSE |  | Each |  | 93 |  | 10.0 |  | NO |  | 65,139 |  | 14,548 |  | 410 |  | 410 |  |
| Total or Weighted Average |  |  |  |  |  |  |  |  | 14.03 |  |  |  | 18,167,140 |  | 1,651,046 |  | 1,655,644 |  | 1,655,644 |  |

Source: Evaluation team analysis.

Table B-2. Verified Cost Effectiveness Inputs – NSG (Income Eligible)

| Program Path | Savings Category | DAC Project* | Units | Quantity | Effective Useful Life | Early Replacement Flag | Verified Gross Annual Water Savings (Gallons) | Ex Ante Gross Savings (Therms) | Verified Gross Savings (Therms) | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Income Eligible | Attic Insulation | FALSE | Square feet | 84,134 | 30.0 | NO | - | 28,743 | 28,743 | 28,743 |
|  | Pipe Insulation | FALSE | Linear feet | 613 | 10.0 | NO | - | 3,347 | 3,347 | 3,347 |
|  | Wall Insulation | FALSE | Square feet | 7,560 | 20.0 | NO | - | 758 | 891 | 891 |
| Total or Weighted Average |  |  |  |  | 27.7 |  |  | 32,849 | 32,982 | 32,982 |

Source: Evaluation team analysis.

Table B-3. Verified Cost Effectiveness Inputs – PGL (Public Housing)

| Program Path | Savings Category | DAC Project* | Units | Quantity | Effective Useful Life | Early Replacement Flag | Verified Gross Annual Water Savings (Gallons) | Ex Ante Gross Savings (Therms) | Verified Gross Savings (Therms) | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Public Housing | Covers and Gap Sealers for Room AC | TRUE | Each | 1,514 | 5.0 | NO | - | 9,173 | 9,173 | 9,173 |

Source: Evaluation team analysis.

Table B-4. Verified Cost Effectiveness Inputs – NSG (Public Housing)

| Program Path | Savings Category | DAC Project* | Units | Quantity | Effective Useful Life | Early Replacement Flag | Verified Gross Annual Water Savings (Gallons) | Ex Ante Gross Savings (Therms) | Verified Gross Savings (Therms) | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Public Housing | Boiler Tune Up | FALSE | Unit | 6 | 3.0 | NO | - | 1,117 | 1,117 | 1,117 |

Source: Evaluation team analysis.