# Multi-Family Market Rate Program Impact Evaluation Report

Energy Efficiency Plan: Program Year 2025

(01/01/2025-12/31/2025)

Prepared for:

Peoples Gas and North Shore Gas

DRAFT

March 27, 2026

Prepared by:

| Aditya Khasnis Vishal Jagdale EcoMetric | Swapnil Lotake EcoMetric | Fahman Khan EcoMetric | Mike Frischmann EcoMetric |
| --- | --- | --- | --- |

Submitted to:

Peoples Gas

North Shore Gas

200 East Randolph Street

Chicago, IL 60601

Submitted by:

Guidehouse Inc.

167 N. Green Street

12th Floor

Chicago, IL 60607

Contact:

| Ed Balbis, Partner 561.644.9407 ebalbis@guidehouse.com | Jeff Erickson, Director 608.616.4962 jeff.erickson@guidehouse.com | Laura Agapay-Read, Associate Director 312.583.4178 laura.agapay.read@guidehouse.com |
| --- | --- | --- |

Charles Ampong, Associate Director

608.446.3172

charles.ampong@guidehouse.com

This report was prepared by Guidehouse for People Gas Light and Coke Company (PGL)and North Shore Gas (NSG) companies. The work presented in this report represents Guidehouse’s professional judgment based on the information available at the time this report was prepared. Use of this report by any other party for whatever purpose should not, and does not, absolve such party from using due diligence in verifying the report’s contents. Neither Guidehouse nor any of its subsidiaries or affiliates assumes any liability or duty of care to such parties and hereby disclaims any such liability.

Table of Contents

1	Introduction	1

2	Program Description	1

3	Program Savings Detail	4

4	Program Savings by Measure	6

5	Impact Analysis Findings and Recommendations	9

5.1	Impact Parameter Estimates	9

5.2	Findings and Recommendations	11

5.2.1	Cross-Cutting	11

5.2.2	Prescriptive	11

5.2.3	Partner Trade Ally	12

5.2.4	Custom	13

Appendix A. Impact Analysis Methodology	16

Appendix B. Program Specific Inputs for the Illinois TRC	19

Table of Tables

Table 1. 2025 Volumetric Summary for PGL	2

Table 2. 2025 Installed Measure Quantities for PGL	2

Table 3. 2025 Volumetric Summary for NSG	4

Table 4. 2025 Installed Measure Quantities for NSG	4

Table 5. 2025 Annual Energy Savings Summary for PGL	5

Table 6. 2025 Annual Energy Savings Summary for NSG	5

Table 7. 2025 Annual Energy Savings by Measure for PGL	6

Table 8. 2025 Annual Energy Savings by Measure for NSG	8

Table 9. Verified Gross Savings Parameters	9

Table 10. Instances with EFLH discrepancy	12

Table 11. Balance Point Temperature discrepancy – Custom measure	13

Table 12. Yearly Base Consumption discrepancy – Custom measure	14

Table 13. Boiler Efficiency discrepancy – Custom measure	15

Table B-1. Verified Cost Effectiveness Inputs – PGL	19

Table B-2. Verified Cost Effectiveness Inputs – NSG	21

# Introduction

This report presents the results of the impact evaluation of the Peoples Gas (PGL) and North Shore Gas (NSG) 2025 Multi-Family Market Rate (MF MR) program. It presents a summary of the energy impacts for the total program and broken out by relevant measure and program structure details. Appendix A presents the impact analysis methodology. Appendix B presents the Total Resource Cost (TRC) cost-effectiveness analysis inputs. Program year 2025 covers January 1 to December 31, 2025 (2025).

# Program Description

The MF MR Program is designed to provide a “one-stop-shop” to multi-family property owners and managers of buildings with three or more units to achieve comprehensive improvements in energy efficiency that previously would have required accessing multiple programs. The delivery approach consists of four paths, described below.

The Joint path is offered jointly with ComEd and provides free assessment and no-cost direct installation (DI) of in-unit (IU) measures in residential multi-family buildings with three or more living units.

The Prescriptive Rebate path provides standardized incentives for energy efficient equipment based on the size and efficiency of the equipment installed or on a per unit basis.

The Partner Trade Ally (PTA) path provides standardized incentives for energy efficient equipment, similar to the Prescriptive Rebate path. Additionally, the PTA path provides higher incentives to a network of partner trade allies (PTAs) who are selected, screened, and registered with the Multi-Family Program. In turn, the PTAs offer larger rebates than prescriptive rebates to their customers to install energy efficient products.

The Custom path provides technical services and custom rebates for non-standard building improvement upgrades as well as incentive opportunities for energy efficient new construction projects in multi-family buildings.

The PGL program had 125 participants in 2025 and completed 264 projects as shown in Table 1.

Table 1. 2025 Volumetric Summary for PGL

| Participation | Joint | PTA | Prescriptive | Custom | Total |
| --- | --- | --- | --- | --- | --- |
| Participants * | 13 | 82 | 19 | 11 | 125 |
| Installed Projects † | 140 | 94 | 19 | 11 | 264 |
| Measure Types Installed ‡ | 10 | 16 | 12 | 2 | 40 |

* Participants are defined as distinct count of gas account numbers.

† Installed Projects are defined as distinct count of project IDs.

‡ Measure Types Installed are defined as distinct count of PGL measure names.

Source: Peoples Gas tracking data and evaluation team analysis.

Table 2 summarizes the installed measure quantities that are the basis for verified energy savings for PGL. The disadvantaged communities (DAC) designation represents measures installed in disadvantaged areas based on census tracts.

Table 2. 2025 Installed Measure Quantities for PGL

| Program Category | Program Path | Measure | Quantity Unit | Installed Quantity |
| --- | --- | --- | --- | --- |
| Non-DAC | Joint | Assessments/No Savings | Unit | 98 |
|  |  | Faucet Aerator – Bathroom (IU) | Unit | 66 |
|  |  | Faucet Aerator – Kitchen (IU) | Unit | 72 |
|  |  | Low Flow Showerheads (IU) | Unit | 84 |
|  |  | Pipe Insulation | LN FT | 18 |
|  |  | Programmable Thermostats | Unit | 9 |
|  |  | Reprogrammable Thermostat | Unit | 1 |
|  |  | Shower Timer | Unit | 102 |
|  | PTA | Assessment/No Savings | Unit | 8,464 |
|  |  | Boiler Chemical Descaling | MBH Input | 89,154 |
|  |  | Boiler Tune Up | MBH Input | 80,517 |
|  |  | High Efficiency Boiler | Unit | 12 |
|  |  | On Demand DHW Controls | Living Units | 142 |
|  |  | Pipe Insulation | LN FT | 15,571 |
|  |  | Pipe Insulation | Unit | 23 |
|  |  | Steam Pipe Averaging Controls | Living Units | 197 |
|  |  | Steam Trap | Unit | 3,251 |
|  | Prescriptive | Advanced Thermostat (CA) | Unit | 1 |
|  |  | Advanced Thermostat (IU) | Unit | 3 |
|  |  | Assessment/No Savings | Unit | 3 |
|  |  | Central Water Heater | Unit | 326 |
|  |  | Central Water Heater | Living Units | 3 |
|  |  | Gas High Efficiency Furnace | Unit | 5 |
|  |  | High Efficiency Boiler | Unit | 5 |
|  |  | Steam Pipe Averaging Controls | Living Units | 191 |
|  | Custom | HVAC – Custom | Unit | 8 |
|  |  | Process – Custom | Unit | 3 |
| DAC | Joint | Assessments/No Savings – DAC | Unit | 42 |
|  |  | Faucet Aerator – Bathroom (IU) – DAC | Unit | 6 |
|  |  | Faucet Aerator – Kitchen (IU) – DAC | Unit | 8 |
|  |  | Low Flow Showerheads (IU) – DAC | Unit | 4 |
|  |  | Programmable Thermostats – DAC | Unit | 6 |
|  |  | Shower Timer – DAC | Unit | 39 |
|  | PTA | Assessment/No Savings – DAC | Unit | 1,351 |
|  |  | Boiler Tune Up – DAC | MBH Input | 16,524 |
|  |  | DHW Boiler Tune-up – DAC | Living Units | 512 |
|  |  | On Demand DHW Controls – DAC | Living Units | 30 |
|  |  | Pipe Insulation – DAC | LN FT | 548 |
|  |  | Pipe Insulation – DAC | Unit | 7 |
|  |  | Steam Pipe Averaging Controls – DAC | Living Units | 30 |
|  |  | Steam Trap – DAC | Unit | 346 |
|  | Prescriptive | Advanced Thermostat (IU) – DAC | Unit | 3 |
|  |  | Assessment/No Savings – DAC | Unit | 1 |
|  |  | Direct Fired Heater – DAC | MBH Input | 5,157 |
|  |  | High Efficiency Boiler – DAC | Unit | 6 |
|  |  | Programmable Thermostat – DAC | Unit | 1 |
|  |  | Water Heater (COM) – DAC | Unit | 2 |

Source: Peoples Gas tracking data and evaluation team analysis.

The NSG program had 5 participants in 2025 and completed 360 projects as shown in the following Table 3.

Table 3. 2025 Volumetric Summary for NSG

| Participation | Joint | PTA (Gas Only) | Prescriptive | Total |
| --- | --- | --- | --- | --- |
| Participants * | 2 | 2 | 1 | 5 |
| Installed Projects † | 357 | 2 | 1 | 360 |
| Measure Types Installed ‡ | 5 | 4 | 2 | 11 |

* Participants are defined as distinct count of gas account numbers.

† Installed Projects are defined as distinct count of project IDs.

‡ Measure Types Installed are defined as distinct count of NSG measure names.

Source: North Shore Gas tracking data and Guidehouse evaluation team analysis.

Table 4 summarizes the installed measure quantities that are the basis for verified energy savings for NSG.

Table 4. 2025 Installed Measure Quantities for NSG

| Program Category | Program Path | Measure | Quantity Unit | Installed Quantity |
| --- | --- | --- | --- | --- |
| Non-DAC | Joint | Assessments/No Savings | Unit | 357 |
|  |  | Faucet Aerator – Bathroom (IU) | Unit | 17 |
|  |  | Faucet Aerator – Kitchen (IU) | Unit | 40 |
|  |  | Low Flow Showerheads (IU) | Unit | 29 |
|  |  | Programmable Thermostats | Unit | 2 |
|  | PTA | Pipe Insulation | LN FT | 1,240 |
|  | Prescriptive | Boiler Reset Controls | MBH Input | 5,507 |
|  |  | Boiler Tune Up | MBH Input | 5,507 |
| DAC | PTA | Boiler Tune Up – DAC | MBH Input | 1,028 |
|  |  | DHW Boiler Tune-up – DAC | Living Units | 10 |
|  |  | Pipe Insulation – DAC | LN FT | 197 |

Source: North Shore Gas tracking data and Guidehouse evaluation team analysis

# Program Savings Detail

Table 5 summarizes the energy savings the PGL Multifamily Market Rate Program achieved by path in 2025.

Table 5. 2025 Annual Energy Savings Summary for PGL

| Program Category | Program Path | Ex Ante Gross Savings (Therms) | Verified Gross RR* | Verified Gross Savings (Therms) | NTG† | NPSO‡ | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Non-DAC | Joint | 2,640 | 100% | 2,640 | 0.99 | 1.083 | 2,843 |
|  | PTA | 793,813 | 104% | 827,992 | 0.88 | 1.083 | 789,110 |
|  | Prescriptive | 26,890 | 98% | 26,264 | 0.87 | 1.083 | 24,746 |
|  | Custom | 101,116 | 89% | 89,905 | 0.87 | 1.083 | 84,709 |
| Non-DAC Subtotal |  | 924,460 | 102% | 946,801 |  |  | 901,408 |
| DAC | PTA | 79,774 | 106% | 84,546 | 1.00 | N/A | 84,546 |
|  | Prescriptive | 49,577 | 100% | 49,608 | 1.00 | N/A | 49,608 |
|  | Joint | 523 | 100% | 523 | 1.00 | 1.083 | 551 |
| DAC Subtotal |  | 129,874 | 104% | 134,676 |  |  | 134,704 |
| Total |  | 1,054,333 | 103% | 1,081,478 |  |  | 1,036,112 |

* Realization Rate (RR) is the ratio of verified gross savings to ex ante gross savings, based on evaluation research findings.

† The program path NTG value is calculated as Verified Net/Verified Gross. The program-level analysis typically used measure-level deemed NTG values (see Table 4), which are listed here: https://www.ilsag.info/evaluator-ntg-recommendations-for-2025.

‡ The market rate net savings were multiplied by a residential non-participant spillover (NPSO) factor of 1.083. Based on SAG Policy, if deemed NTG is multiplied by the NPSO and the resulted NTG value is less than 1.00, the evaluation assigned NTG of 1.00 to projects in DAC zones. If the resulted NTG value is >1.00, the evaluation used the >1.00 value for calculation of net savings impact.

Source: Evaluation team analysis.

Table 6 summarizes the energy savings the NSG Multifamily Program achieved by path in 2025.

Table 6. 2025 Annual Energy Savings Summary for NSG

| Program Category | Program Path | Ex Ante Gross Savings (Therms) | Verified Gross RR | Verified Gross Savings (Therms) | NTG† | NPSO‡ | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Non-DAC | Joint | 830 | 100% | 830 | 1.01 | 1.083 | 904 |
|  | PTA | 3,966 | 100% | 3,966 | 0.88 | 1.083 | 3,780 |
|  | Prescriptive | 9,899 | 100% | 9,899 | 0.87 | 1.083 | 9,327 |
| Non-DAC Subtotal |  | 14,695 | 100% | 14,695 |  |  | 14,011 |
| DAC | PTA | 1,094 | 100% | 1,094 | 1.00 | N/A | 1,094 |
| DAC Subtotal |  | 1,094 | 100% | 1,094 |  |  | 1,094 |
| Total |  | 15,789 | 100% | 15,789 |  |  | 15,105 |

* Realization Rate (RR) is the ratio of verified gross savings to ex ante gross savings, based on evaluation research findings.

† The program path NTG value is calculated as Verified Net/Verified Gross.

‡ The market rate net savings were multiplied by a residential non-participant spillover (NPSO) factor of 1.083. Based on SAG Policy, if deemed NTG is multiplied by the NPSO and the resulted NTG value is less than 1.00, the evaluation assigned NTG of 1.00 to projects in DAC zones. If the resulted NTG value is >1.00, the evaluation used the >1.00 value for calculation of net savings impact. Source: Evaluation team analysis.

# Program Savings by Measure

The PGL program includes 21 measures as shown in the following Table 7. The Steam Trap and Custom measures contributed the most savings.

Table 7. 2025 Annual Energy Savings by Measure for PGL

| Program Category | Program Path | Savings Category | Ex Ante Gross Savings (Therms) | Verified Gross RR* | Verified Gross Savings (Therms) | NTG† | NPSO‡ | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Non-DAC | Joint | Faucet Aerator - Bathroom (IU) | 120 | 100% | 120 | 1.01 | 1.083 | 131 |
|  |  | Faucet Aerator - Kitchen (IU) | 648 | 100% | 648 | 1.01 | 1.083 | 709 |
|  |  | Low Flow Showerheads (IU) | 1,038 | 100% | 1,039 | 1.01 | 1.083 | 1,136 |
|  |  | Pipe Insulation | 44 | 100% | 44 | 0.96 | 1.083 | 46 |
|  |  | Programmable Thermostats | 365 | 100% | 365 | 0.96 | 1.083 | 379 |
|  |  | Reprogrammable Thermostat | 41 | 100% | 41 | 0.96 | 1.083 | 42 |
|  |  | Shower Timer | 385 | 100% | 385 | 0.96 | 1.083 | 400 |
|  |  | Non-DAC Joint Subtotal | 2,640 | 100% | 2,640 |  |  | 2,843 |
|  | PTA | Boiler Chemical Descaling | 37,021 | 100% | 37,021 | 0.88 | 1.083 | 35,283 |
|  |  | Boiler Tune Up | 37,742 | 100% | 37,742 | 0.88 | 1.083 | 35,970 |
|  |  | High Efficiency Boiler | 21,301 | 100% | 21,301 | 0.88 | 1.083 | 20,301 |
|  |  | On Demand DHW Controls | 8,903 | 100% | 8,903 | 0.88 | 1.083 | 8,485 |
|  |  | Pipe Insulation | 80,566 | 100% | 80,566 | 0.88 | 1.083 | 76,782 |
|  |  | Steam Pipe Averaging Controls | 13,033 | 92% | 12,034 | 0.88 | 1.083 | 11,469 |
|  |  | Steam Trap | 595,247 | 107% | 630,424 | 0.88 | 1.083 | 600,820 |
|  |  | Non-DAC PTA Subtotal | 793,813 | 105% | 827,992 |  |  | 789,110 |
|  | Prescriptive | Advanced Thermostat (CA) | 68 | 100% | 68 | 0.87 | 1.083 | 64 |
|  |  | Advanced Thermostat (IU) | 148 | 112% | 167 | 0.87 | 1.083 | 157 |
|  |  | Central Water Heater | 3,233 | 100% | 3,218 | 0.87 | 1.083 | 3,032 |
|  |  | Gas High Efficiency Furnace | 630 | 154% | 969 | 0.87 | 1.083 | 913 |
|  |  | High Efficiency Boiler | 10,174 | 100% | 10,174 | 0.87 | 1.083 | 9,586 |
|  |  | Steam Pipe Averaging Controls | 12,636 | 92% | 11,668 | 0.87 | 1.083 | 10,994 |
|  |  | Non-DAC Prescriptive Subtotal | 26,890 | 98% | 26,264 |  |  | 24,746 |
|  | Custom | HVAC – Custom | 90,768 | 85% | 77,478 | 0.87 | 1.083 | 73,001 |
|  |  | Process – Custom | 10,348 | 120% | 12,427 | 0.87 | 1.083 | 11,709 |
|  |  | Non-DAC Custom Subtotal | 101,116 | 89% | 89,905 |  |  | 84,709 |
| Non-DAC Subtotal |  |  | 924,460 | 102% | 946,801 |  |  | 901,408 |
| DAC | Joint | Faucet Aerator – Bathroom (IU) – DAC | 11 | 100% | 11 | 1.00 | 1.083 | 12 |
|  |  | Faucet Aerator – Kitchen (IU) – DAC | 72 | 100% | 72 | 1.00 | 1.083 | 79 |
|  |  | Low Flow Showerheads (IU) – DAC | 49 | 100% | 49 | 1.00 | 1.083 | 54 |
|  |  | Programmable Thermostats – DAC | 243 | 100% | 243 | 1.00 | 1.083 | 253 |
|  |  | Shower Timer – DAC | 147 | 100% | 147 | 1.00 | 1.083 | 153 |
|  |  | DAC Joint Subtotal | 523 | 100% | 523 |  |  | 551 |
|  | PTA | Boiler Tune Up – DAC | 7,746 | 100% | 7,746 | 1.00 | N/A | 7,746 |
|  |  | DHW Boiler Tune-up – DAC | 1,434 | 100% | 1,439 | 1.00 | N/A | 1,439 |
|  |  | On Demand DHW Controls – DAC | 1,881 | 100% | 1,881 | 1.00 | N/A | 1,881 |
|  |  | Pipe Insulation – DAC | 4,127 | 100% | 4,127 | 1.00 | N/A | 4,127 |
|  |  | Steam Pipe Averaging Controls – DAC | 1,985 | 92% | 1,833 | 1.00 | N/A | 1,833 |
|  |  | Steam Trap – DAC | 62,601 | 108% | 67,520 | 1.00 | N/A | 67,520 |
|  |  | DAC PTA Subtotal | 79,774 | 106% | 84,546 |  |  | 84,546 |
|  | Prescriptive | Advanced Thermostat (IU) – DAC | 139 | 120% | 167 | 1.00 | N/A | 167 |
|  |  | Direct Fired Heater – DAC | 12,849 | 100% | 12,849 | 1.00 | N/A | 12,849 |
|  |  | High Efficiency Boiler – DAC | 36,478 | 100% | 36,478 | 1.00 | N/A | 36,478 |
|  |  | Programmable Thermostat – DAC | 37 | 109% | 41 | 1.00 | N/A | 41 |
|  |  | Water Heater (COM) - DAC | 74 | 100% | 74 | 1.00 | N/A | 74 |
|  |  | DAC Prescriptive Subtotal | 49,577 | 100% | 49,608 |  |  | 49,608 |
| DAC Subtotal |  |  | 129,874 | 104% | 134,676 |  |  | 134,704 |
| Total |  |  | 1,054,333 | 103% | 1,081,478 |  |  | 1,036,112 |

* Realization Rate (RR) is the ratio of verified gross savings to ex ante gross savings, based on evaluation research findings.

† NTG, Net to Gross is the deemed value available on the SAG website: https://www.ilsag.info/evaluator-ntg-recommendations-for-2025/ .

‡ The market rate net savings were multiplied by a residential non-participant spillover (NPSO) factor of 1.083. Based on SAG Policy, if deemed NTG is multiplied by the NPSO and the resulted NTG value is less than 1.00, the evaluation assigned NTG of 1.00 to projects in DAC zones. If the resulted NTG value is >1.00, the evaluation used the >1.00 value for calculation of net savings impact.

Source: Evaluation team analysis.

The NSG program includes 7 measures as shown in Table 8. The Boiler Reset Controls and Pipe Insulation measures contributed the most savings.

Table 8. 2025 Annual Energy Savings by Measure for NSG

| Program Category | Program Path | Savings Category | Ex Ante Gross Savings (Therms) | Verified Gross RR* | Verified Gross Savings (Therms) | NTG† | NPSO‡ | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Non-DAC | Joint | Faucet Aerator – Bathroom (IU) | 31 | 100% | 31 | 1.01 | 1.083 | 34 |
|  |  | Faucet Aerator – Kitchen (IU) | 360 | 100% | 360 | 1.01 | 1.083 | 394 |
|  |  | Low Flow Showerheads (IU) | 358 | 100% | 359 | 1.01 | 1.083 | 392 |
|  |  | Programmable Thermostats | 81 | 100% | 81 | 0.96 | 1.083 | 84 |
|  |  | Non-DAC Joint Subtotal | 830 | 100% | 830 |  |  | 904 |
|  | PTA | Pipe Insulation | 3,966 | 100% | 3,966 | 0.88 | 1.083 | 3,780 |
|  | Prescriptive | Boiler Reset Controls | 7,318 | 100% | 7,318 | 0.87 | 1.083 | 6,895 |
|  |  | Boiler Tune Up | 2,581 | 100% | 2,581 | 0.87 | 1.083 | 2,432 |
|  |  | Non-DAC Prescriptive Subtotal | 13,865 | 100% | 13,865 |  |  | 13,107 |
| Non-DAC Subtotal |  |  | 14,695 | 100% | 14,695 |  |  | 14,011 |
| DAC | PTA | Boiler Tune Up – DAC | 482 | 100% | 482 | 1.00 | N/A | 482 |
|  |  | DHW Boiler Tune-up – DAC | 28 | 100% | 28 | 1.00 | N/A | 28 |
|  |  | Pipe Insulation – DAC | 584 | 100% | 584 | 1.00 | N/A | 584 |
|  |  | DAC PTA Subtotal | 1,094 | 100% | 1,094 |  |  | 1,094 |
| DAC Subtotal |  |  | 1,094 | 100% | 1,094 |  |  | 1,094 |
| Total |  |  | 15,789 | 100% | 15,789 |  |  | 15,105 |

* Realization Rate (RR) is the ratio of verified gross savings to ex ante gross savings, based on evaluation research findings.

† NTG, Net to Gross is the deemed value available on the SAG website: https://www.ilsag.info/evaluator-ntg-recommendations-for-2025/ .

‡ The market rate net savings were multiplied by a residential non-participant spillover (NPSO) factor of 1.083. Based on SAG Policy, if deemed NTG is multiplied by the NPSO and the resulted NTG value is less than 1.00, the evaluation assigned NTG of 1.00 to projects in DAC zones. If the resulted NTG value is >1.00, the evaluation used the >1.00 value for calculation of net savings impact.

Source: Evaluation team analysis.

# Impact Analysis Findings and Recommendations

The evaluation team developed several findings and recommendations based on the 2025 evaluation. The overall impact of these findings on the program resulted in a 103% and 100% realization rates for Peoples Gas and North Shore Gas, respectively.

## Impact Parameter Estimates

Table 9 shows the unit therm savings and realization rate findings by measure from our review. The realization rate is the ratio of the verified savings to the ex ante savings. Following the table, we provide findings and recommendations, including discussion of all measures with realization rates above or below 100%. Appendix A provides a description of the impact analysis methodology.

Table 9. Verified Gross Savings Parameters

| Measure | Unit Basis | Ex Ante Gross (therms/unit) | Verified Gross (therms/unit) | Realization Rate | Data Source(s) |
| --- | --- | --- | --- | --- | --- |
| Advanced Thermostat (CA) | Unit | 68.45 | 68.45 | 100% | Illinois TRM, v13.0†, Section 4.4.48 |
| Advanced Thermostat (IU) | Unit | Varies | 55.53 | 116% | Illinois TRM, v13.0†, Section 5.3.16 |
| Boiler Chemical Descaling | MBH Input | 0.42 | 0.42 | 100% | Illinois TRM, v13.0†, Section 4.4.49 |
| Boiler Reset Controls | MBH Input | 1.33 | 1.33 | 100% | Illinois TRM, v13.0†, Section 4.4.4 |
| Boiler Tune Up | MBH Input | Varies | 0.47 | 100% | Illinois TRM, v13.0†, Section 4.4.2 |
| Central Water Heater | Unit | 9.49 | 9.45 | 100% | Illinois TRM, v13.0†, Section 4.3.7 |
| Central Water Heater | Living Units | 45.97 | 45.97 | 100% | Illinois TRM, v13.0†, Section 4.3.7 |
| DHW Boiler Tune-up | Living Units | 2.80 | 2.29 | 82% | Illinois TRM, v13.0†, Section 4.3.10 |
| Direct Fired Heater | MBH Input | 2.49 | 2.49 | 100% | Illinois TRM, v13.0†, Section 4.4.39 |
| Gas High Efficiency Furnace | Unit | 125.91 | 193.70 | 154% | Illinois TRM, v13.0†, Section 5.3.7 |
| High Efficiency Boiler | Unit | Varies | Varies | 100% | Illinois TRM, v13.0†, Section 4.4.10 |
| HVAC – Custom | Unit | Varies | Varies | 85% | Project File Review, Monthly Billing Data‡ |
| Faucet Aerator – Bathroom (IU) | Unit | 1.82 | 1.82 | 100% | Illinois TRM, v13.0†, Section 5.4.4 |
| Faucet Aerator – Kitchen (IU) | Unit | 9.00 | 9.00 | 100% | Illinois TRM, v13.0†, Section 5.4.4 |
| Low Flow Showerheads (IU) | Unit | 12.36 | 12.36 | 100% | Illinois TRM, v13.0†, Section 5.4.5 |
| On Demand DHW Controls | Living Units | 62.70 | 62.70 | 100% | Illinois TRM, v13.0†, Section 4.3.8 |
| Pipe Insulation | LN FT | Varies | Varies | 100% | Illinois TRM, v13.0†, Section 4.4.14 |
| Pipe Insulation | Unit | Varies | Varies | 100% | Illinois TRM, v13.0†, Section 4.4.14 |
| Process – Custom | Unit | Varies | Varies | 120% | Project File Review, Monthly Billing Data‡ |
| Programmable Thermostats | Unit | Varies | 40.50 | 109% | Illinois TRM, v13.0†, Section 5.3.11 |
| Reprogrammable Thermostat | Unit | 40.50 | 40.50 | 100% | Illinois TRM, v13.0†, Section 5.3.11 |
| Shower Timer | Unit | 3.78 | 3.78 | 100% | Illinois TRM, v13.0†, Section 5.4.9 |
| Steam Pipe Averaging Controls | Living Units | 66.16 | 61.09 | 92% | Illinois TRM, v13.0†, Section 4.4.36 |
| Steam Trap | Unit | Varies | 195.14 | 107% | Illinois TRM, v13.0†, Section 4.4.16 |
| Water Heater (COM) | Unit | 37.09 | 37.08 | 100% | Illinois TRM, v13.0†, Section 4.3.10 |

† State of Illinois Technical Reference Manual version 13.0 from http://www.ilsag.info/technical-reference-manual.html.

‡ Project files and monthly billing data provided by Peoples Gas.

Source: Program tracking data and evaluation team analysis.

## Findings and Recommendations

The evaluation team developed findings and recommendations based on the 2025 evaluation, organized by path type in the following sections.

### Cross-Cutting

Finding 1. For all the 16 instances of boiler replacement measures listed below, the tracking data did not report boiler input capacity.

Hot Water Boiler-PG MF Pres

Hot Water Boiler-PG MF PTA

Steam Boiler-PG MF Pres

Steam Boiler-PG MF PTA

The evaluation team reviewed the available inputs reported in the tracking data and confirmed alignment with applicable IL-TRM guidance. The missing inputs are required to fully replicate and evaluate these measures and should be reported in future program years.

Recommendation 1.  Ensure all inputs relevant to the savings calculation are reported in the tracking data.

Finding 2. For all 14 instances of the 1 Pipe Steam Averaging Controls-PG MF PTA and 2 instances of 1 Pipe Steam Averaging Controls-PG MF Pres measures (WO-6966455 and WO-6966480), the ex ante savings calculations applied a Non-Participant Spillover (NPSO) of 1.083 to the gross therm savings. The evaluation team calculated gross savings excluding the NPSO factor. This measure accounts for 16% of PGL Prescriptive and 2% of PGL PTA component’s reported gross Therms savings, and the impact of this adjustment resulted in a realization rate of 92% for these measures.

Recommendation 2.  Ensure that gross savings are calculated excluding NPSO.

### Prescriptive

Finding 3. For 3 out of 4 instances of the IU-Smart Thermostat-PG MF Pres measures (WO‑8095934, WO‑8095713 and WO‑7860560), the ex ante savings calculations applied a 7.1% Heating Reduction factor, applicable when the existing thermostat is Programmable. However, tracking data did not report existing thermostat type. Hence, the evaluation team applied the Heating Reduction factor of 8.5%, corresponding to an Unknown existing thermostat type, consistent with IL-TRM v13.0. This measure accounts for 0.4% of PGL Prescriptive component’s reported gross Therms savings, and the impact of this adjustment resulted in a realization rate of 116% for this measure.

Recommendation 3.  Apply the default heating reduction factor from the applicable IL-TRM when the existing thermostat type is unknown.

Finding 4. For all five instances of Water Heater 88% TE-Central Plant-Units-PG MF Pres measure, ex ante calculations considered number of people in multifamily household (MFHH) as 2.1, consistent with IL‑TRM v12.0. The evaluation team updated MFHH to 2.09, consistent with Section 4.3.7 IL-TRM v13.0. This measure accounts for 4.2% of PGL Prescriptive component’s reported gross Therms savings, and the impact of this adjustment resulted in a realization rate of 99.5% for this measure.

Recommendation 4.  Use multi-family household assumption that is consistent with Section 4.3.7 of the IL-TRM.

Finding 5.  For all instances of the IU–Furnace >95% AFUE–PG MF Pres measure, the ex ante savings calculations applied a Multifamily Household Factor of 0.65. The evaluation team did not apply the scaling Factor to be consistent with Section 5.3.7 of IL-TRM v13.0. This measure accounts for 0.8% of PGL Prescriptive component’s reported gross Therms savings, and the impact of this adjustment resulted in a realization rate of 154% for this measure.

Recommendation 5. Use inputs and algorithm from Section 5.3.7 of the IL-TRM to calculate savings.

### Partner Trade Ally

Finding 6.  For all instances of F&T/Inverted Bucket Steam Traps-HVAC with Audit-PG MF PTA, Radiator Steam Traps-HVAC with Audit-PG MF PTA and Steam Traps-HVAC No Audit-PG MF PTA measures, the ex ante savings calculations applied EFLH values of either 1,540 hours (MF High-Rise) or 1,782 hours (MF Mid-Rise). However, the tracking data did not report Building Type for these measures. As a result, the evaluation team applied an EFLH value of 1,661 hours (the average of 1,540 and 1,782) across all instances. Table 10 provides information on instances using MF Mid-Rise or MF High-Rise EFLH values. This measure accounts for 75% of PGL PTA component’s reported gross Therms savings, and the impact of this adjustment resulted in a realization rate of 106% for this measure.

Table 10. Instances with EFLH discrepancy

| Measure | Ex Ante Building Type | Heating EFLH (Hours) | Impacted Instances | Total Instances |
| --- | --- | --- | --- | --- |
| F&T/Inverted Bucket Steam Traps-HVAC with Audit-PG MF PTA | MF High-Rise | 1,540 | 16 | 20 |
|  | MF Mid-Rise | 1,782 | 4 |  |
| Radiator Steam Traps-HVAC with Audit-PG MF PTA | MF High-Rise | 1,540 | 24 | 28 |
|  | MF Mid-Rise | 1,782 | 4 |  |
| Steam Traps-HVAC No Audit-PG MF PTA | MF High-Rise | 1,540 | 1 | 1 |

Source: Program tracking data and evaluation team analysis.

Recommendation 6. Ensure building type is reported in the tracking data and calculate savings using EFLH consistent with the reported building type.

Finding 7. For all instances of DHW Boiler Tune-Up-PG MF PTA and DHW Boiler Tune-Up-NSG MF PTA measures, the ex ante savings calculations applied an incoming water temperature of 51°F, compared to 50.7°F specified in the IL-TRM v13.0. The evaluation team applied a 50.7°F incoming water. This measure accounts for 0.2% of PGL PTA and 0.6% of NSG PTA component’s reported gross Therms savings, and the impact of this adjustment resulted in a realization rate of 100.4% for this measure.

Recommendation 7.  Use incoming water temperature as specified in Section 4.3.10 of the IL-TRM.

Finding 7. For four out of 20 instances of F&T/Inverted Bucket Steam Traps-HVAC with Audit-PG MF PTA measure, the ex ante savings calculations applied a NTG ratio of 0.87, applicable to stream trap measures under the Prescriptive program. The evaluation team applied a 0.88 NTG ratio, applicable to the PTA program, consistent with CY2025 ILSAG – evaluator NTG recommendation. This measure accounts for 75% of PGL PTA component’s reported gross Therms savings, and the impact of this adjustment increased net savings for this measure.

Recommendation 7.  Ensure quality control to use appropriate NTG ratio specified in 2025 ILSAG for the program paths.

### Custom

Finding 8.  For six instances of HVAC – Other – PG MF Custom and all three instances of Process – Other – PG MF Custom measures, ex ante calculation assumed a uniform balance point temperature of 65°F. The evaluation team determined the actual balance point temperature of the building using historical monthly utility consumption and the corresponding actual average monthly temperature data, selecting the balance point that produced the best fit between monthly therm consumption and heating degree days. The difference between ex ante and ex post balance point temperatures is provided in Table 11 below. These measures account for 100% of PGL Custom component’s reported gross Therms savings, and the impact of adjustments from Findings 8 – 12 resulted in a realization rate of 85% for HVAC – Other and 120% for Process – Other measures.

Table 11. Balance Point Temperature discrepancy – Custom measure

| Measure | Work Order ID | Balance Point Temperature – Ex Ante | Balance Point Temperature – Ex Post * |
| --- | --- | --- | --- |
| HVAC – Other – PG MF Custom | WO-5770434 | 65°F | 72°F |
|  | WO-7022224 | 65°F | 60°F |
|  | WO-6994879 | 65°F | 60°F |
|  | WO-6989785 | 65°F | 73°F |
|  | WO-7022061 | 65°F | 71°F |
|  | WO-6346458 | 65°F | 72°F |
| Process – Other – PG MF Custom | WO-7173695 | 65°F | 66°F |
|  | WO-7208901 | 65°F | 69°F |
|  | WO-7173670 | 65°F | 69°F |

Source: Program tracking data, project documentation and evaluation team analysis.

Recommendation 8. Determine building specific balance point temperature using actual Therm consumption and weather data, and incorporate it into custom calculations.

Finding 9.  For all instances of HVAC – Other – PG MF Custom and Process – Other – PG MF Custom measures, ex ante savings were normalized using Typical Meteorological Year 3 (TMY3) weather data (1991 – 2005). The evaluation team updated the savings using a more recent TMY dataset (2009 – 2023). These measures account for 100% of PGL Custom component’s reported gross Therms savings and the impact of adjustments from Findings 8 – 12 resulted in a realization rate of 85% for HVAC – Other and 120% for Process – Other measures.

Recommendation 9. Use a more recent, representative TMY weather dataset (TMYx) to calculate custom billing analysis savings.

Finding 10.  For six instances of HVAC – Other – PG MF Custom and all three instances of Process – Other – PG MF Custom measures, ex ante calculations estimated the annual baseload as the average therm consumption during June through September. The evaluation team estimated annual baseload using a regression-based approach, modeling the relationship between monthly Heating Degree Days (HDD) and therm consumption to separate weather-dependent and base load components. Monthly HDD was calculated using building-specific balance point temperature as shown in Table 11. The difference between ex ante and ex post annual baseload is provided in Table 12.  These measures account for 100% of PGL Custom component’s reported gross Therms savings, and the impact of adjustments from Findings 8 – 12 resulted in a realization rate of 85% for HVAC – Other and 120% for Process – Other measures.

Table 12. Yearly Base Consumption discrepancy – Custom measure

| Measure | Work Order ID | Ex ante Annual Baseload – (Therms) | Ex post Annual Baseload – (Therms) |
| --- | --- | --- | --- |
| HVAC – Other – PG MF Custom | WO-5770434 | 123,733 | 118,577 |
|  | WO-7022224 | 231,079 | 237,075 |
|  | WO-6994879 | 34,205 | 40,323 |
|  | WO-6989785 | 12,665 | 25,640 |
|  | WO-7022061 | 34,205 | 27,511 |
|  | WO-6346458 | 135,294 | 121,282 |
| Process – Other – PG MF Custom | WO-7173695 | 42,008 | 42,282 |
|  | WO-7208901 | 55,440 | 56,061 |
|  | WO-7173670 | 55,440 | 56,061 |

Source: Program tracking data, project documentation and evaluation team analysis.

Recommendation 10.  Estimate annual baseload using a regression-based method with monthly therm consumption and temperature data.

Finding 11. For three instances of HVAC – Other – PG MF Custom and three instances of Process – Other – PG MF Custom measures, the ex ante savings calculated main boiler combustion efficiency by averaging the main and backup boiler efficiencies. The evaluation team used the main boiler efficiency to calculate verified savings as the custom calculator indicates that the main boiler accounts for 100% of the operating time share. The difference between ex ante and ex post boiler efficiencies is provided in the Table 13 .

Table 13. Boiler Efficiency discrepancy – Custom measure

| Measure | Work Order ID | Main Boiler Efficiency – Pre Installation (Ex Ante) | Main Boiler Efficiency – Pre Installation (Ex Post) | Main Boiler Efficiency – Post Installation (Ex Ante) | Main Boiler Efficiency – Post Installation (Ex Post) |
| --- | --- | --- | --- | --- | --- |
| HVAC – Other – PG MF Custom | WO-7173701 | 76.4% | 76.8% | 80.3% | 80.0% |
|  | WO-6989785 | 73.8% | 71.0% | 77.1% | 74.3% |
|  | WO-7022061 | 75.7% | 73.2% | 79.0% | 76.4% |
| Process – Other – PG MF Custom | WO-7173695 | 74.7% | 79.2% | 77.7% | 82.0% |
|  | WO-7208901 | 81.2% | 79.0% | 83.7% | 82.5% |
|  | WO-7173670 | 83.7% | 81.9% | 85.4% | 84.3% |

Source: Program tracking data, project documentation and evaluation team analysis.

Recommendation 11.  Ensure that all reported boiler efficiencies align with actual values as documented in project application files.

Finding 12.  For one instance of HVAC – Other – PG MF Custom measure (WO-7173732), therm savings reported in the tracking data did not align with the ex ante calculator. The evaluation team utilized the ex ante calculator to calculate verified savings.

Recommendation 12. Ensure that savings reported in the tracking data align with measure calculators like the MMDB.

Finding 13.  For all instances of HVAC – Other – PG MF Custom and Process – Other – PG MF Custom measures, ex ante did not apply a NPSO of 1.083 to calculate Net Savings. The evaluation team calculated verified net savings including an NPSO of 1.083. These measures account for 100% of PGL Custom component’s reported gross Therms savings, and the impact of this adjustment resulted in an increase in net savings by 8.3% for this measure.

Recommendation 13. When reporting net savings in the tracking data, ensure that the NPSO of 1.083 is used to calculate ex ante net savings.

# Appendix A. Impact Analysis Methodology

The evaluation team used the same impact methodology for each component. Verified gross savings were determined for each program measure by:

- Reviewing the savings algorithm inputs in the measure workbook for agreement with the IL-TRM v13.0 and IL-TRM Errata, where applicable.

- Validating the savings algorithm was applied correctly.

- Cross-checking per-unit savings values in the program tracking data with the verified values in the measure workbook or Guidehouse’s calculations if the MMDB workbook did not agree with the IL-TRM v13.0.

- Multiplying the verified per-unit savings value by the quantity reported in the tracking data. The team calculated verified net savings by multiplying the verified gross savings estimates by an NTG ratio. In Program Year 2025, NTG estimates used to calculate the net verified savings were based on past evaluation research and defined by a consensus process through the Illinois SAG.

- The market rate net savings were multiplied by a residential non-participant spillover (NPSO) factor of 1.083. Based on SAG Policy, if deemed NTG is multiplied by the NPSO and the resulted NTG value is less than 1.00, the evaluation assigned NTG of 1.00 to projects in DAC zones. If the resulted NTG value is >1.00, the evaluation used the >1.00 value for calculation of net savings impact.

# Appendix B. Program Specific Inputs for the Illinois TRC

The tables below show the Total Resource Cost (TRC) cost-effectiveness analysis inputs available at the time of producing this impact evaluation report. Additional required cost data (e.g., measure costs, program level incentive and non-incentive costs) are not included in these tables and will be provided to the evaluation team later. Guidehouse will include annual and lifetime water savings and greenhouse gas reductions in the end of year summary report.

Table B-1. Verified Cost Effectiveness Inputs – PGL

| Program Category | Program Path | Savings Category | DAC Project | Units | Quantity | Effective Useful Life | Early Replacement Flag† | Verified Gross Annual Water Savings (Gallons) | Ex Ante Gross Savings (Therms) | Verified Gross Savings (Therms) | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Non-DAC | Joint | Faucet Aerator - Bathroom (IU) | FALSE | Each | 66 | 10.0 | NO | 27,253 | 120 | 120 | 131 |
|  |  | Faucet Aerator - Kitchen (IU) | FALSE | Each | 72 | 10.0 | NO | 122,235 | 648 | 648 | 709 |
|  |  | Low Flow Showerheads (IU) | FALSE | Each | 84 | 10.0 | NO | 164,846 | 1,038 | 1,039 | 1,136 |
|  |  | Pipe Insulation | FALSE | LN FT | 18 | 15.0 | NO | - | 44 | 44 | 46 |
|  |  | Programmable Thermostats | FALSE | Each | 9 | 16.0 | NO | - | 365 | 365 | 379 |
|  |  | Reprogrammable Thermostat | FALSE | Each | 1 | 2.0 | NO | - | 41 | 41 | 42 |
|  |  | Shower Timer | FALSE | Each | 102 | 2.0 | NO | 1,133 | 385 | 385 | 400 |
|  | PTA | Boiler Chemical Descaling | FALSE | MBH Input | 89,154 | 6.0 | NO | - | 37,021 | 37,021 | 35,283 |
|  |  | Boiler Tune Up | FALSE | MBH Input | 80,517 | 3.0 | NO | - | 37,742 | 37,742 | 35,970 |
|  |  | High Efficiency Boiler | FALSE | Unit | 12 | 25.0 | NO | - | 21,301 | 21,301 | 20,301 |
|  |  | On Demand DHW Controls | FALSE | Living Units | 142 | 15.0 | NO | - | 8,903 | 8,903 | 8,485 |
|  |  | Pipe Insulation | FALSE | LN FT | 15,571 | 15.0 | NO | - | 79,797 | 79,797 | 76,050 |
|  |  | Pipe Insulation | FALSE | Unit | 23 | 15.0 | NO | - | 769 | 769 | 733 |
|  |  | Steam Pipe Averaging Controls | FALSE | Living Units | 197 | 20.0 | NO | - | 13,033 | 12,034 | 11,469 |
|  |  | Steam Trap | FALSE | Unit | 3,251 | 6.0 | NO | 37,025,125 | 595,247 | 630,424 | 600,820 |
|  | Prescriptive | Advanced Thermostat (CA) | FALSE | Unit | 1 | 11.0 | NO | - | 68 | 68 | 64 |
|  |  | Advanced Thermostat (IU) | FALSE | Unit | 3 | 11.0 | NO | - | 148 | 167 | 157 |
|  |  | Central Water Heater | FALSE | Each | 326 | 15.0 | NO | - | 3,095 | 3,080 | 2,902 |
|  |  | Central Water Heater | FALSE | Living Units | 3 | 15.0 | NO | - | 138 | 138 | 130 |
|  |  | Gas High Efficiency Furnace | FALSE | Unit | 5 | 20.0 | NO | - | 630 | 969 | 913 |
|  |  | High Efficiency Boiler | FALSE | Unit | 5 | 25.0 | NO | - | 10,174 | 10,174 | 9,586 |
|  |  | Steam Pipe Averaging Controls | FALSE | Living Units | 191 | 20.0 | NO | - | 12,636 | 11,668 | 10,994 |
|  | Custom | HVAC – Custom | FALSE | Unit | 8 | 14.9 | NO | - | 90,768 | 77,478 | 73,001 |
|  |  | Process – Custom | FALSE | Unit | 3 | 8.0 | NO | - | 10,348 | 12,427 | 11,709 |
| DAC | Joint | Faucet Aerator – Bathroom (IU) – DAC | TRUE | Each | 6 | 10.0 | NO | 2,478 | 11 | 11 | 12 |
|  |  | Faucet Aerator - Kitchen (IU) – DAC | TRUE | Each | 8 | 10.0 | NO | 13,582 | 72 | 72 | 79 |
|  |  | Low Flow Showerheads (IU) – DAC | TRUE | Each | 4 | 10.0 | NO | 7,850 | 49 | 49 | 54 |
|  |  | Programmable Thermostats – DAC | TRUE | Each | 6 | 16.0 | NO | - | 243 | 243 | 253 |
|  |  | Shower Timer – DAC | TRUE | Each | 39 | 2.0 | NO | 433 | 147 | 147 | 153 |
|  | PTA | Boiler Tune Up – DAC | TRUE | MBH Input | 16,524 | 3.0 | NO | - | 7,746 | 7,746 | 7,746 |
|  |  | DHW Boiler Tune-up – DAC | TRUE | Living Units | 512 | 3.0 | NO | - | 1,434 | 1,439 | 1,439 |
|  |  | On Demand DHW Controls – DAC | TRUE | Living Units | 30 | 15.0 | NO | - | 1,881 | 1,881 | 1,881 |
|  |  | Pipe Insulation – DAC | TRUE | LN FT | 548 | 15.0 | NO | - | 3,986 | 3,986 | 3,986 |
|  |  | Pipe Insulation – DAC | TRUE | Unit | 7 | 15.0 | NO | - | 141 | 141 | 141 |
|  |  | Steam Pipe Averaging Controls – DAC | TRUE | Living Units | 30 | 20.0 | NO | - | 1,985 | 1,833 | 1,833 |
|  |  | Steam Trap – DAC | TRUE | Unit | 346 | 6.0 | NO | 3,965,471 | 62,601 | 67,520 | 67,520 |
|  | Prescriptive | Advanced Thermostat (IU) – DAC | TRUE | Unit | 3 | 11.0 | NO | - | 139 | 167 | 167 |
|  |  | Direct Fired Heater – DAC | TRUE | MBH Input | 5,157 | 15.0 | NO | - | 12,849 | 12,849 | 12,849 |
|  |  | High Efficiency Boiler – DAC | TRUE | Unit | 6 | 25.0 | NO | - | 36,478 | 36,478 | 36,478 |
|  |  | Programmable Thermostat – DAC | TRUE | Unit | 1 | 16.0 | NO | - | 37 | 41 | 41 |
|  |  | Water Heater (COM) – DAC | TRUE | Unit | 2 | 15.0 | NO | - | 74 | 74 | 74 |
| Total or Weighted Average |  |  |  |  |  | 9.0 |  | 41,330,404 | 1,054,333 | 1,081,478 | 1,036,112 |

Source: Evaluation team analysis.

Table B-2. Verified Cost Effectiveness Inputs – NSG

| Program Category | Program Path | Savings Category | DAC Project | Units | Quantity | Effective Useful Life | Early Replacement Flag† | Verified Gross Annual Water Savings (Gallons) | Ex Ante Gross Savings (Therms) | Verified Gross Savings (Therms) | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Non-DAC | Joint | Faucet Aerator – Bathroom (IU) | FALSE | Each | 17 | 10.0 | NO | 7,020 | 31 | 31 | 34 |
|  |  | Faucet Aerator – Kitchen (IU) | FALSE | Each | 40 | 10.0 | NO | 67,908 | 360 | 360 | 394 |
|  |  | Low Flow Showerheads (IU) | FALSE | Each | 29 | 10.0 | NO | 56,911 | 358 | 359 | 392 |
|  |  | Programmable Thermostats | FALSE | Each | 2 | 16.0 | NO | - | 81 | 81 | 84 |
|  | PTA | Pipe Insulation | FALSE | LN FT | 1,240 | 15.0 | NO | - | 3,966 | 3,966 | 3,780 |
|  | Prescriptive | Boiler Reset Controls | FALSE | MBH Input | 5,507 | 16.0 | NO | - | 7,318 | 7,318 | 6,895 |
|  |  | Boiler Tune Up | FALSE | MBH Input | 5,507 | 3.0 | NO | - | 2,581 | 2,581 | 2,432 |
| DAC | PTA | Boiler Tune Up – DAC | TRUE | MBH Input | 1,028 | 3.0 | NO | - | 482 | 482 | 482 |
|  |  | DHW Boiler Tune-up – DAC | TRUE | Living Units | 10 | 3.0 | NO | - | 28 | 28 | 28 |
|  |  | Pipe Insulation – DAC | TRUE | LN FT | 197 | 15.0 | NO | - | 584 | 584 | 584 |
| Total or Weighted Average |  |  |  |  |  | 12.9 |  | 131,839 | 15,789 | 15,789 | 15,105 |

Source: Evaluation team analysis.

Net to Gross is the deemed value available on the SAG website: https://www.ilsag.info/evaluator-ntg-recommendations-for-2025/

NOAA Climate Data Online https://www.ncei.noaa.gov/cdo-web/

Ibid