# Single Family Income Eligible Program Impact Evaluation Report

Energy Efficiency Plan: Program Year 2025

(01/01/2025-12/31/2025)

Prepared for:

Peoples Gas and North Shore Gas

DRAFT

March 18, 2026

Prepared by:

| Yeab Lakew Guidehouse | Aidan Lewis Guidehouse |  |  |
| --- | --- | --- | --- |

Submitted to:

Peoples Gas

North Shore Gas

200 East Randolph Street

Chicago, IL 60601

Submitted by:

Guidehouse Inc.

167 N. Green Street

12th Floor

Chicago, IL 60607

Contact:

| Charles Maglione, Partner 202.481.7352 cmaglione@guidehouse.com | Jeff Erickson, Director 608.616.4962 jeff.erickson@guidehouse.com | Laura Agapay-Read, Associate Director 312.583.4178 laura.agapay.read@guidehouse.com |
| --- | --- | --- |

Charles Ampong, Associate Director

608.616.4922

charles.ampong@guidehouse.com

This report was prepared by Guidehouse for People Gas Light and Coke Company (PGL)and North Shore Gas(NSG) companies. The work presented in this report represents Guidehouse’s professional judgment based on the information available at the time this report was prepared. Use of this report by any other party for whatever purpose should not, and does not, absolve such party from using due diligence in verifying the report’s contents. Neither Guidehouse nor any of its subsidiaries or affiliates assumes any liability or duty of care to such parties and hereby disclaims any such liability.

Table of Contents

1	Introduction	1

2	Program Description	1

3	Program Savings Detail	4

4	Program Savings by Measure	5

5	Impact Analysis Findings and Recommendations	9

5.1	Impact Parameter Estimates	9

5.2	Findings and Recommendations	10

Appendix A. Impact Analysis Methodology	10

Appendix B. Program Specific Inputs for the Illinois TRC	11

Table of Tables, Figures, and Equations

Table 1. 2025 Volumetric Summary for PGL	1

Table 2. 2025 Installed Measure Quantities for PGL	2

Table 3. 2025 Volumetric Summary for NSG	3

Table 4. 2025 Installed Measure Quantities for NSG	4

Table 5. 2025 Annual Energy Savings Summary for PGL	5

Table 6. 2025 Annual Energy Savings Summary for NSG	6

Table 7. 2025 Annual Energy Savings by Measure for PGL	6

Table 8. 2025 Annual Energy Savings by Measure for NSG	8

Table 9. Verified Gross Savings Parameters	9

# Introduction

This report presents the results of the impact evaluation of the Peoples Gas (PGL) and North Shore Gas (NSG) 2025 Single Family Income Eligible (SFIE) program. It presents a summary of the energy impacts for the total program and broken out by relevant measure and program structure details. The appendices present the impact analysis methodology. Program year 2025 covers January 1 to December 31, 2025.

# Program Description

The SFIE program offers installation of energy efficiency measures for qualifying homeowners with a household income at or below 80% of the area median income (AMI). Eligible measures include programmable and advanced thermostats, water efficiency devices, weatherization measures, pipe insulation, heating and cooling equipment, and custom energy-saving measures for eligible properties. The program also offers installation of health and safety measures, including vents, electrical repairs, and asbestos and mold remediation.

The SFIE program has four paths:

- The SFIE Retrofit offers no-cost direct install products and weatherization services for customers. The PGL and NSG Retrofit path is delivered jointly with ComEd through approved agencies or partners (e.g., Chicago Bungalow Association) that perform outreach and intake.

- The Single Family – Illinois Home Weatherization Assistance Program (IHWAP) is administered by ComEd, PGL, NSG, and Nicor Gas. Weatherization services are provided to low-income residents through local community action agencies or not for profit agencies.

- The Home Energy Assessment (HEA) path is administered by ComEd, Nicor Gas, PGL, and NSG. The HEA offers energy-saving products installed in homes at no-cost to the customer. Homeowners can reduce their energy and water use with the installation of products available to owners of single-family homes, two-flats, and individually metered condos and townhomes. Renters are also eligible with permission from their landlords.

- The SFIE HEA offer includes the Self-Assessment Program (SAP) where a customer completes an online survey and has a kit of products shipped to their home for direct installation.

The program had 2,801 PGL participants in 2025 and completed 2,812 projects as shown in the following table.

Table 1. 2025 Volumetric Summary for PGL

| Participation | HEA | IHWAP | Retrofits | SAP | Total |
| --- | --- | --- | --- | --- | --- |
| Participants * | 2,146 | 49 | 475 | 131 | 2,801 |
| Installed Projects † | 2,153 | 49 | 479 | 131 | 2,812 |
| Measure Types Installed ‡ | 9 | 16 | 15 | 8 | 20 |

* Participants are defined as unique site addresses.

† Installed Projects are defined as unique gas account numbers.

‡ Measure Types Installed are the distinct count of measures.

Source: North Shore Gas tracking data and Guidehouse evaluation team analysis.

Table 2 summarizes the installed measure quantities that are the basis for verified energy savings for PGL.

Table 2. 2025 Installed Measure Quantities for PGL

| Program Path | Measure | Quantity Unit | Installed Quantity |
| --- | --- | --- | --- |
| HEA | Advanced Thermostats | Unit | 690 |
| HEA | Air Handler Filter Cleaning/Replacement | Unit | 1,488 |
| HEA | Air Sealing - Method 2 | Unit | 1,421 |
| HEA | Boiler Pipe Insulation | Unit | 243 |
| HEA | Domestic Hot Water Pipe Insulation | LN FT | 2,754 |
| HEA | Low Flow Faucet Aerators | Unit | 1,021 |
| HEA | Low Flow Showerheads | Unit | 1,221 |
| HEA | Programmable Thermostats | Unit | 506 |
| HEA | Water Heater Temperature Setback | Unit | 1,580 |
| IHWAP | Advanced Thermostats | Unit | 15 |
| IHWAP | Air Sealing - Method 1 | Unit | 49 |
| IHWAP | Basement Sidewall Insulation | SQ FT | 1,057 |
| IHWAP | Ceiling/Attic Insulation | SQ FT | 36,904 |
| IHWAP | Domestic Hot Water Pipe Insulation | LN FT | 419 |
| IHWAP | Duct Insulation and Sealing | Unit | 6 |
| IHWAP | Floor Insulation Above Crawlspace | SQ FT | 316 |
| IHWAP | Gas High Efficiency Boiler | Unit | 7 |
| IHWAP | Gas High Efficiency Furnace | Unit | 35 |
| IHWAP | Gas Water Heater | Unit | 26 |
| IHWAP | Low Flow Faucet Aerators | Unit | 21 |
| IHWAP | Low Flow Showerheads | Unit | 11 |
| IHWAP | Programmable Thermostats | Unit | 15 |
| IHWAP | Residential Furnace Tune Up | Unit | 1 |
| IHWAP | Rim/Band Joist Insulation | Unit | 371 |
| IHWAP | Wall Insulation | SQ FT | 39,837 |
| Retrofits | Advanced Thermostats | Unit | 21 |
| Retrofits | Air Sealing - Method 1 | Unit | 195 |
| Retrofits | Air Sealing - Method 2 | Unit | 45,158 |
| Retrofits | Basement Sidewall Insulation | SQ FT | 13,557 |
| Retrofits | Ceiling/Attic Insulation | SQ FT | 226,850 |
| Retrofits | Domestic Hot Water Pipe Insulation | LN FT | 27 |
| Retrofits | Duct Insulation and Sealing | Unit | 43 |
| Retrofits | Floor Insulation Above Crawlspace | SQ FT | 1,896 |
| Retrofits | Gas High Efficiency Boiler | Unit | 8 |
| Retrofits | Gas High Efficiency Furnace | Unit | 16 |
| Retrofits | Gas Water Heater | Unit | 8 |
| Retrofits | Programmable Thermostats | Unit | 22 |
| Retrofits | Residential Furnace Tune Up | Unit | 81 |
| Retrofits | Rim/Band Joist Insulation | Unit | 5,097 |
| Retrofits | Wall Insulation | SQ FT | 257,024 |
| SAP | Advanced Thermostats | Unit | 67 |
| SAP | Air Sealing - Method 2 | Unit | 174 |
| SAP | Boiler Pipe Insulation | Unit | 36 |
| SAP | Domestic Hot Water Pipe Insulation | LN FT | 192 |
| SAP | Low Flow Faucet Aerators | Unit | 272 |
| SAP | Low Flow Showerheads | Unit | 193 |
| SAP | Programmable Thermostats | Unit | 10 |
| SAP | Water Heater Temperature Setback | Unit | 31 |

Source: Peoples Gas tracking data and evaluation team analysis.

The program had 378 NSG participants in 2025 and completed 380 projects as shown in the following table.

Table 3. 2025 Volumetric Summary for NSG

| Participation | HEA - Joint | IHWAP | Retrofits | SAP | HEA - Gas Only | Total |
| --- | --- | --- | --- | --- | --- | --- |
| Participants * | 135 | 3 | 51 | 10 | 179 | 378 |
| Installed Projects † | 135 | 3 | 52 | 10 | 180 | 380 |
| Measure Types Installed ‡ | 8 | 12 | 12 | 6 | 4 | 18 |

* Participants are defined as unique site addresses.

† Installed Projects are defined as unique gas account numbers.

‡ Measure Types Installed are the distinct count of measures.

Source: North Shore Gas tracking data and Guidehouse evaluation team analysis.

Table 4 summarizes the installed measure quantities that are the basis for verified energy savings for NSG.

Table 4. 2025 Installed Measure Quantities for NSG

| Program Path | Measure | Quantity Unit | Installed Quantity |
| --- | --- | --- | --- |
| HEA - Joint | Advanced Thermostats | Unit | 48 |
| HEA - Joint | Air Handler Filter Cleaning/Replacement | Unit | 83 |
| HEA - Joint | Air Sealing - Method 2 | Unit | 80 |
| HEA - Joint | Domestic Hot Water Pipe Insulation | LN FT | 162 |
| HEA - Joint | Low Flow Faucet Aerators | Unit | 79 |
| HEA - Joint | Low Flow Showerheads | Unit | 58 |
| HEA - Joint | Programmable Thermostats | Unit | 24 |
| HEA - Joint | Water Heater Temperature Setback | Unit | 86 |
| IHWAP | Advanced Thermostats | Unit | 2 |
| IHWAP | Air Sealing - Method 1 | Unit | 3 |
| IHWAP | Basement Sidewall Insulation | SQ FT | 120 |
| IHWAP | Ceiling/Attic Insulation | SQ FT | 344 |
| IHWAP | Domestic Hot Water Pipe Insulation | LN FT | 24 |
| IHWAP | Gas High Efficiency Boiler | Unit | 1 |
| IHWAP | Gas High Efficiency Furnace | Unit | 2 |
| IHWAP | Gas Water Heater | Unit | 2 |
| IHWAP | Low Flow Faucet Aerators | Unit | 2 |
| IHWAP | Low Flow Showerheads | Unit | 1 |
| IHWAP | Rim/Band Joist Insulation | Unit | 94 |
| IHWAP | Wall Insulation | SQ FT | 1,680 |
| Retrofits | Advanced Thermostats | Unit | 1 |
| Retrofits | Air Sealing - Method 1 | Unit | 27 |
| Retrofits | Air Sealing - Method 2 | Unit | 1,277 |
| Retrofits | Basement Sidewall Insulation | SQ FT | 2,824 |
| Retrofits | Ceiling/Attic Insulation | SQ FT | 31,683 |
| Retrofits | Duct Insulation and Sealing | Unit | 9 |
| Retrofits | Gas High Efficiency Furnace | Unit | 4 |
| Retrofits | Gas Water Heater | Unit | 3 |
| Retrofits | Programmable Thermostats | Unit | 1 |
| Retrofits | Residential Furnace Tune Up | Unit | 3 |
| Retrofits | Rim/Band Joist Insulation | Unit | 4,352 |
| Retrofits | Wall Insulation | SQ FT | 2,793 |
| SAP | Advanced Thermostats | Unit | 6 |
| SAP | Air Sealing - Method 2 | Unit | 9 |
| SAP | Domestic Hot Water Pipe Insulation | LN FT | 12 |
| SAP | Low Flow Faucet Aerators | Unit | 16 |
| SAP | Low Flow Showerheads | Unit | 14 |
| SAP | Water Heater Temperature Setback | Unit | 3 |
| HEA - Gas Only | Advanced Thermostats | Unit | 1 |
| HEA - Gas Only | Air Handler Filter Cleaning/Replacement | Unit | 3 |
| HEA - Gas Only | Air Sealing - Method 2 | Unit | 2 |
| HEA - Gas Only | Water Heater Temperature Setback | Unit | 1 |

Source: North Shore Gas tracking data and Guidehouse evaluation team analysis

# Program Savings Detail

Table 5 summarizes the energy savings the PGL SFIE Program achieved by path in 2025.

Table 5. 2025 Annual Energy Savings Summary for PGL

| Program Path | Ex Ante Gross Savings (Therms) | Verified Gross RR* | Verified Gross Savings (Therms) | NTG† | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- |
| HEA | 139,051 | 100% | 139,048 | 1.0 | 140,286 |
| IHWAP | 40,568 | 100% | 40,565 | 1.0 | 40,565 |
| Retrofits | 148,680 | 100% | 148,775 | 1.0 | 148,961 |
| SAP | 7,620 | 99% | 7,539 | 1.0 | 7,919 |
| Total | 335,919 | 100% | 335,927 | 1.0 | 335,927 |

* Realization Rate (RR) is the ratio of verified gross savings to ex ante gross savings, based on evaluation research findings.

† NTG, Net to Gross is the deemed value available on the SAG website: https://www.ilsag.info/evaluator-ntg-recommendations-for-2025/.

Source: Peoples Gas tracking data and evaluation team analysis.

Table 6 summarizes the energy savings the NSG SFIE Program achieved by path in 2025.

Table 6. 2025 Annual Energy Savings Summary for NSG

| Program Path | Ex Ante Gross Savings (Therms) | Verified Gross RR* | Verified Gross Savings (Therms) | NTG† | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- |
| HEA - Joint | 7,751 | 100% | 7,736 | 1.0 | 7,736 |
| IHWAP | 1,940 | 100% | 1,940 | 1.0 | 1,940 |
| Retrofits | 10,609 | 100% | 10,616 | 1.0 | 10,616 |
| SAP | 597 | 100% | 596 | 1.0 | 596 |
| HEA - Gas Only | 156 | 100% | 156 | 1.0 | 156 |
| Total | 21,053 | 100% | 21,044 | 1.0 | 21,044 |

* Realization Rate (RR) is the ratio of verified gross savings to ex ante gross savings, based on evaluation research findings.

† NTG, Net to Gross is the deemed value available on the SAG website: https://www.ilsag.info/evaluator-ntg-recommendations-for-2025/.

Source: North Shore Gas tracking data and Guidehouse evaluation team analysis.

# Program Savings by Measure

The program includes 20 measures as shown in the following table. The Advanced Thermostats and Ceiling/Attic Insulation measures contributed the most savings.

Table 7. 2025 Annual Energy Savings by Measure for PGL

| Program Path | Savings Category | Ex Ante Gross Savings (Therms) | Verified Gross RR* | Verified Gross Savings (Therms) | NTG† | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- |
| HEA | Advanced Thermostats | 55,220 | 100% | 55,218 | 1.0 | 55,218 |
| HEA | Air Handler Filter Cleaning/Replacement | 11,725 | 100% | 11,725 | 1.0 | 11,725 |
| HEA | Air Sealing - Method 2 | 10,379 | 100% | 10,379 | 1.0 | 10,379 |
| HEA | Boiler Pipe Insulation | 105 | 100% | 105 | 1.0 | 105 |
| HEA | Domestic Hot Water Pipe Insulation | 5,237 | 100% | 5,237 | 1.0 | 5,237 |
| HEA | Low Flow Faucet Aerators | 5,123 | 100% | 5,124 | 1.0 | 5,124 |
| HEA | Low Flow Showerheads | 12,255 | 100% | 12,255 | 1.0 | 12,255 |
| HEA | Programmable Thermostats | 38,610 | 100% | 38,610 | 1.0 | 38,610 |
| HEA | Water Heater Temperature Setback | 398 | 100% | 397 | 1.0 | 397 |
| IHWAP | Advanced Thermostats | 1,325 | 100% | 1,325 | 1.0 | 1,325 |
| IHWAP | Air Sealing - Method 1 | 6,755 | 100% | 6,755 | 1.0 | 6,755 |
| IHWAP | Basement Sidewall Insulation | 285 | 100% | 285 | 1.0 | 285 |
| IHWAP | Ceiling/Attic Insulation | 8,201 | 100% | 8,201 | 1.0 | 8,201 |
| IHWAP | Domestic Hot Water Pipe Insulation | 845 | 100% | 845 | 1.0 | 845 |
| IHWAP | Duct Insulation and Sealing | 1,084 | 100% | 1,081 | 1.0 | 1,081 |
| IHWAP | Floor Insulation Above Crawlspace | 45.29 | 100% | 45.29 | 1.0 | 45.29 |
| IHWAP | Gas High Efficiency Boiler | 3,336 | 100% | 3,336 | 1.0 | 3,336 |
| IHWAP | Gas High Efficiency Furnace | 12,214 | 100% | 12,215 | 1.0 | 12,215 |
| IHWAP | Gas Water Heater | 1,309 | 100% | 1,309 | 1.0 | 1,309 |
| IHWAP | Low Flow Faucet Aerators | 48.84 | 100% | 48.84 | 1.0 | 48.84 |
| IHWAP | Low Flow Showerheads | 23.31 | 100% | 23.31 | 1.0 | 23.31 |
| IHWAP | Programmable Thermostats | 1,047 | 100% | 1,047 | 1.0 | 1,047 |
| IHWAP | Residential Furnace Tune Up | 10.58 | 100% | 10.58 | 1.0 | 10.58 |
| IHWAP | Rim/Band Joist Insulation | 57.92 | 100% | 57.92 | 1.0 | 57.92 |
| IHWAP | Wall Insulation | 3,980 | 100% | 3,980 | 1.0 | 3,980 |
| Retrofits | Advanced Thermostats | 1,663 | 100% | 1,663 | 1.0 | 1,663 |
| Retrofits | Air Sealing - Method 1 | 35,890 | 100% | 35,890 | 1.0 | 35,890 |
| Retrofits | Air Sealing - Method 2 | 20,379 | 100% | 20,379 | 1.0 | 20,379 |
| Retrofits | Basement Sidewall Insulation | 1,293 | 100% | 1,293 | 1.0 | 1,293 |
| Retrofits | Ceiling/Attic Insulation | 43,676 | 100% | 43,676 | 1.0 | 43,676 |
| Retrofits | Domestic Hot Water Pipe Insulation | 54.43 | 100% | 54.43 | 1.0 | 54.43 |
| Retrofits | Duct Insulation and Sealing | 3,473 | 103% | 3,585 | 1.0 | 3,585 |
| Retrofits | Floor Insulation Above Crawlspace | 266 | 100% | 266 | 1.0 | 266 |
| Retrofits | Gas High Efficiency Boiler | 1,580 | 100% | 1,580 | 1.0 | 1,580 |
| Retrofits | Gas High Efficiency Furnace | 3,599 | 100% | 3,599 | 1.0 | 3,599 |
| Retrofits | Gas Water Heater | 262 | 100% | 262 | 1.0 | 262 |
| Retrofits | Programmable Thermostats | 1,486 | 100% | 1,486 | 1.0 | 1,486 |
| Retrofits | Residential Furnace Tune Up | 2,263 | 99% | 2,246 | 1.0 | 2,246 |
| Retrofits | Rim/Band Joist Insulation | 535 | 100% | 535 | 1.0 | 535 |
| Retrofits | Wall Insulation | 32,259 | 100% | 32,259 | 1.0 | 32,259 |
| SAP | Advanced Thermostats | 4,289 | 100% | 4,287 | 1.0 | 4,287 |
| SAP | Air Sealing - Method 2 | 864 | 100% | 864 | 1.0 | 864 |
| SAP | Boiler Pipe Insulation | 16.46 | 100% | 16.46 | 1.0 | 16.46 |
| SAP | Domestic Hot Water Pipe Insulation | 387 | 100% | 387 | 1.0 | 387 |
| SAP | Low Flow Faucet Aerators | 799 | 96% | 768 | 1.0 | 768 |
| SAP | Low Flow Showerheads | 772 | 94% | 724 | 1.0 | 724 |
| SAP | Programmable Thermostats | 484 | 100% | 484 | 1.0 | 484 |
| SAP | Water Heater Temperature Setback | 7.80 | 100% | 7.80 | 1.0 | 7.80 |
| Total |  | 335,919 | 100% | 335,927 | 1.0 | 335,927 |

Source: Peoples Gas tracking data and evaluation team analysis.

Table 8. 2025 Annual Energy Savings by Measure for NSG

| Program Path | Savings Category | Ex Ante Gross Savings (Therms) | Verified Gross RR* | Verified Gross Savings (Therms) | NTG† | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- |
| HEA - Joint | Advanced Thermostats | 3,456 | 100% | 3,456 | 1.0 | 3,456 |
| HEA - Joint | Air Handler Filter Cleaning/Replacement | 658 | 100% | 658 | 1.0 | 658 |
| HEA - Joint | Air Sealing - Method 2 | 584 | 100% | 584 | 1.0 | 584 |
| HEA - Joint | Domestic Hot Water Pipe Insulation | 308 | 100% | 308 | 1.0 | 308 |
| HEA - Joint | Low Flow Faucet Aerators | 284 | 100% | 285 | 1.0 | 285 |
| HEA - Joint | Low Flow Showerheads | 640 | 98% | 626 | 1.0 | 626 |
| HEA - Joint | Programmable Thermostats | 1,796 | 100% | 1,797 | 1.0 | 1,797 |
| HEA - Joint | Water Heater Temperature Setback | 21.65 | 100% | 21.65 | 1.0 | 21.65 |
| IHWAP | Advanced Thermostats | 190 | 100% | 190 | 1.0 | 190 |
| IHWAP | Air Sealing - Method 1 | 667 | 100% | 667 | 1.0 | 667 |
| IHWAP | Basement Sidewall Insulation | 113 | 100% | 113 | 1.0 | 113 |
| IHWAP | Ceiling/Attic Insulation | 47.59 | 100% | 47.59 | 1.0 | 47.59 |
| IHWAP | Domestic Hot Water Pipe Insulation | 49.89 | 100% | 49.89 | 1.0 | 49.89 |
| IHWAP | Gas High Efficiency Boiler | 230 | 100% | 230 | 1.0 | 230 |
| IHWAP | Gas High Efficiency Furnace | 315 | 100% | 315 | 1.0 | 315 |
| IHWAP | Gas Water Heater | 56.99 | 100% | 56.99 | 1.0 | 56.99 |
| IHWAP | Low Flow Faucet Aerators | 8.11 | 100% | 8.11 | 1.0 | 8.11 |
| IHWAP | Low Flow Showerheads | 2.39 | 100% | 2.39 | 1.0 | 2.39 |
| IHWAP | Rim/Band Joist Insulation | 15.82 | 100% | 15.82 | 1.0 | 15.82 |
| IHWAP | Wall Insulation | 245 | 100% | 245 | 1.0 | 245 |
| Retrofits | Advanced Thermostats | 71.36 | 100% | 71.36 | 1.0 | 71.36 |
| Retrofits | Air Sealing - Method 1 | 3,355 | 100% | 3,356 | 1.0 | 3,356 |
| Retrofits | Air Sealing - Method 2 | 531 | 100% | 531 | 1.0 | 531 |
| Retrofits | Basement Sidewall Insulation | 434 | 100% | 434 | 1.0 | 434 |
| Retrofits | Ceiling/Attic Insulation | 2,929 | 100% | 2,929 | 1.0 | 2,929 |
| Retrofits | Duct Insulation and Sealing | 1,159 | 100% | 1,161 | 1.0 | 1,161 |
| Retrofits | Gas High Efficiency Furnace | 661 | 100% | 661 | 1.0 | 661 |
| Retrofits | Gas Water Heater | 108 | 110% | 120 | 1.0 | 120 |
| Retrofits | Programmable Thermostats | 62.31 | 100% | 62.31 | 1.0 | 62.31 |
| Retrofits | Residential Furnace Tune Up | 175 | 96% | 169 | 1.0 | 169 |
| Retrofits | Rim/Band Joist Insulation | 806 | 100% | 806 | 1.0 | 806 |
| Retrofits | Wall Insulation | 317 | 100% | 317 | 1.0 | 317 |
| SAP | Advanced Thermostats | 415 | 100% | 414 | 1.0 | 414 |
| SAP | Air Sealing - Method 2 | 44.70 | 100% | 44.70 | 1.0 | 44.70 |
| SAP | Domestic Hot Water Pipe Insulation | 24.19 | 100% | 24.19 | 1.0 | 24.19 |
| SAP | Low Flow Faucet Aerators | 51.98 | 100% | 51.79 | 1.0 | 51.79 |
| SAP | Low Flow Showerheads | 60.52 | 100% | 60.31 | 1.0 | 60.31 |
| SAP | Water Heater Temperature Setback | 0.79 | 95% | 0.76 | 1.0 | 0.76 |
| HEA - Gas Only | Advanced Thermostats | 103 | 100% | 103 | 1.0 | 103 |
| HEA - Gas Only | Air Handler Filter Cleaning/ Replacement | 38.68 | 100% | 38.68 | 1.0 | 38.68 |
| HEA - Gas Only | Air Sealing - Method 2 | 14.61 | 100% | 14.61 | 1.0 | 14.61 |
| HEA - Gas Only | Water Heater Temperature Setback | 0.25 | 100% | 0.25 | 1.0 | 0.25 |
| Total or Weighted Average |  | 21,053 | 100% | 21,044 | 1.0 | 21,044 |

Source: North Shore Gas tracking data and Guidehouse evaluation team analysis.

# Impact Analysis Findings and Recommendations

## Impact Parameter Estimates

Table 9 shows the unit per therm savings and realization rate findings by measure from our review. The realization rate is the ratio of the verified savings to the ex ante savings. Following the table, we provide findings and recommendations, including discussion of all measures with realization rates above or below 100%. Appendix A provides a description of the impact analysis methodology.

Table 9. Verified Gross Savings Parameters

| Measure | Unit Basis | Ex Ante Gross (therms/unit) | Verified Gross (therms/unit) | Realization Rate | IL-TRM v13.0 Section |
| --- | --- | --- | --- | --- | --- |
| Advanced Thermostats | Unit | 78.42 | 78.41 | 100% | 5.3.16 |
| Air Handler Filter Cleaning/Replacement | Unit | 7.89 | 7.89 | 100% | 5.3.10 |
| Air Sealing - Method 1 | Unit | 170.32 | 170.32 | 100% | 5.6.1 |
| Air Sealing - Method 2 | Unit | 0.68 | 0.68 | 100% | 5.6.1 |
| Basement Sidewall Insulation | SQ FT | 0.12 | 0.12 | 100% | 5.6.2 |
| Boiler Pipe Insulation | Unit | 0.43 | 0.43 | 100% | 5.3.2 |
| Ceiling/Attic Insulation | SQ FT | 0.19 | 0.19 | 100% | 5.6.5 |
| Domestic Hot Water Pipe Insulation | LN FT | 1.92 | 1.92 | 100% | 5.4.1 |
| Duct Insulation and Sealing | Unit | 98.55 | 100.46 | 102% | 5.3.4 |
| Floor Insulation Above Crawlspace | SQ FT | 0.14 | 0.14 | 100% | 5.6.3 |
| Gas High Efficiency Boiler | Unit | 321.65 | 321.65 | 100% | 5.3.6 |
| Gas High Efficiency Furnace | Unit | 294.53 | 294.54 | 100% | 5.3.7 |
| Gas Water Heater | Unit | 44.54 | 44.83 | 101% | 5.4.2 |
| Low Flow Faucet Aerators | Unit | 4.48 | 4.45 | 100% | 5.4.4 |
| Low Flow Showerheads | Unit | 9.18 | 9.14 | 100% | 5.4.5 |
| Programmable Thermostats | Unit | 75.24 | 75.24 | 100% | 5.3.11 |
| Residential Furnace Tune Up | Unit | 28.81 | 28.54 | 99% | 5.3.13 |
| Rim/Band Joist Insulation | Unit | 0.14 | 0.14 | 100% | 5.6.6 |
| Wall Insulation | SQ FT | 0.12 | 0.12 | 100% | 5.6.4 |
| Water Heater Temperature Setback | Unit | 0.25 | 0.25 | 100% | 5.4.6 |

Source: Program Tracking Data (PTD) provided by Peoples and North Shore Gas, extract dated January 30, 2026 and Evaluation analysis.

† State of Illinois Technical Reference Manual version 13.0 from http://www.ilsag.info/technical-reference-manual.html.

## Findings and Recommendations

Finding 1.  The tracking data did not include values for the Pre Duct Sealing Heating System Efficiency (ηSystem) input for Duct Insulation and Sealing measures. The evaluation team used the IL TRM v13 default value 0.7 for ηSystem, however 37 out of the 58 Duct Insulation and Sealing measures have ex ante savings that do not reflect this value, with a realization rate between 54% and 137%.

Recommendation 1.  Ensure the tracking data includes ηSystem values used to calculate ex-ante savings.

Finding 2. The evaluation team was unable to replicate ex ante savings for one Gas Water Heater measure (measure ID MEA-2024.11.11-806823). The evaluation team used the algorithm and determined values found in the IL TRM v13, however this instance had a realization rate of 119%.

Recommendation 2.  Ensure ex ante savings is using variables provided in the IL TRM v13 and tracking data.

# Appendix A. Impact Analysis Methodology

Impact Analysis Supplemental Information

The evaluation team determined the verified gross savings for each program measure by the following methodology:

- Reviewing the savings algorithm inputs in the measure workbook with the IL TRM v13 and IL -TRM Errata, when applicable.

- Referencing Resource Innovation’s SFHU Savings Calculator to validate the savings algorithm was applied correctly.

- Cross-checking ComEd’s 2025 year-end data to validate missing data.

# Appendix B. Program Specific Inputs for the Illinois TRC

The table below shows the Total Resource Cost (TRC) cost-effectiveness analysis inputs available at the time of producing this impact evaluation report. Additional required cost data (e.g., measure costs, program level incentive and non-incentive costs) are not included in this table and will be provided to the evaluation team later. Guidehouse will include annual and lifetime water savings and greenhouse gas reductions in the end of year summary report.

Table B-1. Verified Cost Effectiveness Inputs

| Program Path | Savings Category | DAC Project | Units | Quantity | Effective Useful Life | Early Replacement Flag† | Verified Gross Annual Water Savings (Gallons) | Ex Ante Gross Savings (Therms) | Verified Gross Savings (Therms) | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HEA | Advanced Thermostats | FALSE | Unit | 690 | 11.00 | NO |  | 55,220 | 55,218 | 55,218 |
| HEA | Air Handler Filter Cleaning/Replacement | FALSE | Unit | 1,488 | 3.00 | NO |  | 11,725 | 11,725 | 11,725 |
| HEA | Air Sealing - Method 2 | FALSE | Unit | 1,421 | 20.00 | YES |  | 10,379 | 10,379 | 10,379 |
| HEA | Boiler Pipe Insulation | FALSE | Unit | 243 | 11.00 | YES |  | 105 | 105 | 105 |
| HEA | Domestic Hot Water Pipe Insulation | FALSE | Ln Ft | 2,754 | 15.00 | NO |  | 5,237 | 5,237 | 5,237 |
| HEA | Low Flow Faucet Aerators | FALSE | Unit | 1,021 | 10.00 | NO | 1,161,100 | 5,123 | 5,124 | 5,124 |
| HEA | Low Flow Showerheads | FALSE | Unit | 1,221 | 10.00 | NO | 2,269,502 | 12,255 | 12,255 | 12,255 |
| HEA | Programmable Thermostats | FALSE | Unit | 506 | 16.00 | YES |  | 38,610 | 38,610 | 38,610 |
| HEA | Water Heater Temperature Setback | FALSE | Unit | 1,580 | 2.00 | NO |  | 398 | 397 | 397 |
| IHWAP | Advanced Thermostats | FALSE | Unit | 15.00 | 11.00 | NO |  | 1,325 | 1,325 | 1,325 |
| IHWAP | Air Sealing - Method 1 | FALSE | Unit | 49.00 | 20.00 | YES |  | 6,755 | 6,755 | 6,755 |
| IHWAP | Basement Sidewall Insulation | FALSE | Sq Ft | 1,057 | 30.00 | YES |  | 285 | 285 | 285 |
| IHWAP | Ceiling/Attic Insulation | FALSE | Sq Ft | 36,904 | 30.00 | YES |  | 8,201 | 8,201 | 8,201 |
| IHWAP | Domestic Hot Water Pipe Insulation | FALSE | Ln Ft | 419 | 15.00 | NO |  | 845 | 845 | 845 |
| IHWAP | Duct Insulation and Sealing | FALSE | Unit | 6.00 | 20.00 | YES |  | 1,084 | 1,081 | 1,081 |
| IHWAP | Floor Insulation Above Crawlspace | FALSE | Sq Ft | 316 | 30.00 | YES |  | 45.29 | 45.29 | 45.29 |
| IHWAP | Gas High Efficiency Boiler | FALSE | Unit | 7.00 | 25.00 | YES |  | 3,336 | 3,336 | 3,336 |
| IHWAP | Gas High Efficiency Furnace | FALSE | Unit | 35.00 | 20.00 | YES |  | 12,214 | 12,215 | 12,215 |
| IHWAP | Gas Water Heater | FALSE | Unit | 26.00 | 14.50 | YES |  | 1,309 | 1,309 | 1,309 |
| IHWAP | Low Flow Faucet Aerators | FALSE | Unit | 21.00 | 10.00 | NO | 11,428 | 48.84 | 48.84 | 48.84 |
| IHWAP | Low Flow Showerheads | FALSE | Unit | 11.00 | 10.00 | NO | 4,317 | 23.31 | 23.31 | 23.31 |
| IHWAP | Programmable Thermostats | FALSE | Unit | 15.00 | 16.00 | YES |  | 1,047 | 1,047 | 1,047 |
| IHWAP | Residential Furnace Tune Up | FALSE | Unit | 1.00 | 3.00 | NO |  | 10.58 | 10.58 | 10.58 |
| IHWAP | Rim/Band Joist Insulation | FALSE | Unit | 371 | 30.00 | YES |  | 57.92 | 57.92 | 57.92 |
| IHWAP | Wall Insulation | FALSE | Sq Ft | 39,837 | 30.00 | YES |  | 3,980 | 3,980 | 3,980 |
| Retrofits | Advanced Thermostats | FALSE | Unit | 21.00 | 11.00 | NO |  | 1,663 | 1,663 | 1,663 |
| Retrofits | Air Sealing - Method 1 | FALSE | Unit | 195 | 20.00 | YES |  | 35,890 | 35,890 | 35,890 |
| Retrofits | Air Sealing - Method 2 | FALSE | Unit | 45,158 | 20.00 | NO |  | 20,379 | 20,379 | 20,379 |
| Retrofits | Basement Sidewall Insulation | FALSE | Sq Ft | 13,557 | 30.00 | YES |  | 1,293 | 1,293 | 1,293 |
| Retrofits | Ceiling/Attic Insulation | FALSE | Sq Ft | 226,850 | 30.00 | YES |  | 43,676 | 43,676 | 43,676 |
| Retrofits | Domestic Hot Water Pipe Insulation | FALSE | Ln Ft | 27.00 | 15.00 | NO |  | 54.43 | 54.43 | 54.43 |
| Retrofits | Duct Insulation and Sealing | FALSE | Unit | 43.00 | 20.00 | YES |  | 3,473 | 3,585 | 3,585 |
| Retrofits | Floor Insulation Above Crawlspace | FALSE | Sq Ft | 1,896 | 30.00 | YES |  | 266 | 266 | 266 |
| Retrofits | Gas High Efficiency Boiler | FALSE | Unit | 8.00 | 25.00 | NO |  | 1,580 | 1,580 | 1,580 |
| Retrofits | Gas High Efficiency Furnace | FALSE | Unit | 16.00 | 20.00 | NO |  | 3,599 | 3,599 | 3,599 |
| Retrofits | Gas Water Heater | FALSE | Unit | 8.00 | 14.50 | YES |  | 262 | 262 | 262 |
| Retrofits | Programmable Thermostats | FALSE | Unit | 22.00 | 16.00 | YES |  | 1,486 | 1,486 | 1,486 |
| Retrofits | Residential Furnace Tune Up | FALSE | Unit | 81.00 | 3.00 | NO |  | 2,263 | 2,246 | 2,246 |
| Retrofits | Rim/Band Joist Insulation | FALSE | Unit | 5,097 | 30.00 | YES |  | 535 | 535 | 535 |
| Retrofits | Wall Insulation | FALSE | Sq Ft | 257,024 | 30.00 | YES |  | 32,259 | 32,259 | 32,259 |
| SAP | Advanced Thermostats | FALSE | Unit | 67.00 | 11.00 | NO |  | 4,289 | 4,287 | 4,287 |
| SAP | Air Sealing - Method 2 | FALSE | Unit | 174 | 20.00 | NO |  | 864 | 864 | 864 |
| SAP | Boiler Pipe Insulation | FALSE | Unit | 36.00 | 11.00 | YES |  | 16.46 | 16.46 | 16.46 |
| SAP | Domestic Hot Water Pipe Insulation | FALSE | Ln Ft | 192 | 15.00 | NO |  | 387 | 387 | 387 |
| SAP | Low Flow Faucet Aerators | FALSE | Unit | 272 | 10.00 | NO | 184,630 | 799 | 768 | 768 |
| SAP | Low Flow Showerheads | FALSE | Unit | 193 | 10.00 | NO | 142,966 | 772 | 724 | 724 |
| SAP | Programmable Thermostats | FALSE | Unit | 10.00 | 16.00 | YES |  | 484 | 484 | 484 |
| SAP | Water Heater Temperature Setback | FALSE | Unit | 31.00 | 2.00 | NO |  | 7.80 | 7.80 | 7.80 |
| Total or Weighted Average |  |  |  |  | 19.16 |  | 3,773,943 | 335,919 | 335,927 | 335,927 |

Source: Peoples Gas tracking data and evaluation team analysis.

Table B2. Verified Cost Effectiveness Inputs – NSG

| Program Path | Savings Category | DAC Project | Units | Quantity | Effective Useful Life | Early Replacement Flag† | Verified Gross Annual Water Savings (Gallons) | Ex Ante Gross Savings (Therms) | Verified Gross Savings (Therms) | Verified Net Savings (Therms) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HEA - Joint | Advanced Thermostats | FALSE | Unit | 48.00 | 11.00 | NO |  | 3,456 | 3,456 | 3,456 |
| HEA - Joint | Air Handler Filter Cleaning/Replacement | FALSE | Unit | 83.00 | 3.00 | NO |  | 658 | 658 | 658 |
| HEA - Joint | Air Sealing - Method 2 | FALSE | Unit | 80.00 | 20.00 | NO |  | 584 | 584 | 584 |
| HEA - Joint | Domestic Hot Water Pipe Insulation | FALSE | LN FT | 162 | 15.00 | NO |  | 308 | 308 | 308 |
| HEA - Joint | Low Flow Faucet Aerators | FALSE | Unit | 79.00 | 10.00 | NO | 66,277 | 285 | 285 | 285 |
| HEA - Joint | Low Flow Showerheads | FALSE | Unit | 58.00 | 10.00 | NO | 118,656 | 641 | 626 | 626 |
| HEA - Joint | Programmable Thermostats | FALSE | Unit | 24.00 | 16.00 | YES |  | 1,797 | 1,797 | 1,797 |
| HEA - Joint | Water Heater Temperature Setback | FALSE | Unit | 86.00 | 2.00 | NO |  | 21.65 | 21.65 | 21.65 |
| IHWAP | Advanced Thermostats | FALSE | Unit | 2.00 | 11.00 | NO |  | 190 | 190 | 190 |
| IHWAP | Air Sealing - Method 1 | FALSE | Unit | 3.00 | 20.00 | YES |  | 667 | 667 | 667 |
| IHWAP | Basement Sidewall Insulation | FALSE | SQ FT | 120 | 30.00 | YES |  | 113 | 113 | 113 |
| IHWAP | Ceiling/Attic Insulation | FALSE | SQ FT | 344 | 30.00 | YES |  | 47.59 | 47.59 | 47.59 |
| IHWAP | Domestic Hot Water Pipe Insulation | FALSE | LN FT | 24.00 | 15.00 | NO |  | 49.89 | 49.89 | 49.89 |
| IHWAP | Gas High Efficiency Boiler | FALSE | Unit | 1.00 | 25.00 | NO |  | 230 | 230 | 230 |
| IHWAP | Gas High Efficiency Furnace | FALSE | Unit | 2.00 | 20.00 | NO |  | 315 | 315 | 315 |
| IHWAP | Gas Water Heater | FALSE | Unit | 2.00 | 14.50 | YES |  | 56.99 | 56.99 | 56.99 |
| IHWAP | Low Flow Faucet Aerators | FALSE | Unit | 2.00 | 10.00 | NO | 1,834 | 8.11 | 8.11 | 8.11 |
| IHWAP | Low Flow Showerheads | FALSE | Unit | 1.00 | 10.00 | NO | 442 | 2.39 | 2.39 | 2.39 |
| IHWAP | Rim/Band Joist Insulation | FALSE | Unit | 94.00 | 30.00 | YES |  | 15.82 | 15.82 | 15.82 |
| IHWAP | Wall Insulation | FALSE | SQ FT | 1,680 | 30.00 | YES |  | 245 | 245 | 245 |
| Retrofits | Advanced Thermostats | FALSE | Unit | 1.00 | 11.00 | NO |  | 71.36 | 71.36 | 71.36 |
| Retrofits | Air Sealing - Method 1 | FALSE | Unit | 27.00 | 20.00 | YES |  | 3,356 | 3,356 | 3,356 |
| Retrofits | Air Sealing - Method 2 | FALSE | Unit | 1,277 | 20.00 | NO |  | 531 | 531 | 531 |
| Retrofits | Basement Sidewall Insulation | FALSE | SQ FT | 2,824 | 30.00 | YES |  | 434 | 434 | 434 |
| Retrofits | Ceiling/Attic Insulation | FALSE | SQ FT | 31,683 | 30.00 | YES |  | 2,929 | 2,929 | 2,929 |
| Retrofits | Duct Insulation and Sealing | FALSE | Unit | 9.00 | 20.00 | NO |  | 1,159 | 1,161 | 1,161 |
| Retrofits | Gas High Efficiency Furnace | FALSE | Unit | 4.00 | 20.00 | NO |  | 661 | 661 | 661 |
| Retrofits | Gas Water Heater | FALSE | Unit | 3.00 | 14.50 | YES |  | 108 | 120 | 120 |
| Retrofits | Programmable Thermostats | FALSE | Unit | 1.00 | 16.00 | YES |  | 62.31 | 62.31 | 62.31 |
| Retrofits | Residential Furnace Tune Up | FALSE | Unit | 3.00 | 3.00 | NO |  | 175 | 169 | 169 |
| Retrofits | Rim/Band Joist Insulation | FALSE | Unit | 4,352 | 30.00 | YES |  | 806 | 806 | 806 |
| Retrofits | Wall Insulation | FALSE | SQ FT | 2,793 | 30.00 | YES |  | 317 | 317 | 317 |
| SAP | Advanced Thermostats | FALSE | Unit | 6.00 | 11.00 | NO |  | 415 | 414 | 414 |
| SAP | Air Sealing - Method 2 | FALSE | Unit | 9.00 | 20.00 | NO |  | 44.70 | 44.70 | 44.70 |
| SAP | Domestic Hot Water Pipe Insulation | FALSE | LN FT | 12.00 | 15.00 | NO |  | 24.19 | 24.19 | 24.19 |
| SAP | Low Flow Faucet Aerators | FALSE | Unit | 16.00 | 10.00 | NO | 11,602 | 51.98 | 51.79 | 51.79 |
| SAP | Low Flow Showerheads | FALSE | Unit | 14.00 | 10.00 | NO | 10,371 | 60.52 | 60.31 | 60.31 |
| SAP | Water Heater Temperature Setback | FALSE | Unit | 3.00 | 2.00 | NO |  | 0.80 | 0.76 | 0.76 |
| HEA - Gas Only | Advanced Thermostats | FALSE | Unit | 1.00 | 11.00 | NO |  | 103 | 103 | 103 |
| HEA - Gas Only | Air Handler Filter Cleaning/Replacement | FALSE | Unit | 3.00 | 3.00 | NO |  | 38.68 | 38.68 | 38.68 |
| HEA - Gas Only | Air Sealing - Method 2 | FALSE | Unit | 2.00 | 20.00 | NO |  | 14.61 | 14.61 | 14.61 |
| HEA - Gas Only | Water Heater Temperature Setback | FALSE | Unit | 1.00 | 2.00 | NO |  | 0.25 | 0.25 | 0.25 |
| Total or Weighted Average |  |  |  |  | 18.88 |  | 209,182 | 21,053 | 21,044 | 21,044 |

Source: North Shore Gas tracking data and Guidehouse evaluation team analysis.