Programmable Thermostats

Report to KeySpan Energy Delivery on Energy Savings and Cost Effectiveness

December 2002

Prepared by

#### Table of Contents

1.	Introduction	1

1.1.	Description of Sources of Information	2

2.	Energy/Cost Savings from Thermostat Manufacturers	3

3.	Utility Evaluations of Programmable Thermostats	4

3.1.	Northeast Utilities	4

3.2.	Pacific Gas & Electric	4

4.	Pacific Northwest National Laboratory Study	4

5.	Energy Center of Wisconsin Study	5

6.	Conclusions	6

# Introduction

The energy savings associated with installation of programmable thermostats in homes has been a subject of discussion for such research facilities as Lawrence Berkeley Laboratory, Pacific Northwest Laboratory, NYSERDA, ACEEE, the Energy Center of Wisconsin, PG&E and thermostat manufacturers, just to name a few organizations.  The purpose of this report is to document and summarize recent technical reports, information from manufacturers, and program evaluation studies that present estimates of the annual energy savings (and cost effectiveness) resulting from the installation and use of programmable thermostats.  This report presents the following information:

- a summary of energy savings estimates determined by various entities

- benefit-cost analysis results associated with programmable thermostats based upon the studies examined in this report.

GDS collected information from over 25 data sources, including utilities, thermostat manufacturers, national research laboratories, and other research organizations. Energy savings estimates ranged from a low of 2.5 percent of household space heating energy use to a high of 35 percent. The studies reviewed indicate that the energy savings due to installation and use of a programmable thermostat for homes with natural gas heating are significantly higher than that for homes with electric heating.

This GDS report examined the benefit/cost ratio for the KeySpan Energy Delivery (KED) Energy Star® Thermostat Program and concludes that the program is cost effective based on KeySpan’s conservative estimate of annual savings of 44 therms per participating household per year, a unit incremental cost estimate of $65 per programmable thermostat in the residential retrofit market, and a ten-year useful life for this equipment.  The program benefit/cost ratio using these assumptions is 2.6 for the residential retrofit market if thermostat incremental costs remain constant at $65 per unit in the future. The program is even more cost effective in the residential new construction market because incremental costs decline substantially.

The KED savings estimate per home is based on a detailed end use metering study conducted by gas utilities in Massachusetts. The energy savings of 44 therms per year per household represents about 3.6% of average annual gas use per home for space heating. The savings estimate of 44 therms per year per household is a reliable estimate having detailed support, and reflects a 55.8% realization rate determined from a detailed evaluation of this program conducted by gas utilities in Massachusetts. The KED estimate is also consistent with recent research conducted by Pacific Gas and Electric and Northeast Utilities.

## Description of Sources of Information

The information collected from websites, articles, and representatives and lead energy efficiency advisors at various organizations is summarized in Table 1 and includes the organization, the source of information, and the estimated annual energy savings in terms of energy or dollars.  Energy savings range significantly across the data sources.  Some of the sources contacted were not able to provide details, assumptions, or descriptions of how the savings estimates were computed.

### Table 1 – Annual Energy Savings Estimates for Programmable Thermostats

| Data from Industry Organizations |  |  |
| --- | --- | --- |
| Name | Source | Savings Range |
| Lawrence Berkeley Laboratory | Report | $29 annual bill savings & 30% rate of return |
| Wisconsin Public Benefits | Article | 3% heating bill savings for each degree the thermostat is lowered |
| National Renewable Energy Lab | Report | 10%-15% savings on home energy bill |
| NYSERDA | Website | 3% savings for each degree the thermostat is lowered |
| ACEEE | Website | 2% savings for each degree the thermostat is lowered |
| Green Building Program | Sourcebook | Cooling savings 11%-15%. Heating savings up to 30% |
| New Hampshire Electric Coop. | Website Article | 15%-30% energy cost savings |
| PG&E | Fact Sheet | 3% of monthly energy usage |
| Energy Efficiency and Renewable Energy Network | EREC Fact Sheet | 5%-15% annual heating bill savings |
| Lux Products | Article | 35% reduction in heating costs. 25% reduction in cooling costs |
| PageWise | Article | 25%-30% |
| Business 2.0 | Article | 15%-30% annual fuel cost savings |
| Business 2.0 | Article | 30% heating and cooling bill savings |
| California Energy Commission | Report | Specific energy savings estimate per household were not provided |
| International Energy Program Evaluation Conference | Survey | Specific energy savings estimate per household were not provided |
| Renewable Energy Policy Project | Article | Specific energy savings estimate per household were not provided |
| Consumer Reports | Website Article | Cuts costs up to 20% |

| Thermostat Manufacturers |  |  |
| --- | --- | --- |
| Name | Source | Savings Range |
| Honeywell | Brochure | Save up to 33% on annual heating and cooling costs |
| Hunter | Fact Sheet | Reduce annual heating costs by up to 33% and cooling costs by up to 25% |
| Lux Products | Article | 35% reduction in heating costs. 25% reduction in cooling costs |
| White-Rodgers | Brochure | Energy savings up to 33% |

| Energy Industry Research |  |  |
| --- | --- | --- |
| Name | Source | Savings Range for Programmable Thermostats |
| KeySpan Energy Delivery | End use metering study in Massachusetts | 44 therms per year per home (3.6% of the 1,237 therms per household used for space heating) |
| Energy Center of Wisconsin | White Paper summarizing research | 2.5% savings on heating energy, 3% savings in cost per degree reduction on thermostat setting |
| Pacific Northwest National Laboratory | Q & A document | Energy savings are significantly less than advertised |
| Pacific Gas & Electric | Paper summarizing research | 424 kWh per year 42 therms per year |
| Northeast Utilities | Paper summarizing research | 7.1% savings on baseboard electric heaters |
| North Attleboro (Mass.) Gas | Regulatory filing | 60 therms per year |

# Energy/Cost Savings from Thermostat Manufacturers

GDS contacted four manufacturers of programmable thermostats. Information provided by these thermostat manufacturers indicates in most instances significant energy savings resulting from the installation and use of programmable thermostats.  As shown above in Table 1, annual energy savings are estimated to be as high as 33 percent with respect to heating and cooling by four top manufacturers.  The estimated cost savings, up to 35 percent, are more than twice as high as the estimated 15 percent developed by the Alliance to Save Energy.  The information provided by these sources may not always address consumer behavior; that is, the extent to which consumers using setback thermostats might already be conserving energy by manually setting back their standard thermostats prior to installing the programmable models.

# Utility Evaluations of Programmable Thermostats

## Northeast Utilities

In April 1999, Northeast Utilities, assisted by CDH Energy Corp., completed a study of the impacts of mechanical thermostat replacements.  The purpose of the study was to determine the energy impact with the replacement of mechanical line voltage thermostats on baseboard electric heaters with electronic thermostats.

The study was based on a sample of 60 sites, and it was determined that heating use was reduced by approximately 7 percent.  Results varied widely at the sites, ranging from a reduction of 39 percent (2,087 kWh) to and increase of 79 percent (955 kWh).  During the study, it was determined that homeowners frequently changed the settings on the thermostat, resulting in wide variability in consumption and energy savings.

## Pacific Gas & Electric

During 2000, Pacific Gas & Electric (PG&E) performed a study entitled Market Effects Study and Evaluation of PG&E’s 2000 Time of Sale Energy Renovation Program.  Based on a sample of 150 homes, mean energy savings were developed for various energy efficiency measures.  Estimated savings for setback thermostats were 424 kWh per year for home with electric heating and 42 therms per year for homes with natural gas heating systems.  These amounts indicate that the energy savings for homes with natural gas heating are significantly higher than that for homes with electric heating.

# Pacific Northwest National Laboratory Study

In a study conducted by Pacific Northwest National Laboratory, it was concluded that the energy savings from programmable thermostats are significantly less than what has been advertised by manufacturers.

The study was based on a sample of 150 homes in the Pacific Northwest – 25 homes with programmable thermostats, and the rest with the conventional manual models. Data loggers were used to monitor energy and temperature in each house, providing thermostat set points and timing. The study concluded that the value of programmable thermostats has been greatly overestimated in the past “because the mere presence of the thermostat doesn’t necessarily alter the homeowner’s behavior.”  The attitude of the occupants of the home was the major factor in the operation of the thermostats.

It was concluded that the value of programmable thermostats is that they can be set to reflect the lifestyles of the occupants of the home. The thermostat can be adjusted to raise or lower the temperature when the occupants go to sleep, when they leave the homes for the day, or when they are on vacation. While setbacks can be performed manually, the programmable thermostat can be set so that the timetable is followed exactly as desired.

The study found that people used the thermostats in widely varying ways. Some homeowners adjusted the thermostats carefully to move the settings several times every day. Some didn’t make any adjustments at all. In many cases, even though the thermostats can be pre-set and then left alone, homeowners kept adjusting them frequently. The results showed that people with adjustable thermostats basically followed the same manual adjustments made by people with conventional thermostats.

# Energy Center of Wisconsin Study

A study completed by the Energy Center of Wisconsin indicates that programmable thermostats may not provide the energy savings expected.  Furthermore, in some instances, programmable thermostats have resulted in higher energy costs.

The residential characterization study, conducted by the Energy Center of Wisconsin during 1998-1999, is based on a sample of 299 owner-occupied single-family homes, which represented the state’s housing stock, including low-income households and new construction units. Survey participants were interviewed to obtain information pertaining to their respective habits regarding thermostat settings at various times of the day, manual settings of their thermostats, and whether they chose to practice the temperature setbacks.

The study concludes that Wisconsin homeowners who learn how to work the programming on their setback thermostats would also set back temperatures with manual stats, and usually to the same degree difference; therefore, energy savings would be minimal if there were any at all.  Homeowners with little or no interest in energy conservation will resort to the "hold" function on programmable stats, bringing it back to a steady temperature with no setback.

The study also concludes that the 15% cost savings estimated by the Alliance to Save Energy has already been disproved to some degree. In 1997, researchers Cross and Judd found that "Actual energy savings from programmable thermostats were less than estimated, due to attrition from the use of programmable thermostats by homeowners and the fact that customers frequently already practiced a manual setback.

In some instances it was reported that while owners of setback thermostats set temperatures lower than owners of manual thermostats during the heating season, during non-setback times, programmable thermostat owners raised the temperature setting higher, essentially negating their deeper setback.

While the study concluded that homes with programmable thermostats use approximately 2.5 percent less energy for heating than do homes with standard thermostats, the estimate was not statistically significant.  The 90 percent confidence interval associated with the estimate suggests that homes with programmable thermostats use between 10 percent lower and 5 percent higher heating energy intensity than homes with manual thermostats.  The study indicates that the average homeowner saves about 3 percent from his/her heating bill with every degree of reduction in their average winter thermostat setting.

While details were not available on the types of thermostats compared in the study (other than whether they were manual or programmable), it may be reasonably assumed that most programmable thermostats were older models. Newer programmable thermostats with features like adaptive recovery offer more energy savings, because they can make intelligent control decisions between first- and second-stage heating.

# Conclusions

There are numerous data sources from which to obtain annual energy savings estimates for installation and use of programmable thermostats. Thermostat manufacturers, for example, report very high energy and cost savings associated with their setback thermostats.  On the other hand, field research and surveys focused on the impact of programmable thermostats indicate that the associated annual energy and costs savings are considerably lower than that promoted by manufacturers.

A key element driving the level of annual energy savings is consumer behavior, or the degree to which consumers that have programmable thermostats would manually set a standard, or non-programmable model.  There would be little, if any, savings associated with a programmable thermostat for consumers that routinely adjusted their non-programmable models.  On the other hand, savings could equal levels promoted by manufacturers for consumers using a programmable thermostat who otherwise would not routinely adjust a non-programmable model. It is important to note that the energy savings estimate developed by KeySpan is based on a detailed end use monitoring study, and it incorporates a 55.8% realization rate to reflect the impact of this consumer behavior phenomenon.

Studies conducted by Pacific Northwest National Laboratory (PNL) and the Energy Center of Wisconsin (ECW) are relevant in that the research focused on determining the impact of consumer behavior on annual energy savings estimates.  These studies indicated three significant findings:

- Consumers with adjustable thermostats basically followed the same manual adjustments made by people with conventional thermostats

- Consumers who learn how to work the programming on their setback thermostats would also set back temperatures with manual stats, and usually to the same degree difference

- Homeowners with little or no interest in energy conservation will resort to the “hold” function on programmable stats, bringing it back to a steady temperature with no setback.

Given these findings, GDS concludes that the energy savings associated with programmable thermostats in the KeySpan program are most likely to be considerably less than the 33 percent estimates advertised by manufacturers and the 15 percent  estimate developed by the Alliance to Save Energy. New programmable thermostats, however, can make intelligent control decisions between first- and second-stage heating. Thus we conclude that are likely to be greater energy savings associated with the newer programmable thermostats.

Based on the information GDS collected from manufacturers, industry organizations, and researchers, it is our conclusion that the energy savings estimate of 44 therms per household per year (4.4% of annual space heating usage) used by KED for the impact of due to the installation and use of a programmable thermostat is a valid and reliable estimate.

Second, there are differences in annual energy savings with respect to the type of heating system.  Many electric heating systems have multiple thermostats and heat for a room can be controlled on a room-by-room basis. On the other hand, homes with natural gas heating systems typically have one thermostat per floor, and do not have the flexibility of controlling the temperature on a room-by-room basis. We conclude that more manipulation of thermostat settings occur on the electric heating systems; therefore, there is greater energy savings associated with programmable thermostats on natural gas heating systems than on electric heating systems.  This conclusion is supported by the research conducted by PG&E.

KeySpan Energy Delivery (KED) uses an estimate of 1,237 therms per year per household for space heating for residential gas heating customers in Massachusetts. The source of this number is the 2002 KED Report titled “Energy Cost Analysis - MA & NH". The American Gas Association “2001 Gas Facts” statistical report states that average annual gas use for space heating in New England is approximately 93.4 Mcf per year. This is equivalent to 959 therms. For this study GDS assumed that the average Massachusetts home heated with natural gas would use 1,237 therms annually for space heating.

Johnson, Russ and Bhagani, Dinesh, Northeast Utilities, Carlson, Steven, CDH Energy Corp., Measured Impact of Mechanical Thermostat Replacement, 2000 ACEEE conference.

Gobris, Mary Kay, et al, Market Effects Study and Evaluation of PG&E's 2000 Time of Sale Energy Renovation Program, 2001 International Program Evaluation Conference

www.repp.org/articles/static/1/988055428_982762890.html

www.achrnews.com/CDA/ArticleInformation/coverstory/BNPCoverStoryItem/0,1336,16691,00.html